MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

WILTON WATER DEPARTMENT

VARNUM POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

prepared by

Drumlin Environmental, LLC

Portland, Maine

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

WILTON WATER DEPARTMENT

VARNUM POND WATERSHED

MARCH 2003

prepared for

Source Water Assessment Program

Drinking Water Program

Maine Department of Human Services

11 State House Station

Augusta, Maine  04333

prepared by

Drumlin Environmental, LLC

15 Franklin Street, P. O. Box 392

Portland, Maine 04112-0392

(207) 771-5546

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

WILTON WATER DEPARTMENT

VARNUM POND WATERSHED

TABLE OF CONTENTS

Section      Title                                                                                                            Page No.

|  | EXECUTIVE SUMMARY | ii |
| --- | --- | --- |
| 1.0 | INTRODUCTION | 1 |
| 2.0 | WATER SUPPLY SOURCE | 1 |
|  | 2.1 Description | 1 |
|  | 2.2 Land Use | 2 |
| 3.0 | INVENTORY OF POTENTIAL WATER QUALITY CONCERNS | 3 |
|  | 3.1 Background | 3 |
|  | 3.2 Source Sensitivity to Land Use | 4 |
| 4.0 | # ASSESSMENT OF VARNUM POND | 5 |
|  | 4.1 Watershed | 5 |
|  | 4.2 Shoreland | 5 |
|  | 4.3 Intake | 6 |
| 5.0 | SWAP RANKING AND RECOMMENDATIONS | 6 |
|  | 5.1 Ranking of Susceptibility | 6 |
|  | 5.2 Recommendations | 7 |
|  | Table 1 - Varnum Pond Supply | 2 |
|  | Table 2 - Sources of SWAP Information | 4 |
|  | Table 3 - Varnum Pond Surface Water Assessment | 7 |
|  | Figure 1 - Public Water Supply and Surrounding Watershed | End |
|  | Figure 2 - Land Under Protective Ownership | End |
|  | Figure 3 - Multi-Resolution Land Characterization | End |
|  | Figure 4 - Potential Erodible Soils | End |
|  | Appendix A - SWAP Assessment Guidelines | End |

# WILTON WATER DEPARTMENT

# VARNUM POND WATERSHED

# EXECUTIVE SUMMARY

The Wilton Water Department serves approximately 2,400 people from Varnum Pond.  The 2,650-acre watershed of Varnum Pond is located in Wilton and Temple.  There are no significant streams that discharge into the pond.  The pond is small but is relatively deep and therefore is considered to be a spring fed supply.   Approximately 95 percent of the watershed exists as forest growth.  The northern and eastern shoreland areas are developed with a limited number of camps or seasonal homes.  The southern shore, including the intake, and western shore are owned by the Wilton Water Department.  The North Pond Road access to the intake area is unrestricted although the road conditions are difficult to traverse with a vehicle.   Additional safeguards for land use in Wilton are provided through a 250-foot Resource Protection Zone established around the pond and the Watershed Overlay Zone.

No significant land use threats were identified during the reconnaissance of the watershed.  The principal activities to monitor in the watershed include recreational use, timber harvesting and camp road development/maintenance.  The Water Department is currently in the process of posting water supply signs around the pond and along the intake pipeline to keep a public awareness for protecting the pond.

The Water Company intake, located in the reservoir in approximately 18 feet of water at a distance of 300 feet from shore, delivers water by gravity through a pipeline to the sand filter treatment plant located about a mile to the south along the Water Department access road.  Raw water monitoring for routine parameters, and testing for TE2, TE3 and other parameters have indicated good water quality from this source, i.e., showing all test parameters reported below public drinking water standards.

Based on the largely undeveloped nature of the watershed, rural setting, existing protection strategies and ongoing public awareness program of the Water Department, the overall susceptibility of the Varnum Pond water supply is considered to be low.

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

WILTON WATER DEPARTMENT

VARNUM POND WATERSHED

1.0    INTRODUCTION

The operation of public water systems in Maine is governed by the federal Safe Drinking Water Act (SDWA), which was first passed in the 1970s and later amended in 1986 and 1996.   The federal government delegated authority for enforcing the SDWA to the state under the Maine Department of Human Services (DHS) Drinking Water Program (DWP).   In 1998, the Source Water Protection Section was established in the DWP to implement the Maine Public Drinking Water Source Water Assessment Program (SWAP).  The SWAP process is being conducted through a cooperative effort between the DWP staff, Wilton Water Department (Water Department) staff and Drumlin Environmental, LLC (Drumlin), the environmental consulting firm contracted through DHS to provide technical assistance for the project.

The purpose of the SWAP evaluation for the Water Department is to assess the susceptibility of its Varnum Pond drinking water supply (PWSID  91620) to potential threats of contamination.   The long-term goal is to protect the water supply source.   To do this, the SWAP process has compiled information to assist the Water Department in future planning that will help to control potential threats to the water quality of Varnum Pond.

This Report describes the SWAP for the Wilton Water Department and is organized as follows:

- Section 2.0  summarizes the physical characteristics of the water supply at Varnum Pond;

- Section 3.0  describes the variety of conditions and activities that could pose a threat to water quality;

- Section 4.0  provides an assessment of the threats within the Varnum Pond watershed; and,

- Section 5.0  ranks the susceptibility of the water quality at Varnum Pond and provides recommendations for future protection of water quality.

2.0   WATER SUPPLY SOURCE

2.1   Description

Nearly half of the Varnum Pond watershed is located in Wilton with the remaining, slightly greater, portion located in Temple, Maine.  The pond and watershed area are depicted in Figure 1.  The components and description of the Water Department source are presented below in Table 1.

# TABLE 1

# VARNUM POND SUPPLY

| # Category | # Description |
| --- | --- |
| Water System Name: | Wilton Water Department (municipal) |
| Surface Water Source(s): | Varnum Pond |
| Water System Type: | Non-transient, community system |
| Watershed Location: | Wilton and Temple, Maine |
| Source Surface Water Area: | 330 Acres |
| Source Watershed Area: | 2,650 Acres |
| Average/Maximum Depths (estimated): | 30 to 40 ft; 80 to 90 ft |
| Water Volume in Storage: | 4 billion gallons |
| Population Served (customers): | Services: 1,000 Population: 2,375 (serving Wilton & North Jay) |
| Type of Treatment: | Initial Coagulation, Chlorination, Sodium Hydroxide–caustic, & Sodium Bicarbonate for corrosion control |
| Filtration: | Rapid sand filtration system in use |
| Estimated Daily Water Use: | 375,000 GPD average; 550,000 GPD peak use |

The Water Department intake is positioned in the pond approximately 300 feet from shore at the southern end of the pond near the outlet into Varnum Stream.  The intake rests on the pond bottom and the top of the pipe is located approximately 18 feet below the water surface.  The intake is not marked with buoys or signs to indicate its exact location, which is preferred by the Water Department.  Water flows by gravity from the intake into the filtration plant, which was built in 1993.  Water flow into the plant is controlled with valves that operate automatically in response to the demand required by the users on the distribution system.  The plant is located approximately one mile to the south along the gravel access road, North Pond Road.  The raw water enters the plant and is first treated with a coagulant.  The water flows through the sand filters and is subsequently treated with chlorine to remove microbial threats.  The water is also treated with sodium hydroxide and sodium bicarbonate to raise the pH level, reduce corrosion potential and lower lead solubility prior to use in the distribution system.

2.2   Land Use

The Water Department owns 131 acres of land at the southern end of the pond and includes the intake area.  The ownership extends along the western side of the pond and includes the shoreland up to the municipal boundary with Temple.  Figure 2 shows the land under permanent protection in the watershed.  The remaining watershed area in Wilton and Temple is privately owned.

A Resource Protection Zone (250-ft setback) applies around the shore of the pond.  The state guidelines for shoreland zoning were generally adopted by the Town of Wilton for this zone.  In addition, Wilton has adopted a Watershed Protection Overlay Zone to encourage land use practices in the watershed that are protective of the water supply.  The overlay zone provisions include many of the performance standards that exist for the shoreland zone.   Specifically, these standards address activities involving timber harvesting, septic system installations, agricultural practices, construction of piers and docks, land clearing, camp construction, campgrounds, roadways and driveways.

Several small, older camps are located on the east shore of Varnum Pond.   In addition, there are several old and newer seasonal camps located on the northern peninsula of land and along the northeast shoreland area.  A half-dozen or so gravel roads and driveways provide access to the camps and shoreland areas primarily from the north and east sides of the pond, i.e., from Varnum Pond Road and Farmer Road.  Currently, the density of these camps is low when considering the five plus miles of shoreland that comprise the perimeter of the pond.  Except for the limited number of small camp lots described previously, the remaining shoreland around the pond is undeveloped and in forest growth.   Occasionally, permits are issued under the Overlay Zone provisions to allow timber harvesting within the watershed.

The Varnum Pond watershed is sparsely developed with only a few residential homes and limited roadways located in the northern and eastern portions of the watershed.  Approximately 95 percent of the watershed consists of evergreen, deciduous and mixed forest growth.  Wetlands, limited pasture and croplands comprise most of the remaining area other than roadways and residential lots.   A map of the land use classifications for the watershed and surrounding area is shown in Figure 3.

There is no formal public access for recreational activities on the pond.  However, there is limited public use of the pond with access from the gravel roads and camp driveways.   Access into the intake area is unrestricted since no signs or gates exist to prevent transient visitors from entering the Water Department property.   Swimming and motorboats are not allowed on the pond although non-motor boats are permitted.   Fishing is allowed throughout the year.  Recreational vehicles and snowmobiles can use the roadways in the watershed, but are not allowed on the ice in the winter.

3.0   INVENTORY OF POTENTIAL WATER QUALITY CONCERNS

### 3.1   Background

### The Source Water Assessment Program (SWAP) is intended to assist water companies and departments in protecting the quality of their surface water supply by identifying potential threats to water quality.  Two factors have been considered in identifying and assessing the potential risk posed by a threat: (a) location in relation to the intake and (b) the nature of a potential threat.

This SWAP assessment looks for potential threats to water quality in three zones:

- The intake zone within 1,000 feet of the intake;

- The shoreline zone, within 250 feet of the shoreline of the surface water body; and,

- The watershed zone, extending to the limits of the surface watershed.

Activities in each of these zones have a different potential effect on the quality of water in the surface supply.

Within each of the three zones, the SWAP assessment examined a variety of conditions, land uses and practices that have a potential to influence water quality.  These features generally fall into one of the following categories:

- Physical characteristics of the watershed, for example the presence of wetlands, steep topography or erodible soils;

- General land uses and development patterns, for example the percentage of developed versus undeveloped land and controls to guide future development; and,

- Specific activities that involve chemical handling or may release pathogens (e.g., fecal coliform) with the potential to degrade water quality.

The SWAP assessment also considered available water quality data from the surface supply as an indicator of existing conditions.  Appendix A describes specific guidance used during the SWAP to inventory potential factors influencing water quality.  Appendix A also lists specific activities included in the SWAP assessment.  Table 2 lists general sources of information consulted during SWAP research.

# TABLE 2

SOURCES OF SWAP INFORMATION

| Information Type | Information Source |
| --- | --- |
| Watershed Characteristics | US Geological Survey (Topography, Hydrology), Soil Conservation Service (Soils), Maine Geological Survey (Geology) |
| Raw Water Quality | Water Department Monitoring |
| Potential Contamination Sources | DEP Databases and Water Department |
| Land Use and Zoning | Water Department and Municipal Records |
| Other Analyses | Other DEP Databases |

3.2  Source Sensitivity to Land Use

The key watershed features that define the sensitivity of the water supply source to land uses are briefly described below:

- Natural Features:   The watershed is largely forested with minor agricultural uses located  remote from the shore of the pond.  The natural, undeveloped watershed conditions are protective of the water supply source.   The relatively large volume of water stored in the pond provides a good buffer to potential impacts from runoff.

- Man-made Features:   Only a few residential homes exist within the watershed and the density of camps in the shoreland area is quite low.  The features by which the source could become sensitive include timber harvesting and gravel roadways/driveways located close to runoff drainages and the pond shoreline where erosion/siltation problems could develop.

- Specific Activities:   Access to the pond shoreland is unrestricted.  Recreational activities, specifically fishing on the pond and RV use on watershed trails, are popular with the local public.  The rural nature of the watershed area, remote location of the water supply and local public awareness, however, tend to help mitigate these pressures.

4.0   ASSESSMENT OF VARNUM POND

Varnum Pond is used as a filtered and chlorinated water supply source.   Historically, the water quality has been very good.   Raw water test records (i.e., TE2 & TE3) maintained by the Water Department show all parameters are within acceptable standards for public drinking water.  The last round of testing for inorganics, microbiological activity and radon was completed in 2000.  In 1999, the system was granted a “synthetic organics waiver” which provides for a three-year exemption from certain testing requirements (i.e., pesticides, herbicides, fungicides and organic chemicals).  The waiver status will need to be revisited for 2002.

4.1   Watershed

Varnum Pond and its watershed are of moderate size with no significant stream drainages flowing into the pond.  The Water Department believes that the pond is largely spring-fed.  The   depth of the pond provides for a sizeable storage volume as stated previously.  Land use in the watershed is comprised of a few residential homes, limited farming and timber harvesting which is permitted under the provisions of the Watershed Overlay Zone adopted by Wilton.  The water supply intake is protected by Water Department ownership of the shoreland although the access road is not gated to restrict public use.  The trails and roads in the watershed are used regularly for RV recreation.

Review of the DEP databases of regulated activities and observations made during the SWAP reconnaissance of the watershed found no significant point or non-point sources of potential contamination that would be a concern to the water supply.

4.2   Shoreland

Nearly all the shoreland presently exists as undeveloped property with forest growth and small wetland areas.  The shoreland is protected, in part, by zoning that extends 250 feet from the shore.  Several private camps occupy sites along the northern and eastern shoreland areas.  These camps reportedly rely on subsurface systems for the disposal of domestic wastewaters.  The shoreland topography around the pond varies from gentle to steep slopes.

The forested, undeveloped slopes around the pond are generally not subject to erosion.  Soils found on steep slopes are sensitive to erosion, if disturbed (see Figure 4).  Encroachment near the shore by gravel roadways has caused some soil disturbance and ongoing maintenance is needed to minimize erosion.  For example, shorefront rip-rap construction was observed along a camp road located on the north shore.  One location of sand accumulation in the pond is suspected to have been caused by erosion.  No other reports of significant erosion were noted.   Vehicles traveling close to the shoreline pose a concern if fuels or chemical product containers accidentally spill into the pond.

4.3   Intake

The water supply intake is located in the pond approximately 300 feet from shore.  The intake is not presently identified by signs or buoys and the Water Department prefers that its specific location remain unmarked.   Activity on the pond is reportedly minor and limited primarily to the camp owners and local population.   However, the Water Department recently purchased signs to post at the intake, around the pond and along the water main leading to the treatment plant.   The signs notify the public that Varnum Pond is a water supply and should be protected.

Lakes and ponds normally experience an aging process known as eutrophication, which is caused by various natural and man-made influences.   Phosphorus from runoff, fertilizers, sewage and other sources is a primary factor affecting eutrophication since phosphorus nourishes plant and algae growth in the pond.   The amount of algae productivity in the pond is characterized in terms of three Trophic States: (1) Oligotrophic = low, (2) Mesotrophic = moderate, and (3) Eutrophic = high.  The Trophic State can be identified based on chlorophyll and phosphorus content and by secchi disk transparency data, which measures the clarity of the pond water.  The results of TE2 testing show low color and turbidity generally below 0.5 NTU.   Occasionally, turbidity and color may show slightly higher values during spring runoff.

According to a surface water classification database developed by the DEP, the Trophic State of Varnum Pond is listed as Mesotrophic.  The pond water is ranked as fully attaining the GPA classification with respect to primary contact (i.e., swimming), aquatic life and trophic stability criteria.

Based on all the testing completed to date on the raw water from Varnum Pond, the water is a high quality source.

5.0   SWAP RANKING AND RECOMMENDATIONS

5.1   Ranking of Susceptibility

The SWAP assessment factors indicate that overall susceptibility of the water quality in Varnum Pond is low.  This conclusion is based on the general conditions observed, including the lack of development in the watershed, the predominantly rural, forested land use and the absence of potential threats from land use activities.  Specific factors considered in assessing the overall risk are summarized in Table 3.

# TABLE  3

# VARNUM POND SURFACE WATER ASSESSMENT

| Zone | Measure | Findings | Risk Level |
| --- | --- | --- | --- |
| Watershed | Ambient Water Quality Existing Conditions Future Development Overall | Class GPA, in full compliance. Land use characteristics present a low threat to water quality; timber harvesting & RV use managed through permits and public awareness. Future development is controlled in part by zoning in Wilton although no watershed zoning exists in Temple; pressure for future development is low. | Low Low Low Low |
| Shoreland | Pond Classification Soils Activities Posing a Threat Potential for Future Threats Overall | Mesotrophic (sustaining conditions) Some potentially erodible soils along camp roads, (Daigle (8-15%), Marlow (8-25%). Vehicular travel near the shoreline. Development of private shoreland; limited protection for Temple shoreland. | Low Low Low-Moderate Low-Moderate Low |
| Intake | Raw Water Quality Ownership/Control Activities Posing a Threat Potential for Future Threats Overall | TE2 & TE3 testing shows good quality. Ownership protection for intake area and western shoreland. Public access to intake area unrestricted. None identified. | Low Low Low Low Low |
| Overall |  |  | Low |

5.2   Recommendations

The overall ranking for the pond susceptibility to threats of contamination is low.  Several additional actions could be considered by the Water Department to provide added protection to the quality of the water supply.

- The access road into the intake area is difficult to traverse with a vehicle.  However, the Water Department may want to consider installing a locking gate in order to discourage use of the road by the public to access the pond.   The posting of water supply signs is a good strategy to provide public awareness in any event.

- Maintain a good working relationship with the large landowners in the watershed that are managing the forest for wood production in order to encourage forest cutting practices that are protective of the water resource and avoid the use of pesticides.

- Shoreland and watershed areas currently have some protection under existing zoning and ownership.  Further enhancement of this protection should be pursued through additional land purchases, conservation easements, etc., and by establishing a partnership program with Temple to encourage similar protective strategies in the northern half of the watershed.

APPENDIX A

#### SWAP ASSESSMENT GUIDELINES

MAINE PUBLIC DRINKING WATER

SOURCE WATER ASSESSMENT PROGRAM

WILTON WATER DEPARTMENT

VARNUM POND WATERSHED

# APPENDIX A

# ASSESSMENT GUIDELINES

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Physical Characteristics of the Watershed Watershed boundary, area Area of water body Tributaries Watershed topography Wetlands Water body depth Sand and gravel aquifers Soil types (erodibility and slope) | smaller smaller few low many deep few low | >> >> >> moderate >> >> some moderate | larger larger many steep, rugged few shallow many high |
| Raw Water Quality Secchi Disk Transparency Turbidity pH Phosphorus Pesticides and herbicides Color Total Coliform and E Coli Dissolved Oxygen Temperature VOCs Chlorophyll-a Total Organic Carbon | > 8 M < 1 NTU 6.5-8.5 < 10 ppb ND < 5 > 29/100 ml >7 mg/L ambient absent < 2 ppb < 4 mg/L | 4-8 M < 5 NTU N/A < 20 ppb N/A 5-15 occasional 5-7 mg/L N/A 2-6 ppb 4-8 mg/L | < 4 M > 5 NTU < 6.5 or > 8.5 > 30 ppb detected > 15 > 142/100 ml < 5 mg/L > 10o C ambient detected > 6 ppb > 8 mg/L |
| Potential Contamination Source Inventory UST/AST leak sites Municipal/Comm/Special Landfill Compost sites Demolition debris sites Ash/septage storage/utilization Superfund site Sand/salt storage sites Petroleum spill sites Hazardous waste sites Wastewater treatment facilities Food residuals utilization sites Uncontrolled hazardous sites Tank farms Industrial complexes | Absent from watershed or actively monitored and controlled or fully remediated. | Present in watershed, some operational deficiencies and/or remediation is underway. | Present and unmonitored and/or unremediated. |

| Assessment Item/Threat | Low | Moderate | Significant |
| --- | --- | --- | --- |
| Potential Contamination Source Inventory Non-point pollution sources Transfer stations Sludge utilization sites Automobile graveyards Engineered subsurface systems Woodyards Underground injection wells Surface impoundments Mining/mineral processing sites Overboard discharges to rivers Active UST sites Roadways Railroads Utility Rights-of-way Boat launches Commercial farms CAFOs Commercial forest operations Private septic systems Home heating oil tanks Airports Bathing beaches Combined sewage overflows | If present, at low density, properly installed and operated. In compliance with applicable local, state and federal regulations. | >>>> | High density, little evidence of active management, documented incidence of problems and failures. |
| Land Uses and Zoning Residential density Percent of watershed Owned by PWS Public lands or conservation Forested With protective zoning Agricultural Impervious cover Industrial/commercial Waterbody uses: Restricted areas Ice fishing Boating Swimming Recreational vehicles | > 2 Ac/lot total > 80% for all protected uses < 20 % total < 20 % total < 20 % total large, posted limited to absent | 1-2 Ac/lot intake and shoreland protected, others developed 20-30% total 20-30% total 20-30% total intake zone moderate, well-managed | < 1 Ac/lot critical areas (intake, shoreland) unprotected or developed > 30% total of 3 > 30% total of 3 > 30% total of 3 small to none prevalent, uncontrolled |
| Other Analyses Trophic State Index Vulnerability Index Build-out Analysis | < 25 low fully developed | 25-60 moderate >>> | > 60 high high potential for development |