NLEV, LEV & ZEV INPUTS for MOVES2014

(05/12/15)

Purpose: Some states have adopted California Low Emission Vehicle (LEV) standards.  Other states (in the northeast) that did not adopt LEV standards received early implementation of National Low Emission Vehicle (NLEV) standards for the 1999 and 2000 model years.

This document explains the process used to input for the National Low Emitting Vehicle (NLEV), and California Low Emission Vehicle (LEV) and Lev programs for Maine by following the “Instructions for Using LEV and NLEV Inputs for MOVES2014”. While the instructions imply LEV and NLEV inputs they also include Zero Emission Vehicle (ZEV) inputs as well.

This package (LEV and early NLEV modeling information (ZIP) (2.69MB) Updated September 2014) includes instructions and input files that can be used with MOVES2014 to model the emissions from these vehicles found under “Tools to develop special case MOVES inputs” on the following website… http://www.epa.gov/otaq/models/moves/tools.htm

Maine’s Program

The Maine Department of Environmental Protection regulates motor vehicle emissions standards for new passenger cars, light duty trucks, medium duty vehicles, and heavy-duty diesel vehicles under Chapter 127.

Program criteria in the document that applies to Maine are as follows:

- For the OTC states that adopted California LEV standards beginning with model year 2001 or later are subject to applying inputs for Sections 1 & 2 in the instructions.  Section 1 applies NLEV (A.K.A. earlyLEV).  Section 2 applies to LEV (A.K.A. Cal LEV).  Maine adopted LEV in 2001 so both sections apply to all modeling activities.

- States that adopted the California ZEV standard in any year are subject to applying inputs for Section 3 in the instructions which refer to the ZEV inputs.  Maine’s ZEV program began in 2009.

Based upon these two criteria outlined in the instructions, we needed to follow sections 1-3 to complete the input process.

PREP FOR SECTION 1

All that is needed is to load the “early_NLEV” database into the C:\ProgramData\MySQL\MySQL Server 5.6\data directory.  No adjustments needed to the table.  When setting up a MOVES run, the Manage Input Datasets Panel is used to add this database to the run spec.

PREP FOR SECTION 2

The LEV_standards database has to be modified to reflect when the program began in Maine, otherwise all years will apply to the emissions inventory.  Maine adopted the LEV standards in 2001.  The instructions are detailed and easy to follow.  The LEV_standards database is copied to the C:\ProgramData\MySQL\MySQL Server 5.6\data directory along with the script used to make adjustments to the number of years that apply (see Appendix I).  It goes back to 1994.  The script is used to delete years 94-2000 and create another emissions rate table (see Appendix II).  The Manage Input Datasets Panel is then used to add the new database to the run spec.

PREP FOR SECTION 3

The MOVES information posted on the web was updated for MOVE2014. The ZEV_AVFT_MOVES2014.xls file contains two worksheets.  One is used to generate the fractions based upon the years that apply.  For Maine’s program we changed columns B and C to zero from 2005 -2008 in the ZEV_AVFT_Generator worksheet.  Formulas in this worksheet make adjustments to the AVFT table containing the fractions needed for the MOVES inputs.  The AVFT worksheet is copied to a new workbook using the paste values function.  This worksheet is then placed in the same location as the CDM input tables.  The CDM GUI is used to import this table to add ZEV to the emissions results for the MOVES run containing evaporative permeation pollutants only!

THINGS TO KNOW BEFOREHAND!!!!!!!!!!!!!!!!!!

Two separate runs are required to load the NLEV, LEV and ZEV inputs.  ZEV inputs should only be generated on pollutants with evaporative permeation emissions. When setting up the run spec file, the only selections should be for evaporative permeation.  The NLEV and LEV runs should exclude evaporative permeation processes.  The combined results for ZEV and the NLEV-LEV runs make up all emissions results.

APPENDIX I

###################################################################

-- MOVES2014MyLEVs.sql

-- 08/14/14

--

-- This script uses the database "MOVES2014_LEV_Standards" to  create a new database, "MOVES2014_MyLEVs"

-- with adjusted LEV emission rates for only a subset of model years

--

-- Users must edit the last command in the script to specify the desired model years.

-- See "Instructions for using LEV and NLEV Inputs for MOVES2014"

###################################################################

-- Create database

drop database if exists MOVES2014_MyLEVs;

create database MOVES2014_MyLEVs;

use MOVES2014_MyLEVs;

###################################################################

-- Create a table to hold the adjusted LEV emission rates.

drop table if exists MOVES2014_MyLEVs.emissionratebyage;

CREATE TABLE MOVES2014_MyLEVs.emissionratebyage (

`sourceBinID` bigint(20) NOT NULL default '0',

`polProcessID` smallint(6) NOT NULL default '0',

`opModeID` smallint(6) NOT NULL default '0',

`ageGroupID` smallint(6) NOT NULL default '0',

`meanBaseRate` float default NULL,

`meanBaseRateCV` float default NULL,

`meanBaseRateIM` float default NULL,

`meanBaseRateIMCV` float default NULL,

`dataSourceId` smallint(6) default NULL,

PRIMARY KEY  (`ageGroupID`,`opModeID`,`polProcessID`,`sourceBinID`),

UNIQUE KEY `XPKEmissionRateByAge`

(`sourceBinID`,`polProcessID`,`opModeID`,`ageGroupID`),

KEY `polProcessID` (`polProcessID`)

) ENGINE=MyISAM DEFAULT CHARSET=latin1 ;

###################################################################

-- Table listing of short codes and model year included in "MOVES2014_LEV_Standards" database

-- shortModYrGroupID | modelYearGroupName

--                94 | 1994

--                95 | 1995

--                96 | 1996

--                97 | 1997

--                98 | 1998

--                99 | 1999

--                20 | 2000

--                21 | 2001

--                22 | 2002

--                23 | 2003

--                24 | 2004

--                25 | 2005

--                26 | 2006

--                27 | 2007

--                28 | 2008

--                29 | 2009

--                30 | 2010

--                31 | 2011

--                32 | 2012

--                33 | 2013

--                34 | 2014

--                35 | 2015

--                36 | 2016

--                37 | 2017

--                38 | 2018

--                39 | 2019

--                40 | 2020

--                41 | 2021

--                42 | 2022

--                43 | 2023

--                44 | 2024

--                45 | 2025

--                46 | 2026

--                47 | 2027

--                48 | 2028

--                49 | 2029

--                50 | 2030

--                 9 | 2031-2050

-- Model years 1994-2003 are LEV I program model years.

-- Model years 2004-2014 are LEV II program model years.

-- Model years 2015-2050 are LEV III program model years.

###################################################################

-- Insert values for LEV emission rates for years covered by program.

-- Edit this command as described in "Instructions for using LEV and NLEV Inputs for MOVES2014"

--       In the sequence of short codes at the bottom of this section,

--       keep the codes that correspond to model years in the LEV Program; and

--       delete the codes that correspond to model years NOT in the LEV Program.

--       Make sure that a comma is not between a number and a parenthesis.

insert into MOVES2014_MyLEVs.emissionratebyage (

sourcebinid,

polprocessid,

opmodeid,

agegroupid,

meanbaserate,

meanbaseratecv,

meanbaserateim,

meanbaserateimcv,

datasourceid )

select

sourcebinid,

polprocessid,

opmodeid,

agegroupid,

meanbaserate,

meanbaseratecv,

meanbaserateim,

meanbaserateimcv,

datasourceid

from MOVES2014_LEV_Standards.emissionratebyage

where polprocessid in (101,201,301,102,202,302,11201,11202,11801,11802)

and mid(sourcebinid,8,2) in

(94,95,96,97,98,99,20,21,22,

23,24,25,26,27,28,29,30,31,32,

33,34,35,36,37,38,39,40,41,42,

43,44,45,46,47,48,49,50,9);

###################################################################

APPENDIX II

In a new SQL Query tab opened after the MY SQL Workbench was closed and then reopened to refresh the data directory to show the newly created MOVES2014_mylevs database, the following statement was typed to verify that the changes were made to the database…

SELECT DISTINCT(MID(sourcebinid,8,2))AS shortcode

FROM moves2014_mylevs.emissionsratebyage

ORDER BY mid(sourcebinin,8,2);

This is the table that was returned from the SQL Query

09

21

22

23

24

25

26

27

28

29

30

31

32

33

34

35

36

37

38

39

40

41

42

43

44

45

46

47

48

49

50