IMCOVERAGE MOVES Tables

(02/16/2012)

Purpose: This document explains the process used to generate IMCoverage tables for the MOVES2010b program. It involves a series of steps where the inspection and maintenance default information for Maine contained in the MOVES default database is downloaded with the MySQL Query Browser, brought over into an Excel workbook and edited to generate MOVES input tables.  In this particular instance templates generated from the County Data Manager are not needed.  When the data is entered into the MOVES model, users must account for all the default I/M program records in some way to avoid double counting emissions from the records stored in the default database.  This document describes how it is done.

Maine’s I/M Program: Maine only has one county (Cumberland 230050) that requires I/M Coverage. Inspections are required annually.  Maine began testing all gasoline fueled vehicle On Board Diagnostic systems (OBD) in 1993.  In 1999 the unloaded idle test was omitted and all OBD from diesel vehicles were added to the testing schedule.  Maine does not have a grace period for I/M inspections.  Testing is performed and reported on all model year vehicles.

MOVES 2010b is not equipped to handle I/M emissions effects for diesel fuel.  No inputs are available in the defaults for diesel fuel type sources. All inputs for diesel testing have to be excluded in the input tables until MOVES is updated to handle I/M Coverage for diesel fuel type.  All gasoline and ethanol fueled passenger cars, trucks and light commercial trucks are tested annually for HC (Hydrocarbons), CO and NOx emissions for start and running exhaust.  To meet Stage II requirements, HC testing is also done for Evaporative Fuel Vapor Venting and Evaporative Fuel Leaks. These tests are making sure that the On Board Diagnostic systems built into the newer model year vehicles for vapor recovery are working properly.

Downloading defaults: The process for obtaining the MOVES defaults from the National database is not provided for in the MOVES User’s Guidance.  The User’s Guidance procedures are designed to allow users to obtain default data by using the County Data Manager.  When requesting default data using the County Data Manager the tables generated for I/M Coverage are often blank.  Another approach is necessary to obtain the actual I/M Coverage data located in the MOVES database.  The MOVES database contains all of the data needed to run the model.  This file is large because it contains all of the National data.  The file name contains the date for the time period that MOVES was last updated.  The MySQL Query Browser is used to access any of the tables containing default data in the MOVES database.  NEVER delete, remove or edit any data in these tables as these are the tables needed for MOVES to run properly.

1.) Open the MySQL Query Browser and locate the database containing the MOVES default tables.  The database is represented by a cylinder type icon in the schemata table on the MySQL browser screen. Click on the file named movesdb20100830 to open the list of tables.

2.) Locate the table named imcoverage. Click on the table until a SQL command “Select*From;” appears in the menu bar at the top of the screen. Click on the green thunderbolt button labeled execute to display the table contents.  It should include all national defaults.

3.) This file is too large to download, select the defaults for state id 23 that represents Maine only data.  In the menu bar click over the last character in the command string just before the semicolon and replace the character command with the following “ where stateID = 23” (you don’t need the quotation marks). Click on the execute button.  This will load Maine’s data into the browser view screen.

4.) Save the data by going to the File menu bar and using the ExportResultset command to export the file as an Excel document named (movesdb20100830 IMCOVERAGE ME DEFAULTS.xls) to a directory where yearID tables can be created.

Create default file for inventory year: The Maine default database contains inspection and maintenance records for all of the years in the inventory. Make a copy of the default database worksheet and use Excel filter feature to select the defaults for an individual year or county if you have more than one.

1.) Open the Excel file movesdb20100830 IMCOVERAGE ME DEFAULTS.xls. Copy Table 1 worksheet into the same workbook and label the new worksheet YearID filter.  Use the excel filter icon to apply an auto filter to the top row of the data set.  Use the filter to change the yearID to the year that you are applying to the inventory.

2.) When the year shows up in the workbook after you applied the filter for the year that is needed for the inventory, generate a copy of the worksheet and store it in the file containing the following naming convention…

YYYY_IM_movesdbdefault.xlsx.

3.) Rename the worksheet YYYY_movesdb_defaults.

Constructing MOVES Inputs: It is important to note that the I/M Coverage MOVES default database table does not contain the same formats as the County Data Manager templates. Some of the columns containing data are in a different order.  Our understanding is that MOVES will take both as long as the column headings are formatted properly.

The MOVES OTAG group recommends working off of the MOVES default database tables directly to make certain that all of the MOVES defaults are accounted for in the modeling run in some way.  MOVES will automatically use the data contained in the defaults because there is a field labeled UseIMyn containing “Y”s telling MOVES to use that data.  The data represents emissions test for all source types, pollutants and fuels where I/M Coverage typically applies.  IT is not always a true representation of the State program.  When submitting data, care must be taken to include all of the defaults that are in the default database the intended State program, records to turn defaults off that don’t apply, and to submit new records for State programs where inspection and maintenance programs apply.

The following was found on the MOVES webpage addressing I/M Coverage submittals.

MOVES USER INFORMATION SUGGEST USING THE MOVES DEFAULT TABLES TO ENTER DATA!  WE NOTICED A DISCREPANCY BETWEEN THE MOVES DEFAULT DATABASE TEMPLATE AND THE COUNTY DATA MANAGER TEMPLATE.  SOME OF THE COLUMNS OF INFORMATION ARE IN A DIFFERENT ORDER.    http://moves.supportportal.com/ics/support/splash.asp

The following question and answer was found on the MOVES webpage.

QUESTION: The template created for IM Programs contains rows for all source types and all fuel types; however MOVES does not provide I/M benefits for all source types and fuel types. How do you recommend creating an Excel input table for the IM Programs which I can then import using the County Data Manager?

ANSWER: If you already have an I/M program in your area, you should always begin by exporting the default I/M program in MOVES, reviewing it, making any changes to the default program, and re-importing it, rather than trying to fill a new template with the appropriate information.  If you take the second approach, MOVES may continue to use default information and you may get unexpected results.  When editing the default table, use the "useIMyn" field to turn specific parts of an I/M program off as opposed to deleting the line, which may also cause unintended results.

If you must create a template for an area that does not currently have default I/M program information in MOVES, the template will have rows for all combinations of vehicle types and fuels that are selected in the On Road Vehicle Equipment panel in the RunSpec. However, only gasoline fueled vehicles (fuel type 1) have benefits assigned for the implementation of I/M programs.  Other vehicle/fuel combinations should be deleted from the template.

The information contained above brings to light the manner in which the I/M coverage tables are submitted to MOVES.  This is the only table where the following applies…

Step 1.	All data in the MOVES default database must be entered into the system.  If the default program is representative of the State program and no changes are needed then leave the defaults with “Y” codes in the UseIMyn field.

Step 2.	If a program is listed in the defaults but not used, turn the UseIMyn field off by changing the “Y” default code to “N”.  A record must be submitted telling MOVES to disregard the default record that does not apply to the State program.

Step 3.	When the default data does not agree with State data, two lines of code have to be entered for a single entry.  One to turn the defaults off (“N”) as noted above in Step 2 and a second to submit changes to the default data to include the new values with a (“Y”) code in the UseIMyn field to tell MOVES to use the State program data instead of the defaults.

1.) Copy the yyyy_movesdb_defaults worksheet into a new workbook and name it YYYY_IM_Input_development.xlsx.  This workbook will be used to edit the default data.

2.) Copy the worksheet into the same workbook to begin making edits.  Label the worksheet IMCoverage. This is the naming convention used for MOVES.  This will be the input file.  Once edits are completed this worksheet will be saved into a new workbook.

3.) Make edits to reflect Maine program changes.

a.) Change all useIMyn “Y”’s to “N”.

b.) Copy all rows of data, append in (red fonts) at the bottom of the table.

NOTE: The data in red fonts must be entered into MOVES, we will change the fonts back after we finished making edits to the top portion of the table in black fonts.  LEAVE the red font portion of the file alone for the remaining steps.

c.) Change all (black fonts) useIMyn “N”s in top portion of table to “Y”.

This tells MOVES to use these records once we apply information that reflects our State I/M program.  If any records do not require edits and we find that they do not apply to our State program, we will delete them later.

d.) Change all complianceFactors in top portion (black fonts) to 96.00.

Maine has a compliance rate of 96.00% for all I/M programs.

e.) Change all endModelYearID (black fonts) for newer models to agree with the yearID.  You should see the newer model year cars noted with an endModelYearID 2 years prior to the yearID.  Many State’s give newer model year vehicles a 2 year grace period before they are required to have inspections.  Maine does not have a grace period; inspections are required on all new vehicles.

Ex. When we look at the defaults for 2011, the (endModelYearID) values for newer model year vehicles are listed as 2009.  We would want to change all of the black font endModelYearID 2009’s to 2011 in the table.

For yearID=2011, the model yearID’s are =1996-2009 Change endModelYearID to 2011

NOTE: Do not change any of the inputs in the endModelYearID column for endModelYearID records listed as 1995 in the black font portion of the table. These should be submitted with no changes to the endModelYearID.

f.) Turn the worksheet filter on and filter table to show all testStandards for the unloaded idle test (11).  Delete all of the rows of records with black fonts containing unloaded idle test (11) information.

Maine discontinued all unloaded idle test back in 1999 when they added diesel testing to the inspection and maintenance program.  Maine only performs testStandards for (41) Evaporative Gas Cap Check, (43) Evaporative System OBD Check, and (51) Exhaust OBD Check.

g.) Apply a filter to the fuel type ID and generate all of the fuel type (5) ethanol records.  Delete all of the rows of black fonts containing fuel type 5 records.

MOVES is only able to utilize gasoline fuel type Id’s in MOVES at this time.

h.) Apply a filter to the source type ID’s. Filter one source type at a time to view all of the records left in black fonts.  The remaining records should complete the rest of Maine’s inspection and maintenance program for all of the pollutants and processes noted in the following table.

| The following pollutants and processes apply to each source type ID’s (21) Passenger Cars, (31) PassengerTrucks & (32) Light Commercial Trucks |  |  |  |  |
| --- | --- | --- | --- | --- |
| FuelType | Pollutant | Pol_ProcessID |  | ProcessName |
| (1) Gasoline | HC | 101 | Running Exhaust |  |
| (1) Gasoline | CO | 201 | Running Exhaust |  |
| (1) Gasoline | NOx | 301 | Running Exhaust |  |
| (1) Gasoline | HC | 102 | Start Exhaust |  |
| (1) Gasoline | CO | 202 | Start Exhaust |  |
| (1) Gasoline | NOx | 302 | Start Exhaust |  |
| (1) Gasoline | HC | 112 | Evap Fuel Vapor Venting |  |
| (1) Gasoline | HC | 113 | Evap Fuel Leaks |  |

i.) Scroll down through all of the records noted in black fonts and compare them to the default records in red fonts.  There should be no duplicate records.  If so, go back through the steps noted for the inputs and scan the files again to make certain all records are accounted for and correct.  Delete any empty rows or duplicate records from the table.

j.) The IMCoverage table is now complete and ready to be copied into MOVES workbook for uploading into the MOVES model.

NOTE: Always copy the IMCoverage table to the workbook used for MOVES inputs.  Do not remove the completed version of the IMCoverage table from the YYYY IM Input development.xlsx file.