Warewashing Process (3-Bay)

1	WASH

110ºF minimum

2	RINSE

Fresh water.  Running or dip

3	SANITIZE

Chlorine:

| Concentration Range | Minimum Temperature |  |
| --- | --- | --- |
| ppm | pH 10 or less | pH 8 or less |
| 25-49 | 120°F | 120°F |
| 50-99 | 100°F | 75°F |
| 100 | 55°F | 55°F |

Quaternary Ammonium Compounds (QAC):

Concentration range may vary by product. Generally used between 150-400ppm. Please see product label for manufacturer’s use instructions. Use QAC test papers to check concentration. Minimum temperature of 75oF

4	AIR DRY