# 3-D Modeling Deliverables

Deliverables due at Advertise

# Contracts Package .dgn’s

- All sheeted dgn’s that are a part of the Contract Package (e.g. 001_Title.dgn; 002_Typical.dgn; etc.)

- All Base and Reference drawings (2D .dgn’s)

  - HDPlan.dgn, BDPlan.dgn, RWPlan, GEOPLAN.dgn, UTPLAN.dgn  (as appropriate)

  - MDOT Standard Reference dgn’s

    - Topo.dgn (2D .dgn’s)

    - Points.dgn (2D .dgn’s)

    - Contours.dgn (2D .dgn’s)

    - Text.dgn (2D .dgn’s)

    - Alignments.dgn (2D .dgn’s)

    - Highway.dgn (2D .dgn’s)

    - Bridge.dgn (2D .dgn’s)

    - Wetlands.dgn (2D .dgn’s)

# 3D Alignments .dgn’s

- 3D-Alignments.dgn : Created from all Centerline Alignments using the InRoads  Geometry > View Geometry > 3-D Alignment  command. (3D .dgn)

- Control_Alignments.dgn : Created from all “non-Centerline” Alignments, used to control features within the Design Model. (e.g. Special Ditch Alignments, Alignments used to control Roadway Widening, Alignments used to Control Display Rules such as Driveway Controls, etc.) (3D .dgn)

# 3D Alignments LandXML’s

- 3D-Alignments.xml : LandXML export of the Geometry Project. The Geometry Project should be purged of alignments that were not used in the Final Design. (3D.xml)

- Control_Alignments.xml : LandXML export of the “non-Centerline” Alignments, used to control features within the Design Model. (3D .xml)

# Existing Ground Surface

- LandXML export of the GROUND.dtm  used to interface the Proposed Surface. (.xml)

- 3D .dgn of the GROUND.dtm (3D.dgn)

# Proposed Design Surface(s)

- Finish Grade-Triangles.dgn – 3D .dgn of the Finish Grade.dtm created from the Surface > View Surface > Triangles command within InRoads, or equivalent. (3D.dgn)

- 3d-Design-Features.dgn : Display of the 3D-Features of the Design Surface Model, such as Edge of Travelway, Edge of Pavement, Sidewalk Features, Ditchlines, Cut/Fill Features; etc. (3D .dgn)

- LandXML export of the Finish Grade.dtm (triangles only).   (3D.xml)

- LandXML export of the Finish Grade.dtm features.   (3D.xml)

The finished grade surface was created from contours shown on the grading plan. The approach to modeling was to model each leg of the roundabout separately with contours drawn by hand to smooth the transition from the roadway legs to the circle. The individual surfaces that were created for each leg contain their own 3-D features. The features for each of these surfaces were displayed in the 3D – Design – Features.dgn

- Subgrade.dgn – 3D .dgn of the Subgrade.dtm created from the Surface > View Surface > Triangles command. (3D.dgn)

There was not a separate surface made for the subgrade.

- LandXML export of the SubGrade.dtm (triangles only).  (3D.xml)

There was not a separate surface made for the subgrade.

- LandXML export of the SubGrade.dtm features.  (3D.xml)

There was not a separate surface made for the subgrade.

# InRoads Design File(s)

- Preference File (.xin) - Copy of the Preference File used for the Project

- Geometry Project(s) (.alg) – Copy of the Alignment file.

- Surface File(s) (.dtm) – All surface files used in the Project; including Alternate Surfaces. (GROUND.dtm, Design.dtm, Subgrade.dtm, etc.)

- Template File(s) (.itl) – To contain all Templates used in the Project.

- Roadway Designer File(s) (.ird) – Final Roadway Designer files used for the Project.

- InRoads Project File (.rwk)

This project did not use a .rwk file.