PROJECT INFORMATION

| Bridge Name and Number: |  |
| --- | --- |
| WIN: | [Status] |
| Project Manager: |  |
| Town(s): |  |
| Route Carried: | Route 16 and 27 |
| Feature Intersected: | Reed Brook |
| Programmed Scope: | Bridge Replacement |

Recommended Scope:

☒  Bridge Replacement

☐  Bridge Improvement

☐ Preservation/Rehabilitation

☐ Deck Replacement

☐ Superstructure Replacement

☐  Other:

ROADWAY

| Design Speed (mph): | 55 |
| --- | --- |
| Corridor Priority (1-5): | 2 |
| NHS: | No |
| Current AADT: | 3630 |
| Approach Lane: | 11’ paved |
| Approach Shoulder: | 3’ gravel |
| Sidewalk(s): | 4’ north side |
| Alignment: | (Currently meets XX mph) (Note deficiencies) |

EXISTING BRIDGE DATA

| Span Layout: | 15’ |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rail-to-Rail Width: | 28’ |  |  |  |  |  |
| Vertical Clearance: | 2-feet above pipe @ Q50 (or Hw/D if not full) |  |  |  |  |  |
| Year Built: | 1972 |  |  |  |  |  |
| Culvert Type: | Corrugated steel pipe arch |  |  |  |  |  |
| Condition Ratings: | Deck |  | Superstructure |  | Substructure |  |
|  | Scour |  | Culvert | 4 |  |  |
| Maintenance Issues: | (Debris, scour, deformed inlet, etc.) |  |  |  |  |  |
| Additional Comments: |  |  |  |  |  |  |

OTHER DATA

| Purpose & Need: | (provided by ENV in cooperation with Bridge Program*) |
| --- | --- |
| ENV Sizing Criteria: | 32’ BFW, 39’ recommended to meet 1.2x BFW |
| Geotechnical Summary: | (General subsurface material and depth to bedrock) |
| Additional Borings Required ☐ Additional Geotechnical Evaluations Required ☐ |  |
| Other: |  |

*Most projects can use the following Purpose: To address the structural condition of the bridge and improve safety at the site.

PROPOSED BRIDGE DESIGN CRITERIA

| Span Layout: | 60’ +/- single span |
| --- | --- |
| Bridge Skew: | 20 |
| Out-to-Out Width: |  |
| Curb-to-Curb Width: |  |
| Sidewalk Width(s): |  |
| Vertical Clearance: | 2.38’ @ Q50 (list critical design criteria clearances) |
| Alignment Changes: | 1.25’ vertical increase to improve sight distance (meets comfort criteria) |
| Superstructure Type: | Precast, prestressed concrete NEXT beams |
| Substructure Type(s): | Concrete integral abutments |
| Foundation Type(s): | Spun pipe piles |
| Wearing Surface Type: | HMA |
| Joint Type(s): | None |
| Railing Type: | 3-bar steel |
| Design Load: | HL-93 mod. |

PROJECT DETAILS

| Complete Streets: | Matching existing corridor |
| --- | --- |
| ROW Impacts: | (yes or no) |
| Utility Impacts: | (relocate waterline; relocate aerials) |
| MoT Method: | One lane temporary bridge |
| Construction Duration: | One season plus final paving and landscaping the following spring |
| Design Exceptions: |  |
| Public Involvement: | (note what level or strategy will be used) |
| Municipal Agreement(s): | (yes or no, list which ones) |
| Forecast Advertise: | Q3 2026 |
| Other: |  |

ADDITIONAL COMMENTS:

- Here, we could also list other alternatives broadly considered and dismissed

- We could also include any rough plans if available

ENVIRONMENTAL COORDINATION

[INSERT HERE]

This would be the standard sheet from the Environmental Office.

PROJEX FINANCE SUMMARY

[PASTE HERE]

Once the PM updates the estimate in Projex, this would be a screen shot of that page cropped to fit.

APPROVAL:

☒  Proceed to PIC		☐  Develop PDR		_________________________________________________________

Assistant Bridge Program Manager				Date