Fire Department Ladder Safety Checklist

(NOTE: This is a guideline and not Bureau of Labor Standards-required policy.)

Ladder Hazards

Improper placement and defective parts usually cause fire department ladder accidents. Proper placement is a training issue, while defective parts can be addressed through a proper and thorough inspection. Prevention starts with good standard operating guidelines, adequate ladder maintenance, and inspection procedures.

- Ladder Maintenance

Wood Ladders – Should be protected with a clear sealer varnish, shellac, linseed oil or wood preservative. Wood ladders should not be painted.  Paint will hide defects.

Fiberglass ladders – Should have a surface coat of lacquer maintained. If it is scratched beyond normal wear, it should be lightly sanded before applying a thin coat of lacquer.

- Ground ladder Duty rating

|  | Maximum Load |  |
| --- | --- | --- |
| Type | lbs. | Kg |
| Folding ladders | 300 | 136 |
| Pompier ladders | 300 | 136 |
| Single & roof ladders | 750 | 340 |
| All extension ladders | 750 | 340 |
| Combination ladders | 750 | 340 |

Fire Service Ladder Inspection Guideline

| General | O.K. | Needs Repair | Date Repaired |
| --- | --- | --- | --- |
| Loose steps or rungs (consider loose if they can be moved at all with one hand)? |  |  |  |
| Loose nails, rivets, screws, bolts or other metal parts? |  |  |  |
| Cracked, split, or broken uprights, braces, or rungs? |  |  |  |
| Slivers on uprights, rungs, or steps? |  |  |  |
| Worn, rotted or frayed ropes? |  |  |  |
| Surface corrosion? |  |  |  |
| - Wood | O.K. | Needs Repair | Date Repaired |
| Cracks or Rot? |  |  |  |
| Painted Surfaces? |  |  |  |
| Broken or missing rungs? |  |  |  |
| Hardware in poor condition? |  |  |  |
| - Metal or Aluminum Extension ladders | O.K. | Needs Repair | Date Repaired |
| Any rough burrs or sharp edges? |  |  |  |
| Loose joints or bolts? |  |  |  |
| Faulty welds or cracks? |  |  |  |
| Loose, broken, or missing extension locks? |  |  |  |
| Defective locks that do not seat properly while extended? |  |  |  |
| Bent beams or rungs? |  |  |  |
| Unusual wear? |  |  |  |
| Wire rope on 3 and 4 section ladders is snug? |  |  |  |
| - Roof Ladders (in addition to above if applicable) | O.K. | Needs Repair | Date Repaired |
| Roof hooks sharp and operate properly? |  |  |  |

Note: All ground ladders should be inspected in accordance with the manufacturer’s recommendations.