The Maine 9-1-1
Addressing Officer Manual

Eighth Edition – June 2026

Public Utilities Commission

Emergency Services Communication Bureau

18 State House Station

Augusta, Maine 04333-0018

http://www.maine.gov/maine911

The Enhanced 9-1-1 Addressing Officer Manual - Edition 8

Table of Contents

1. Introduction	3

1.1 Purpose of the 9-1-1 System	3

1.2 Purpose of this Manual	3

1.2.1 Using the Manual	3

1.3 Contact Names and Numbers	3

1.4 Glossary of 9-1-1 Terms	5

2. The Addressing Officer	7

2.1 Municipal Home Rule Powers	7

2.2 Addressing and Road Naming Authority	7

2.3 Administrative Rules Relating to the AO	7

2.4 General Responsibilities of the Addressing Officer	7

2.5 Appointing or Changing an Addressing Officer	8

2.6 Addressing Officer Confirmation Form	9

3. Maintaining an Address System	10

3.1 Assigning a New Address	10

3.2 Changing an Existing Address	10

3.3 Address Numbering Recommendations	10

4. Maintaining the Emergency Road Network	12

4.1 Partnership Between ESCB and MaineDOT	12

4.2 Road Naming Disputes	12

5. Emergency Service Zones (ESZs)	13

5.1 Sample Emergency Service Zones	14

5.2 Changing an ESZ	15

5.3 Changing PSAPs	16

5.4. Dispatching Changes	16

5.5 Emergency Service Zone Designation Form	17

6. Publicizing 9-1-1	18

6.1 Maine Statutes Designating 9-1-1 as the Emergency Telephone Number	18

6.2 The ESCB Website	18

Appendix A	19

Appendix B	24

Appendix C	25

# 1. Introduction

## 1.1 Purpose of the 9-1-1 System

The 9-1-1 System is an emergency communication system that automatically displays the address of a caller at a 9-1-1 emergency center when called from a wireline telephone. If a caller is hysterical, becomes unconscious, or hangs up, the call taker will know where to send help. This is especially helpful if the caller does not speak English or may be unfamiliar with their location. Dialing 9-1-1 from a wireless phone displays an approximate location on a map based on latitude and longitude coordinates.

The Emergency Services Communication Bureau (ESCB) is responsible for the management and maintenance of statewide 9-1-1 service. The ESCB currently contracts with Fidium for 9-1-1 equipment, software, database management and support. Therefore, when a document refers to the 9-1-1 Service Provider it is referring to Fidium.

The Emergency Services Communication Bureau/GIS Department provides mapping and addressing support to towns and Public Safety Answering Points (PSAPs).

## 1.2 Purpose of this Manual

This manual is a reference for Addressing Officers (AOs) to assist them in their duties of managing addresses, 9-1-1 roads, and other emergency service information for their city, town, community, or unorganized territory. Most communities have completed the initial 9-1-1 addressing efforts; i.e. the creation of locatable addresses which affords the Maine population the full benefits of a 9-1-1 system. To keep the system working properly, the address information and emergency road network must be regularly maintained. This maintenance is the responsibility of the Addressing Officer (AO).

This manual does the following:

1. Identifies the responsibilities of the Addressing Officer;

2. Instructs the AO on how to maintain an address network;

3. Explains the methodology for providing road network and Master Street Address Guide (MSAG) changes to the 9-1-1 system; and

4. Describes the process of keeping the 9-1-1 database current.

### 1.2.1 Using the Manual

Instructions critical to this system are bolded and are to be followed by all AOs. The manual includes some forms; however, it is wise to visit the ESCB website for the most up-to-date versions as these may change over time. For more information on the 9-1-1 program, visit the ESCB website:  www.maine.gov/maine911/

## 1.3 Contact Names and Numbers

There are three groups who have a part in 9-1-1 addressing and emergency response information management for the ESCB.  Below is a list of areas of responsibility for each group and their contact information. If you are not sure who the appropriate group would be to contact with your question, please call your ESCB/GIS Coordinator.

For questions on the following:

- Responsibilities of the AO.

- Designating an AO.

- Designating or Changing Emergency Service Zones and Emergency Responders.

- Telephone Company Coordination Issues.

- Database or error correction issues.

- Changes or additions to the 9-1-1 databases

- Training for new AOs

- Indicating road updates on maps

- Assigning new addresses

- Assistance with addressing or location information

- Issues and questions regarding mapping

Emergency Services Communication Bureau

18 SHS

Augusta, ME 04333-0018

Toll Free: 1-800-665-2830
Toll Free Fax: 1-866-710-7381

Email: PUC.GIS911@maine.gov

For questions about the 9-1-1 databases such as:

- Requesting copies of an MSAG or telephone number list

Fidium Email: me911db@fidium.com

For Postal questions:

Contact your local Post Office or the USPS Address Management System office at 207-482-7176.

## 1.4 Glossary of 9-1-1 Terms

9-1-1:

A three-digit telephone number to facilitate the reporting of an emergency requiring response by a public safety agency.

9-1-1 Service Provider

The 9-1-1 Service Provider for Maine is Fidium, by contract with the ESCB. The 9-1-1 Service Provider supplies the 9-1-1 equipment, software, database management, system maintenance, and support.

Addressing Officer (AO):

The AO is a representative designated by the community to be responsible for all issues involving the development and maintenance of 9-1-1 address information for the municipality. In areas of the state where unorganized territories exist, Counties will designate a representative to manage township addresses.

Automatic Location Identification (ALI):

The ALI is the automatic display at the PSAP of the caller’s telephone number, the address/location of the telephone, and supplementary emergency services information of the location from which a call originates.

Automatic Number Identification (ANI):

ANI is the telephone number associated with the call origination, originally associated with the access line of the caller.

Community

A community is any municipality, plantation, or unorganized township in Maine.

Emergency Services Communication Bureau (ESCB or Bureau):

The ESCB is a bureau within the Maine Public Utilities Commission (PUC) and is responsible for provisioning and managing 9-1-1 in Maine. It is often referred to as the Maine 9-1-1 Bureau.  www.maine.gov/maine911

Emergency Road Network (9-1-1 Road Network)

The 9-1-1 Road Network includes all public and private roads that emergency vehicles might travel when responding to a call. The roads to be included in a community’s emergency network are determined by the municipal AO. These roads will have address ranges assigned to them.

Emergency Service Number (ESN):

An ESN is a five digit number that represents a unique combination of emergency service agencies (police, fire, EMS, transport) for a specific Emergency Service Zone (ESZ).

Emergency Service Zone (ESZ):

An ESZ defines a geographic territory consisting of a unique combination of town, law enforcement, fire department, EMS, and ambulance or medical transport coverage areas. Each town in Maine has at least one ESZ (polygon). Some towns have more than one because different emergency service providers are needed to respond to different parts of the town. An ESZ is identified by an Emergency Service Number (ESN).

Geographic Information System (GIS):

GIS is a system for capturing, storing, displaying, analyzing, and managing data and associated attributes which are spatially referenced.

Global Positioning System (GPS):

GPS (Global Positioning System) is a space-based navigation system that provides location and time information in all weather conditions, anywhere on or near the Earth where there is an unobstructed line of sight to four or more GPS satellites.

Next Generation 9-1-1 (NG9-1-1)

The 9-1-1 system has evolved from Basic 9-1-1, to E9-1-1, and now NG9-1-1. The NG9-1-1 system uses newer technologies such as closed IP networks to process 9-1-1 calls. The NG9-1-1 system puts more emphasis on GIS to route calls to the appropriate Public Safety Answering Point. NG9-1-1 currently accepts text-to-9-1-1 messaging if the caller has an active text plan. Some upgrades and standards in the telecommunications industry are still being developed to use the full functionality of NG9-1-1.

Public Safety Answering Point (PSAP):

A PSAP is a centralized office where 9-1-1 calls are answered. In Maine there are currently twenty-five (25) PSAPs. They may be State (Department of Public Safety), Municipal (town police department), or County departments operating the 9-1-1 call-answering centers.

Voice Over Internet Protocol (VoIP):

VoIP is a type of telephone service that uses Internet Protocol (IP) to send and receive voice calls. VoIP 9-1-1 calls will display both an address and a latitude/longitude coordinate. (***Important note: The address provided is based on the registration of the phone. Users of VoIP phones should update their address anytime they relocate the phone. ***)

Wireless Telephone Service (cell phones):

Wireless telephone service use cell-towers to send and receive calls. Wireless calls display an ALI with a latitude/longitude coordinate which is used to plot the call location on the map.

Wireline (or Landline):

Wireline service equates to traditional copper line telephone service. With wireline service, the address is associated with a telephone number which will display in the ALI window on the calltaker’s screen. The call will map using the physical address associated with that telephone number.

# 2. The Addressing Officer

## 2.1 Municipal Home Rule Powers

The Maine Constitution, Article VIII, Part Second grants municipalities authority to govern themselves. Municipal home rule powers are implemented under 30-A M.R.S. §2101, and §3001 grants municipalities the authority to create ordinances.

## 2.2 Addressing and Road Naming Authority

Home rule powers give municipalities the legal authority to create physical addresses and name roads. The ESCB strongly recommends that municipalities enact addressing ordinances. Plantations are granted authority under 30-A M.R.S §7062. Counties with unorganized territories are granted authority under 30-A M.R.S. §7501(8).

## 2.3 Administrative Rules Relating to the AO

Pursuant to 25 M.R.S. § 2926(3), the ESCB has created administrative rules for the AO that can be found at https://www.maine.gov/mpuc/legislative/laws-rules under ESCB Chapter 1.

## 2.4 General Responsibilities of the Addressing Officer

The Addressing Officer (AO) is responsible for creating and providing address information to ESCB and must have signature authority on behalf of the community. Therefore, each municipality and plantation must designate an official AO. County Commissioners with populated unorganized territories must also designate an AO to serve its unorganized townships.

Typical responsibilities of the AO include:

- Assigning addresses and communicating new addresses to town residents.  For new addressable sites, a physical address must be provided to the ESCB to be added to the 9-1-1 address database. The AO will assign addresses to the property owner and will notify them in writing of their new address.

- Providing correct road name and number range information.  The ESCB should be notified of any new road, road name change, address range change, or road maintenance change (town, private, or state-aid).

- Verifying and/or correcting addresses when contacted by the ESCB.  This critical step helps to ensure that our database contains the correct name and location of an emergency caller. The ESCB may reach out to the AO when addresses supplied by customers or telephone service providers are in question and need to be verified, or when cell tower addresses are needed.

- Indicating the community’s Emergency Service Zones (ESZs).  An ESZ is a geographic area in a community comprised of specific roads and ranges of addresses that are served by a unique combination of police, fire, EMS, and medical transport agencies. Most towns will have just one ESZ because the same police, fire, EMS, and medical transport agencies service the entire town. However, a town that is served by two fire departments would have two ESZs. As an example, a town with a lake in the middle of it may have one fire department respond to the east side of the lake while another fire department may respond to the rest of the town based on access roads. It is the AO’s responsibility to notify the ESCB in writing when the town intends to make any changes in emergency response coverage.

- Other duties of an AO may include:

- Answering citizen’s questions about addressing.

- Monitoring local development activities with the planning board, Code Enforcement Officer (CEO), town planner, or city engineer for the creation of new roads and subdivisions, and to assign new addresses.

- Calculating, measuring, and issuing addresses.

- Updating the community’s address database.

- Record when addresses and road names were changed.

- Notify other municipal departments of any address or road name changes.

- Notify local Post Office of new or changed addresses.

In many cases, the AO is the tax assessor, code enforcement officer, public works director, or in some cases, the chair of the Board of Selectmen. These people have firsthand knowledge of new developments within the community. Other communities install police, fire, or rescue chiefs as the AO because these people understand the importance of complete and accurate addressing information. If an emergency department head is assigned this task, then an internal process may need to be developed to keep this person informed of developments within the town.

## 2.5 Appointing or Changing an Addressing Officer

Anytime there is a change of AO a new form must be completed and returned to the ESCB via email at info911@maine.gov.  The current version of the “Addressing Officer Confirmation Form” may be found on the next page. The most current form is also available on the ESCB website at: https://www.maine.gov/maine911/community-addressing/find-addressing-officer/responsibilities

To find out who the current AO is for your community visit the ESCB website at:  https://www.maine.gov/maine911/community-addressing/find-addressing-officer

When filling out the AO Appointment Form please indicate the municipality or county (in the case of unorganized territories) that the appointment represents. To avoid confusion between similarly named towns and counties, please indicate whether the appointment is at the county or town level by checking the appropriate box on the form.

Some towns assign an alternate AO.  Please check the box indicating whether the person is being assigned as a Primary AO or as an Alternate AO.

| ## 2.6 Addressing Officer Confirmation Form Emergency Services Communication Bureau |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ADDRESSING OFFICER (or Alternate AO) CONFIRMATION FORM |  |  |  |  |  |
| The following individual being designated as an Addressing Officer is authorized to approve and provide addressing information to the Emergency Services Communication Bureau for 9-1-1 purposes. Please Note: This form may also be used for an Alternate AO where more than one person is needed to cover a given territory. Please check the box indicating Primary AO or Alternate AO assignment. |  |  |  |  |  |
| Municipality and / or County Represented NOTE: Indicate if this appointment is for a Town (single township) or County AO (multiple towns/townships), by checking the Town or County box to the right. (please check the box) |  |  |  |  |  |
| Municipality or County |  |  |  |  | ☐ Town ☐ County |
| Addressing Officer Information |  |  | ☐ Primary AO or ☐ Alternate AO |  |  |
| First Name | MI | Last Name |  | Suffix | Title |
| Address |  |  |  |  |  |
| City |  |  |  | State | Zip |
| Phone |  |  |  | Fax |  |
| Email |  |  |  |  |  |
| Authorization |  |  |  |  |  |
| Signature of Authorizing Official* |  |  |  | Title |  |
| Printed Name of Authorizing Official |  |  |  | Date |  |
| * Authorizing Officials can either be the Chief Administrative Officer or Chief Elected Official. |  |  |  |  |  |

Please sign and return to:

Mail: Emergency Services Communication Bureau, 18 SHS, Augusta, ME 04333-0018

Email:  info911@maine.gov  Fax: 207-287-1039

# 3. Maintaining an Address System

## 3.1 Assigning a New Address

- When a request is made for an address assignment, the property location should be obtained along with any identifying structures or landmarks that may help locate the new structure requiring the address. New addresses should always be verified in the field to be sure that addresses have been assigned sequentially and on the correct side of the road.

- Use Appendix C, Road Measurements and Property Numbers tables, to calculate addresses (or house numbers).

- Review your calculations to ensure that no other addresses have been impacted with the addition of this newly assigned address. You may need to check road-ranges and road-names or see if it is cause for a new road. The address range should be checked between intersections as well as from the road’s origin to the road’s end. If it is a new road or a road’s address range has been affected, you will need to notify ESCB.

- Assign the address and notify the customer in writing of their new address (see Appendix B for a sample address-notification letter.)

- Send a copy of the notification letter to the following agencies or departments.

- To the local postmaster by mail and on town letterhead.

- The tax clerk or assessor.

- The town/city clerk.

- The registrar of voters.

- Other appropriate local agencies or internal departments.

## 3.2 Changing an Existing Address

Changing existing addresses should only be done on records where previously issued addresses are now causing problems for emergency responders, postal delivery, or where the building density has increased to the point where the initial addressing increments no longer work. Although a community has the authority to change a road name, it is not popular among residents or emergency responders. If you have a situation where existing residents’ addresses need to be fixed, feel free to call the ESCB for help.

## 3.3 Address Numbering Recommendations

Recommendations for specific numbering situations are below. The general logical order of address elements should follow Postal Service conventions e.g., 100 W Main St, Apt 201.

- Apartments: Assign a primary road address, with numbers (not letters) as secondary location indicators, e.g., 111 Main St, Apt 1. Use apartment numbers to indicate the floor location, e.g., Apt 303 (or 3C) is the third apartment on the third floor.

- Circular roads: Numbering should begin at the point where emergency responders are most likely to enter the road, so numbers will ascend as responders search for a location. Where there is no obvious end with greater traffic flow, it is recommended that the starting point be selected at the end closer to the designated numbering origin. Whatever method is selected, apply it consistently within the community.

- Condominiums: Assign addresses as though they were apartments or individual houses along a road.

- Cul-de-sacs: Those without buildings in the center portion should be numbered as if the center line of the street bisects the cul-de-sac with odd numbers on the right and even numbers on the left. The numbers meet at the far end of the center area.

- Duplexes: Assign two separate numbers or address as with apartments.

- Mobile home parks: The park can be assigned one address on the main road, followed by a numeric unit or lot number for the individual homes, such as an apartment building would be numbered. Another option would be to name all the roads and number all the homes within the park, as any property would be numbered on any road.

- Office suites: Numbers should be assigned with a primary road address, followed by a numbered (not lettered) secondary location indicator, e.g., 325 Memorial Drive, Suite 312. Suite numbers should also be used to indicate which floor location.

- Shopping center: Numbers can be assigned to businesses within the range available to the property.

- Islands: On river, lake, or coastal islands with roads, properties should be numbered as any area with roads. On islands without roads, properties can be numbered as follows: Pick a compass point on the island, 0 degrees North for example. Assign odd addresses to lots on the west side of the island, with numbers ascending towards the south. Assign even numbers to lots on the east side, with numbers also ascending towards the south. Use the minimum water frontage requirement for lots on the island as a numbering interval to leave room for additional structures to be built on vacant lots. The road name will be that of the island, resulting in an address such as “4 Goat Island.” If an island does not have a name, consider naming it as a formerly unnamed road might be.

# 4. Maintaining the Emergency Road Network

To ensure a properly working 9-1-1 system, the road network, and corresponding address-range information must be updated as changes occur. Please notify the ESCB when any of the following occurs:

- A new road is created.

- A road name is changed.

- A road is extended.

- A road is closed.

- An address-range change occurs.

- A road is wrongly located on a map.

- The maintenance of a road changes (public, private, summer-only, winter-only, or year-round maintenance.)

Whenever a new road and address-range is developed it’s generally a good idea to have the United States Postal Service (USPS) review it, particularly if a town shares a Zip code with other towns. This may help to prevent mail delivery problems in cases where the same road name is used by multiple towns served by the same Post Office.

## 4.1 Partnership Between ESCB and MaineDOT

In 2009, the ESCB joined with MaineDOT in maintaining a statewide road inventory. This partnership coordinated work between two separate state agencies in an effort to minimize duplication while improving accuracy. If your town changes the maintenance of a road (public to private or summer-only to year-round maintenance), even if the road was not physically changed, that change should be reported to ESCB. Updating road information as changes occur will assist municipalities in receiving correct Local Road Assistance Program payments. The payment amounts are based on the number of lane-miles of public roads as tracked by MaineDOT.

To make this work, a single road update process was developed. This is why the road jurisdiction was added to the forms and to the change request process. AOs must now coordinate with their Public Works/Road Commissioner to keep road jurisdiction updated.

## 4.2 Road Naming Disputes

30-A M.R.S. §3110. Unless otherwise provided by local ordinance or charter, when there is a dispute over the naming of a town way, private way or private road for 9-1-1 purposes, the decision of the municipal officers is final.

# 5. Emergency Service Zones (ESZs)

An Emergency Service Zone (ESZ) is a geographic area made up of a unique combination of police, fire, rescue, and ambulance services assigned to respond to emergencies within that zone or community. In most towns there is only one zone but in some towns, due to the proximity of rivers, lakes, streams, hills or mountains bisecting the community there may be two or more zones. A town could have two ESZs even when the police and/or fire responders are the same but the ambulance coverage is maintained by two separate companies. An ESZ may contain many roads or a single residence based on access. A road may also be split into more than one ESZ based on address-ranges which are served by multiple responders.

Addressing Officers must consider each unique combination of emergency response agencies within the borders of their town when addressing parcels. In cases where multiple responders exist for a given town, a map depicting these unique coverage areas or a list by street and address-ranges become useful tools in maintaining town records. An AO should consult with their police and fire departments and ambulance officials to assist with the development of these tools because this information is provided to dispatchers in the event of actual 9-1-1 calls. “Hot keys” at the PSAPs are programmed with the responder agency information that you provide, making it necessary to be 100% accurate.

If you have questions concerning these designations, then please call the ESCB.

## 5.1 Sample Emergency Service Zones

| Example 1: The same combination of police, fire, & rescue services cover the entire town of Winthrop. In this situation, there is only one ESZ and they are all dispatched by the local Police Dept. |  |  |
| --- | --- | --- |
| Emergency Service Zone Number (5-digits) | Name of Service by Type | Name of Dispatch Agency For Service |
| 00255 | Police |  |
|  | Winthrop PD | Winthrop PD |
|  | Fire |  |
|  | Winthrop FD | Winthrop PD |
|  | EMS or First Responder: |  |
|  | Winthrop Ambulance | Winthrop PD |
|  | Ambulance or Medical Transport: |  |
|  | Winthrop Ambulance | Winthrop PD |

| Example 2: In Glenburn, there are two different ambulance services, but the same sheriff and fire departments serve the entire town dispatched by the same PSAP; so Glenburn has two different ESZs. |  |  |
| --- | --- | --- |
| Emergency Service Zone Number (5-digits) | Name of Service by Type | Name of Dispatch Agency For Service |
| 00774 | Police: |  |
|  | Penobscot SD | Penobscot RCC |
|  | Fire: |  |
|  | Glenburn FD | Penobscot RCC |
|  | EMS or First Responder: |  |
|  | Old Town Rescue | Penobscot RCC |
|  | Ambulance or Medical Transport: |  |
|  | Old Town Rescue | Penobscot RCC |
| 00399 | Police: |  |
|  | Penobscot SD | Penobscot RCC |
|  | Fire: |  |
|  | Glenburn FD | Penobscot RCC |
|  | EMS or First Responder: |  |
|  | Glenburn Rescue | Penobscot RCC |
|  | Ambulance or Medical Transport: |  |
|  | Northern Light EMS | Penobscot RCC |

| Example 3: In the Moosehead Lake area Piscataquis County Islands do not have a police dept. so they rely on the Sheriff Dept (SD) to cover police calls & use the Maine Forest Service for fire coverage. |  |  |
| --- | --- | --- |
| Emergency Service Zone Number (5-digits) | Name of Service by Type | Name of Dispatch Agency For Service |
| 00815 | Police |  |
|  | Piscataquis SD | Piscataquis RCC |
|  | Fire |  |
|  | Maine Forest Service | Maine Forest Service |
|  | EMS or First Responder: |  |
|  | Rockwood Fire Rescue | Somerset RCC |
|  | Ambulance or Medical Transport: |  |
|  | Rockwood Fire Rescue | Somerset RCC |

| Example 4: The town of Windham has a remote section of town that is actually is only accessible by the town of Cumberland roads. Therefore, there are two ESZs/ESNs for this town. |  |  |
| --- | --- | --- |
| Emergency Service Zone Number (5-digits) | Name of Service by Type | Name of Dispatch Agency For Service |
| 00074 | Police: |  |
|  | Windham PD | Cumberland RCC |
|  | Fire: |  |
|  | Windham Fire Rescue | Cumberland RCC |
|  | EMS or First Responder: |  |
|  | Windham Fire Rescue | Cumberland RCC |
|  | Ambulance or Medical Transport: |  |
|  | Windham Fire Rescue | Cumberland RCC |
| 00075 | Police: |  |
|  | Windham PD | Cumberland RCC |
|  | Fire: |  |
|  | Cumberland Fire Rescue | Cumberland RCC |
|  | EMS or First Responder: |  |
|  | Cumberland Fire Rescue | Cumberland RCC |
|  | Ambulance or Medical Transport: |  |
|  | Cumberland Fire Rescue | Cumberland RCC |

The importance of having correct ESZ information is critical to 9-1-1 operations. Therefore, the ESCB requests that AOs verify ESZ information annually and notify the Bureau whenever there is a change in emergency service providers, the dispatching of services, or to an emergency service zone coverage area. Use the form on page 17 for this purpose.

## 5.2 Changing an ESZ

Whenever there is a change to a service provider, please fill out the 	Emergency Service Zone Designation Form. The communication should include:

- Which services are affected by the change (Police/Fire/EMS or First Responders/Ambulance or Transport services),

- The name of the new service,

- The roads affected (if not the entire town), and

- The effective date of the change.

If you are making a change for a town with multiple ESZs and need to modify the roads that are assigned to each ESZ, then please be sure to submit a list and/or map of the affected roads along with address ranges for each ESZ. This documentation should accompany the form on page 17.

## 5.3 Changing PSAPs

Communities contract with a PSAP for call answering services. It is possible to change PSAPs, but only with approval from the Public Utilities Commission (PUC). This is a formal process which requires changes to the 9-1-1 system maintained by the ESCB. If your community wishes to change PSAPs, please notify the ESCB at least two months prior to the expected effective date. The ESCB will instruct you on how to file a request with the PUC.

## 5.4. Dispatching Changes

Service provider agencies (Ambulance, Fire, Police, or First Responders) may also change dispatch agencies. Since these updates require modifications to the databases, it is important that the ESCB be notified well in advance of the desired effective date. Dispatch changes do not require PUC approval and as an AO you may not always be privy to these requests. However, the ESCB may contact you for documentation.

## 5.5 Emergency Service Zone Designation Form

Instructions: For each ESZ, please note the name of the emergency service provider by type, and name the agency that dispatches the service. Each unique geographically split combination of police, fire, and rescue should represent a separate zone. See attached examples. Feel free to reproduce the form as necessary.

If multiple ESZs are indicated, attach a list of streets and address ranges in your town and the appropriate ESN (ESZ number) for each. Roads may be split between ESZs by also splitting the address ranges. Mapping assistance is available from Emergency Services Communication Bureau/GIS if needed PUC.GIS911@Maine.gov or by calling 1-800-665-2830.

Municipality: ___________________________________________________________

Effective Date: _________________________________________________________

Name & Title of Person Completing Form: ___________________________________

Signature: __________________________________

| Emergency Service Zone Number (5 digits) | Name of Service by Type | Name of Dispatch Agency for Service |
| --- | --- | --- |
| (Ex: 00284) | Police: |  |
|  | Fire: |  |
|  | EMS or First Responder: |  |
|  | Ambulance or Medical Transport: |  |
|  | Police: |  |
|  | Fire |  |
|  | EMS or First Responder: |  |
|  | Ambulance or Medical Transport: |  |
|  | Police: |  |
|  | Fire |  |
|  | EMS or First Responder: |  |
|  | Ambulance or Medical Transport: |  |

Return to:  9-1-1 Database Manager, ESCB, 18 SHS, Augusta, ME 04330-0018. Fax: 207-287-1039.

Email:  info911@maine.gov

# 6. Publicizing 9-1-1

## 6.1 Maine Statutes Designating 9-1-1 as the Emergency Telephone Number

25 M.R.S §2932 designates 9-1-1 as the emergency telephone number.

## 6.2 The ESCB Website

For current information about Maine’s 9-1-1 system, visit the ESCB website at https://www.maine.gov/maine911/ Features include:

- Community information

- Forms and Guidebooks

- Public Information

- Statutes and pending legislation

- Administrative Rules

- Telephone company information

- PSAP resources

# Appendix A

Frequently Asked Questions by AOs

This list attempts to include questions you may have had or questions you may have been asked by your constituents. More information may be found at:  https://www.maine.gov/maine911/using-911/faq

Addressing Questions

Why is addressing a critical part of 9-1-1?

A 9-1-1 caller gets his/her call routed directly to the correct PSAP so that emergency services may be dispatched immediately. Accuracy of the emergency caller's address helps dispatchers send the correct emergency services quickly to the person in need.

Do communities have the authority to create physical addresses?

Yes. Every Maine community has the legal authority to create physical addresses under Municipal Home Rule Powers, Article VIII, Part 2, §1 of the Constitution of the State of Maine and Title 30-A M.R.S.A §3001.  In the unorganized territories, the County has the authority to create addresses.

What happens if a 9-1-1 caller's address is not correct?

There are procedures in place at the 9-1-1 call centers (PSAPs) which call-takers must follow to report address discrepancies. Once a call-taker is made aware of an error, the error is reported to the ESCB where these issues are researched and corrected. An AO may be contacted to help with these errors.

Why am I getting calls from residents for an address assignment?

Banks, utilities, insurance companies, etc. are requiring physical addresses. A valid address assigned by an AO becomes the official address for a residence.

What happens if a person moves to a new address?

The telephone company (or service provider) will need to be notified of the new address. They will update their system database which will cause an automatic update to the ALI database. However, VoIP service providers (VSP) often leave this step up to the customer to either call their VSP or go online to update their information on a website.

Street signs keep disappearing in my town, what can I do?

Theft of street signs is against the law and is a problem in many communities. Since signage is vital to responders in locating emergencies, please report incidents of vandalism to the local police by dialing 9-1-1.

29-A MRS §2107 Tampering with signs

https://legislature.maine.gov/legis/statutes/29-A/title29-Asec2107.html

Why does my address not show in my GPS unit, in my navigation device, or on sites like Google Maps?

The ESCB has no control over that but does provide updated streets and address data on The Maine GeoLibrary’s website where commercial companies can get updates. Google Maps has a content partner site that allows local authorities to submit changes. The website is https://contentpartners.maps.google.com/portal/. More information can also be found at https://www.gps.gov/my-address-wrong.

If my address is not in my GPS, will it show up if I dial 9-1-1?

You can verify if your address is in the 9-1-1 system by going to the following site and searching for your address. https://experience.arcgis.com/experience/faa6338e0598415c81868eead6d456d1.

Does 9-1-1 use what3words?

Yes. The 9-1-1 system can plot what3words if provided by the caller. More information about what3words can be found at https://what3words.com/about.

Wireless/Cellular Questions

When I call 9-1-1 from my cell phone will the dispatchers know where I am?

Cell phones will not display your address like traditional landlines but a general location is provided. Try to have your address ready, or use landmarks, mile markers, and road signs to describe where you are.

VoIP Questions

How is a VoIP 9-1-1 call routed to the correct 9-1-1 center?

When you sign up for VoIP service, you are asked to register your address which is used to route to the correct PSAP. If you move the VoIP phone to new location, make sure to update the address.

What if my 9-1-1 call is misrouted to the wrong 9-1-1 center?

If your VoIP call is misrouted to the wrong PSAP or 9-1-1 center, you should tell the call-taker the address, city, county, and state where your emergency is. The call taker will then transfer you to the right PSAP.

Does 9-1-1 know where I am when calling from my VoIP phone?

In most cases, yes. The first thing a call-taker asks is for you to verify your location, calling number, and name, especially if this information is not automatically displayed at the PSAP. Your call is then routed to wherever it needs to go based on the information that you provide.

Can I call 9-1-1 from my VoIP phone when I’m traveling?

Some VoIP providers offer the ability to travel with your phone (called nomadic VoIPs). If so, your VoIP Service Provider (VSP) may provide an online solution where you may update your registered address.

Do service outages affect my ability to call 9-1-1?

They might. Just as a cordless phone may not work without power, your VoIP phone probably won’t work without power either. As a result, during an electrical outage, you may be unable to make any calls, including those to 9-1-1. Similarly, if your cable or broadband service is interrupted, it may keep you from being able to make outbound calls.

If I use an OnStar or similar service in my car, will my location be displayed?

OnStar and similar services relay calls to the right PSAP based on the location of the vehicle, similar to a wireless call; these are called telematics and are becoming more prevalent.

Signage Questions

How to get residents to post house number signs?

This is a very common concern in areas where mailboxes appear infrequently along rural roads. Towns have encouraged residents to post their own house numbers in prominent places alongside the road, or they may have an Addressing Ordinance in place which describes the desired signage for residents. A few towns have also enacted an enforcement ordinance which requires residents to erect appropriate signs and may charge a nominal fine for non-compliance. Other towns have purchased signs and given them to homeowners to install.

What are the recommendations for house number signage?

The following bullet points are general guidelines for house-number signs:

- If the building is within 50 (fifty) feet of the edge of the road right-of-way, the assigned number shall be prominently displayed and visible from the road.

- When the building is greater than 50 (fifty) feet from the edge of the road right-of-way, the assigned number shall be prominently displayed on a post, fence, wall, mailbox, or other structure at the property line adjacent to the walkway or driveway access to the building.

- The address numbers should be a minimum of four (4) inches in height and have a contrasting background. Numbers shall be situated such that they are visible from the road year-round.

- Remove any signage that can be confused with the correct address number.

9-1-1 Data Confidentiality Questions

Where does the ESCB get its Authority?

Maine Revised Statute (MRS) 25 MRS §2926 ESCB establishes the Bureau to implement and manage the state’s 9-1-1 system which includes the requirement to set the “standards and procedures for developing and maintaining the system databases and for ensuring the confidentiality of those databases pursuant to Section 2929.”  Section 2926(2E) also includes the requirement to set “Procedures for developing and maintaining address and routing database.”

Is the 9-1-1 database confidential?

Yes.  25 MRS §2929 defines confidential information to include, the names, addresses and telephone numbers of persons listed in

9-1-1 databases.”

Who owns the databases?

The Emergency Services Communication Bureau owns the database system.  “The system databases, wherever located or stored, are the property of the bureau and their confidentiality is governed by section 2929.”

Can 9-1-1 data be used for commercial purposes?

No. 9-1-1 confidential information may not be utilized for commercial purposes.

Can the Bureau provide a complete listing of street names and address ranges for a town?

Yes, the Bureau may provide town officials with a complete list of street names and address ranges for their town.

Can the Post Office assign addresses?

No. Only a designated AO may assign physical addresses to structures.

Should the AO work with the Post Office when creating new addresses?

Yes. To best meet the needs of the customer and the goal of having a single physical address serve as both mailing & emergency services, an AO should meet with a postal service representative before finalizing a new street name and address range for a community or structure. This will alleviate future mail-delivery issues and/or responder problems. Conflicting street names often occur in areas where the postal service delivers mail to towns who share a zip code, as one example.

When does an AO work with a phone company?

The ESCB requires phone companies to validate addresses assigned to phone services in the event of emergencies so that when someone dials 9-1-1, the correct police, fire, or ambulance service may find the caller quickly and efficiently. Sometimes an address given by a customer cannot be validated against the 9-1-1 database, so a phone company representative will contact an AO for help. For example, if an address was omitted from the 9-1-1 database, the AO would need to work quickly to resolve the issue and correct the database.

Does the Bureau or the AO manage or maintain phone numbers?

Neither. Only the phone company manages and maintains telephone numbers.

For more information, please visit our website at https://www.maine.gov/maine911/ and click on the “Community Addressing” tab.

The State Statutes, regulations and manuals cited in this memorandum are available on our website.

# Appendix B

SAMPLE ADDRESS ASSIGNMENT NOTIFICATION LETTER

TOWN LETTERHEAD

Notice of New Address

Date:

Property owner

Mailing Address

Re: Assessor Map/Lot _____________

Dear Owners Name:

Insert Town Name adopted the State of Maine 911 addressing standards as a part of our municipal ordinance in YYYY.  In order to answer 911 calls properly and to provide emergency services as fast as possible; we are required to maintain accurate road and addressing information.

You have either submitted a building permit for a new habitable structure, or you need a new address due to the creation of a new road, or some issue has arisen which requires clarification of your address.  The address shown below is your correct address effective MM/DD/YYYY  Please retain this letter as a permanent record of your address.

OLD ADDRESS:

NEW ADDRESS:

For Fire and Rescue purposes, please place and display the number(s) assigned to your property(s) prominently on or near your front door, visible from the street.  The height of each number should be at least 4 inches and made of reflective material.  If your house or building is located more than 50 feet (or not visible) from the road, please place the numbers at the beginning of your driveway.  These simple requirements will assist emergency responders in locating you, as well as assisting utilities, delivery services and others who may need to find you.

Thank you for your assistance and cooperation.  Should you require any additional information or assistance please call me at 207-NNN-NNNN.

Sincerely,

911 Addressing Officer

Copies to:

Post Office

Emergency Services Communication Bureau/GIS

Town Dept.

Police Dept.

Fire Dept.or First Responders

Ambulance or Transport Service

# Appendix C

Using Road Measurement and Property Number Tables to Assign New Addresses

Communities may use 25’, 50’, or 100’ numbering intervals to assign property numbers. Please refer to the attached numbering tables and follow the steps below as a guide to assigning correct addresses for new structures:

- Measure the distance from the beginning of the road to an imaginary line running perpendicularly from the front entrance of the new structure to the road. Use an accurate measuring device such as a measuring wheel. If such a measuring device is unavailable, determine the distance as accurately as possible with a measuring tape or by pacing off the distance.

- If the front door of a structure cannot be seen from the road, measure to a point where the middle of the driveway meets the road. In either case, note which side of the road the structure is on.

- Identify the correct measuring interval for assigning property numbers in the town (25, 50, or 100 feet). This interval may be documented in a town ordinance or ask the engineering department. Identify the odd and even sides of the road (typically radiating out from the center of town).

- Using the correct interval table, find the footage range between which your measurement falls.

- Select the correct odd or even property numbers from the table, depending on which side of the road the structure is located.

| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 0 | 25 | 1 | 2 | 1,200 | 1,225 | 97 | 98 | 2,400 | 2,425 | 193 | 194 |
| 25 | 50 | 3 | 4 | 1,225 | 1,250 | 99 | 100 | 2,425 | 2,450 | 195 | 196 |
| 50 | 75 | 5 | 6 | 1,250 | 1,275 | 101 | 102 | 2,450 | 2,475 | 197 | 198 |
| 75 | 100 | 7 | 8 | 1,275 | 1,300 | 103 | 104 | 2,475 | 2,500 | 199 | 200 |
| 100 | 125 | 9 | 10 | 1,300 | 1,325 | 105 | 106 | 2,500 | 2,525 | 201 | 202 |
| 125 | 150 | 11 | 12 | 1,325 | 1,350 | 107 | 108 | 2,525 | 2,550 | 203 | 204 |
| 150 | 175 | 13 | 14 | 1,350 | 1,375 | 109 | 110 | 2,550 | 2,575 | 205 | 206 |
| 175 | 200 | 15 | 16 | 1,375 | 1,400 | 111 | 112 | 2,575 | 2,600 | 207 | 208 |
| 200 | 225 | 17 | 18 | 1,400 | 1,425 | 113 | 114 | 2,600 | 2,625 | 209 | 210 |
| 225 | 250 | 19 | 20 | 1,425 | 1,450 | 115 | 116 | 2,625 | 2,650 | 211 | 212 |
| 250 | 275 | 21 | 22 | 1,450 | 1,475 | 117 | 118 | 2,650 | 2,675 | 213 | 214 |
| 275 | 300 | 23 | 24 | 1,475 | 1,500 | 119 | 120 | 2,675 | 2,700 | 215 | 216 |
| 300 | 325 | 25 | 26 | 1,500 | 1,525 | 121 | 122 | 2,700 | 2,725 | 217 | 218 |
| 325 | 350 | 27 | 28 | 1,525 | 1,550 | 123 | 124 | 2,725 | 2,750 | 219 | 220 |
| 350 | 375 | 29 | 30 | 1,550 | 1,575 | 125 | 126 | 2,750 | 2,775 | 221 | 222 |
| 375 | 400 | 31 | 32 | 1,575 | 1,600 | 127 | 128 | 2,775 | 2,800 | 223 | 224 |
| 400 | 425 | 33 | 34 | 1,600 | 1,625 | 129 | 130 | 2,800 | 2,825 | 225 | 226 |
| 425 | 450 | 35 | 36 | 1,625 | 1,650 | 131 | 132 | 2,825 | 2,850 | 227 | 228 |
| 450 | 475 | 37 | 38 | 1,650 | 1,675 | 133 | 134 | 2,850 | 2,875 | 229 | 230 |
| 475 | 500 | 39 | 40 | 1,675 | 1,700 | 135 | 136 | 2,875 | 2,900 | 231 | 232 |
| 500 | 525 | 41 | 42 | 1,700 | 1,725 | 137 | 138 | 2,900 | 2,925 | 233 | 234 |
| 525 | 550 | 43 | 44 | 1,725 | 1,750 | 139 | 140 | 2,925 | 2,950 | 235 | 236 |
| 550 | 575 | 45 | 46 | 1,750 | 1,775 | 141 | 142 | 2,950 | 2,975 | 237 | 238 |
| 575 | 600 | 47 | 48 | 1,775 | 1,800 | 143 | 144 | 2,975 | 3,000 | 239 | 240 |
| 600 | 625 | 49 | 50 | 1,800 | 1,825 | 145 | 146 | 3,000 | 3,025 | 241 | 242 |
| 625 | 650 | 51 | 52 | 1,825 | 1,850 | 147 | 148 | 3,025 | 3,050 | 243 | 244 |
| 650 | 675 | 53 | 54 | 1,850 | 1,875 | 149 | 150 | 3,050 | 3,075 | 245 | 246 |
| 675 | 700 | 55 | 56 | 1,875 | 1,900 | 151 | 152 | 3,075 | 3,100 | 247 | 248 |
| 700 | 725 | 57 | 58 | 1,900 | 1,925 | 153 | 154 | 3,100 | 3,125 | 249 | 250 |
| 725 | 750 | 59 | 60 | 1,925 | 1,950 | 155 | 156 | 3,125 | 3,150 | 251 | 252 |
| 750 | 775 | 61 | 62 | 1,950 | 1,975 | 157 | 158 | 3,150 | 3,175 | 253 | 254 |
| 775 | 800 | 63 | 64 | 1,975 | 2,000 | 159 | 160 | 3,175 | 3,200 | 255 | 256 |
| 800 | 825 | 65 | 66 | 2,000 | 2,025 | 161 | 162 | 3,200 | 3,225 | 257 | 258 |
| 825 | 850 | 67 | 68 | 2,025 | 2,050 | 163 | 164 | 3,225 | 3,250 | 259 | 260 |
| 850 | 875 | 69 | 70 | 2,050 | 2,075 | 165 | 166 | 3,250 | 3,275 | 261 | 262 |
| 875 | 900 | 71 | 72 | 2,075 | 2,100 | 167 | 168 | 3,275 | 3,300 | 263 | 264 |
| 900 | 925 | 73 | 74 | 2,100 | 2,125 | 169 | 170 | 3,300 | 3,325 | 265 | 266 |
| 925 | 950 | 75 | 76 | 2,125 | 2,150 | 171 | 172 | 3,325 | 3,350 | 267 | 268 |
| 950 | 975 | 77 | 78 | 2,150 | 2,175 | 173 | 174 | 3,350 | 3,375 | 269 | 270 |
| 975 | 1,000 | 79 | 80 | 2,175 | 2,200 | 175 | 176 | 3,375 | 3,400 | 271 | 272 |
| 1,000 | 1,025 | 81 | 82 | 2,200 | 2,225 | 177 | 178 | 3,400 | 3,425 | 273 | 274 |
| 1,025 | 1,050 | 83 | 84 | 2,225 | 2,250 | 179 | 180 | 3,425 | 3,450 | 275 | 276 |
| 1,050 | 1,075 | 85 | 86 | 2,250 | 2,275 | 181 | 182 | 3,450 | 3,475 | 277 | 278 |
| 1,075 | 1,100 | 87 | 88 | 2,275 | 2,300 | 183 | 184 | 3,475 | 3,500 | 279 | 280 |
| 1,100 | 1,125 | 89 | 90 | 2,300 | 2,325 | 185 | 186 | 3,500 | 3,525 | 281 | 282 |
| 1,125 | 1,150 | 91 | 92 | 2,325 | 2,350 | 187 | 188 | 3,525 | 3,550 | 283 | 284 |
| 1,150 | 1,175 | 93 | 94 | 2,350 | 2,375 | 189 | 190 | 3,550 | 3,575 | 285 | 286 |
| 1,175 | 1,200 | 95 | 96 | 2,375 | 2,400 | 191 | 192 | 3,575 | 3,600 | 287 | 288 |
| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 3,600 | 3,625 | 289 | 290 | 4,800 | 4,825 | 385 | 386 | 6,000 | 6,025 | 481 | 482 |
| 3,625 | 3,650 | 291 | 292 | 4,825 | 4,850 | 387 | 388 | 6,025 | 6,050 | 483 | 484 |
| 3,650 | 3,675 | 293 | 294 | 4,850 | 4,875 | 389 | 390 | 6,050 | 6,075 | 485 | 486 |
| 3,675 | 3,700 | 295 | 296 | 4,875 | 4,900 | 391 | 392 | 6,075 | 6,100 | 487 | 488 |
| 3,700 | 3,725 | 297 | 298 | 4,900 | 4,925 | 393 | 394 | 6,100 | 6,125 | 489 | 490 |
| 3,725 | 3,750 | 299 | 300 | 4,925 | 4,950 | 395 | 396 | 6,125 | 6,150 | 491 | 492 |
| 3,750 | 3,775 | 301 | 302 | 4,950 | 4,975 | 397 | 398 | 6,150 | 6,175 | 493 | 494 |
| 3,775 | 3,800 | 303 | 304 | 4,975 | 5,000 | 399 | 400 | 6,175 | 6,200 | 495 | 496 |
| 3,800 | 3,825 | 305 | 306 | 5,000 | 5,025 | 401 | 402 | 6,200 | 6,225 | 497 | 498 |
| 3,825 | 3,850 | 307 | 308 | 5,025 | 5,050 | 403 | 404 | 6,225 | 6,250 | 499 | 500 |
| 3,850 | 3,875 | 309 | 310 | 5,050 | 5,075 | 405 | 406 | 6,250 | 6,275 | 501 | 502 |
| 3,875 | 3,900 | 311 | 312 | 5,075 | 5,100 | 407 | 408 | 6,275 | 6,300 | 503 | 504 |
| 3,900 | 3,925 | 313 | 314 | 5,100 | 5,125 | 409 | 410 | 6,300 | 6,325 | 505 | 506 |
| 3,925 | 3,950 | 315 | 316 | 5,125 | 5,150 | 411 | 412 | 6,325 | 6,350 | 507 | 508 |
| 3,950 | 3,975 | 317 | 318 | 5,150 | 5,175 | 413 | 414 | 6,350 | 6,375 | 509 | 510 |
| 3,975 | 4,000 | 319 | 320 | 5,175 | 5,200 | 415 | 416 | 6,375 | 6,400 | 511 | 512 |
| 4,000 | 4,025 | 321 | 322 | 5,200 | 5,225 | 417 | 418 | 6,400 | 6,425 | 513 | 514 |
| 4,025 | 4,050 | 323 | 324 | 5,225 | 5,250 | 419 | 420 | 6,425 | 6,450 | 515 | 516 |
| 4,050 | 4,075 | 325 | 326 | 5,250 | 5,275 | 421 | 422 | 6,450 | 6,475 | 517 | 518 |
| 4,075 | 4,100 | 327 | 328 | 5,275 | 5,300 | 423 | 424 | 6,475 | 6,500 | 519 | 520 |
| 4,100 | 4,125 | 329 | 330 | 5,300 | 5,325 | 425 | 426 | 6,500 | 6,525 | 521 | 522 |
| 4,125 | 4,150 | 331 | 332 | 5,325 | 5,350 | 427 | 428 | 6,525 | 6,550 | 523 | 524 |
| 4,150 | 4,175 | 333 | 334 | 5,350 | 5,375 | 429 | 430 | 6,550 | 6,575 | 525 | 526 |
| 4,175 | 4,200 | 335 | 336 | 5,375 | 5,400 | 431 | 432 | 6,575 | 6,600 | 527 | 528 |
| 4,200 | 4,225 | 337 | 338 | 5,400 | 5,425 | 433 | 434 | 6,600 | 6,625 | 529 | 530 |
| 4,225 | 4,250 | 339 | 340 | 5,425 | 5,450 | 435 | 436 | 6,625 | 6,650 | 531 | 532 |
| 4,250 | 4,275 | 341 | 342 | 5,450 | 5,475 | 437 | 438 | 6,650 | 6,675 | 533 | 534 |
| 4,275 | 4,300 | 343 | 344 | 5,475 | 5,500 | 439 | 440 | 6,675 | 6,700 | 535 | 536 |
| 4,300 | 4,325 | 345 | 346 | 5,500 | 5,525 | 441 | 442 | 6,700 | 6,725 | 537 | 538 |
| 4,325 | 4,350 | 347 | 348 | 5,525 | 5,550 | 443 | 444 | 6,725 | 6,750 | 539 | 540 |
| 4,350 | 4,375 | 349 | 350 | 5,550 | 5,575 | 445 | 446 | 6,750 | 6,775 | 541 | 542 |
| 4,375 | 4,400 | 351 | 352 | 5,575 | 5,600 | 447 | 448 | 6,775 | 6,800 | 543 | 544 |
| 4,400 | 4,425 | 353 | 354 | 5,600 | 5,625 | 449 | 450 | 6,800 | 6,825 | 545 | 546 |
| 4,425 | 4,450 | 355 | 356 | 5,625 | 5,650 | 451 | 452 | 6,825 | 6,850 | 547 | 548 |
| 4,450 | 4,475 | 357 | 358 | 5,650 | 5,675 | 453 | 454 | 6,850 | 6,875 | 549 | 550 |
| 4,475 | 4,500 | 359 | 360 | 5,675 | 5,700 | 455 | 456 | 6,875 | 6,900 | 551 | 552 |
| 4,500 | 4,525 | 361 | 362 | 5,700 | 5,725 | 457 | 458 | 6,900 | 6,925 | 553 | 554 |
| 4,525 | 4,550 | 363 | 364 | 5,725 | 5,750 | 459 | 460 | 6,925 | 6,950 | 555 | 556 |
| 4,550 | 4,575 | 365 | 366 | 5,750 | 5,775 | 461 | 462 | 6,950 | 6,975 | 557 | 558 |
| 4,575 | 4,600 | 367 | 368 | 5,775 | 5,800 | 463 | 464 | 6,975 | 7,000 | 559 | 560 |
| 4,600 | 4,625 | 369 | 370 | 5,800 | 5,825 | 465 | 466 | 7,000 | 7,025 | 561 | 562 |
| 4,625 | 4,650 | 371 | 372 | 5,825 | 5,850 | 467 | 468 | 7,025 | 7,050 | 563 | 564 |
| 4,650 | 4,675 | 373 | 374 | 5,850 | 5,875 | 469 | 470 | 7,050 | 7,075 | 565 | 566 |
| 4,675 | 4,700 | 375 | 376 | 5,875 | 5,900 | 471 | 472 | 7,075 | 7,100 | 567 | 568 |
| 4,700 | 4,725 | 377 | 378 | 5,900 | 5,925 | 473 | 474 | 7,100 | 7,125 | 569 | 570 |
| 4,725 | 4,750 | 379 | 380 | 5,925 | 5,950 | 475 | 476 | 7,125 | 7,150 | 571 | 572 |
| 4,750 | 4,775 | 381 | 382 | 5,950 | 5,975 | 477 | 478 | 7,150 | 7,175 | 573 | 574 |
| 4,775 | 4,800 | 383 | 384 | 5,975 | 6,000 | 479 | 480 | 7,175 | 7,200 | 575 | 576 |
| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 7,200 | 7,225 | 577 | 578 | 8,400 | 8,425 | 673 | 674 | 9,600 | 9,625 | 769 | 770 |
| 7,225 | 7,250 | 579 | 580 | 8,425 | 8,450 | 675 | 676 | 9,625 | 9,650 | 771 | 772 |
| 7,250 | 7,275 | 581 | 582 | 8,450 | 8,475 | 677 | 678 | 9,650 | 9,675 | 773 | 774 |
| 7,275 | 7,300 | 583 | 584 | 8,475 | 8,500 | 679 | 680 | 9,675 | 9,700 | 775 | 776 |
| 7,300 | 7,325 | 585 | 586 | 8,500 | 8,525 | 681 | 682 | 9,700 | 9,725 | 777 | 778 |
| 7,325 | 7,350 | 587 | 588 | 8,525 | 8,550 | 683 | 684 | 9,725 | 9,750 | 779 | 780 |
| 7,350 | 7,375 | 589 | 590 | 8,550 | 8,575 | 685 | 686 | 9,750 | 9,775 | 781 | 782 |
| 7,375 | 7,400 | 591 | 592 | 8,575 | 8,600 | 687 | 688 | 9,775 | 9,800 | 783 | 784 |
| 7,400 | 7,425 | 593 | 594 | 8,600 | 8,625 | 689 | 690 | 9,800 | 9,825 | 785 | 786 |
| 7,425 | 7,450 | 595 | 596 | 8,625 | 8,650 | 691 | 692 | 9,825 | 9,850 | 787 | 788 |
| 7,450 | 7,475 | 597 | 598 | 8,650 | 8,675 | 693 | 694 | 9,850 | 9,875 | 789 | 790 |
| 7,475 | 7,500 | 599 | 600 | 8,675 | 8,700 | 695 | 696 | 9,875 | 9,900 | 791 | 792 |
| 7,500 | 7,525 | 601 | 602 | 8,700 | 8,725 | 697 | 698 | 9,900 | 9,925 | 793 | 794 |
| 7,525 | 7,550 | 603 | 604 | 8,725 | 8,750 | 699 | 700 | 9,925 | 9,950 | 795 | 796 |
| 7,550 | 7,575 | 605 | 606 | 8,750 | 8,775 | 701 | 702 | 9,950 | 9,975 | 797 | 798 |
| 7,575 | 7,600 | 607 | 608 | 8,775 | 8,800 | 703 | 704 | 9,975 | 10,000 | 799 | 800 |
| 7,600 | 7,625 | 609 | 610 | 8,800 | 8,825 | 705 | 706 | 10,000 | 10,025 | 801 | 802 |
| 7,625 | 7,650 | 611 | 612 | 8,825 | 8,850 | 707 | 708 | 10,025 | 10,050 | 803 | 804 |
| 7,650 | 7,675 | 613 | 614 | 8,850 | 8,875 | 709 | 710 | 10,050 | 10,075 | 805 | 806 |
| 7,675 | 7,700 | 615 | 616 | 8,875 | 8,900 | 711 | 712 | 10,075 | 10,100 | 807 | 808 |
| 7,700 | 7,725 | 617 | 618 | 8,900 | 8,925 | 713 | 714 | 10,100 | 10,125 | 809 | 810 |
| 7,725 | 7,750 | 619 | 620 | 8,925 | 8,950 | 715 | 716 | 10,125 | 10,150 | 811 | 812 |
| 7,750 | 7,775 | 621 | 622 | 8,950 | 8,975 | 717 | 718 | 10,150 | 10,175 | 813 | 814 |
| 7,775 | 7,800 | 623 | 624 | 8,975 | 9,000 | 719 | 720 | 10,175 | 10,200 | 815 | 816 |
| 7,800 | 7,825 | 625 | 626 | 9,000 | 9,025 | 721 | 722 | 10,200 | 10,225 | 817 | 818 |
| 7,825 | 7,850 | 627 | 628 | 9,025 | 9,050 | 723 | 724 | 10,225 | 10,250 | 819 | 820 |
| 7,850 | 7,875 | 629 | 630 | 9,050 | 9,075 | 725 | 726 | 10,250 | 10,275 | 821 | 822 |
| 7,875 | 7,900 | 631 | 632 | 9,075 | 9,100 | 727 | 728 | 10,275 | 10,300 | 823 | 824 |
| 7,900 | 7,925 | 633 | 634 | 9,100 | 9,125 | 729 | 730 | 10,300 | 10,325 | 825 | 826 |
| 7,925 | 7,950 | 635 | 636 | 9,125 | 9,150 | 731 | 732 | 10,325 | 10,350 | 827 | 828 |
| 7,950 | 7,975 | 637 | 638 | 9,150 | 9,175 | 733 | 734 | 10,350 | 10,375 | 829 | 830 |
| 7,975 | 8,000 | 639 | 640 | 9,175 | 9,200 | 735 | 736 | 10,375 | 10,400 | 831 | 832 |
| 8,000 | 8,025 | 641 | 642 | 9,200 | 9,225 | 737 | 738 | 10,400 | 10,425 | 833 | 834 |
| 8,025 | 8,050 | 643 | 644 | 9,225 | 9,250 | 739 | 740 | 10,425 | 10,450 | 835 | 836 |
| 8,050 | 8,075 | 645 | 646 | 9,250 | 9,275 | 741 | 742 | 10,450 | 10,475 | 837 | 838 |
| 8,075 | 8,100 | 647 | 648 | 9,275 | 9,300 | 743 | 744 | 10,475 | 10,500 | 839 | 840 |
| 8,100 | 8,125 | 649 | 650 | 9,300 | 9,325 | 745 | 746 | 10,500 | 10,525 | 841 | 842 |
| 8,125 | 8,150 | 651 | 652 | 9,325 | 9,350 | 747 | 748 | 10,525 | 10,550 | 843 | 844 |
| 8,150 | 8,175 | 653 | 654 | 9,350 | 9,375 | 749 | 750 | 10,550 | 10,575 | 845 | 846 |
| 8,175 | 8,200 | 655 | 656 | 9,375 | 9,400 | 751 | 752 | 10,575 | 10,600 | 847 | 848 |
| 8,200 | 8,225 | 657 | 658 | 9,400 | 9,425 | 753 | 754 | 10,600 | 10,625 | 849 | 850 |
| 8,225 | 8,250 | 659 | 660 | 9,425 | 9,450 | 755 | 756 | 10,625 | 10,650 | 851 | 852 |
| 8,250 | 8,275 | 661 | 662 | 9,450 | 9,475 | 757 | 758 | 10,650 | 10,675 | 853 | 854 |
| 8,275 | 8,300 | 663 | 664 | 9,475 | 9,500 | 759 | 760 | 10,675 | 10,700 | 855 | 856 |
| 8,300 | 8,325 | 665 | 666 | 9,500 | 9,525 | 761 | 762 | 10,700 | 10,725 | 857 | 858 |
| 8,325 | 8,350 | 667 | 668 | 9,525 | 9,550 | 763 | 764 | 10,725 | 10,750 | 859 | 860 |
| 8,350 | 8,375 | 669 | 670 | 9,550 | 9,575 | 765 | 766 | 10,750 | 10,775 | 861 | 862 |
| 8,375 | 8,400 | 671 | 672 | 9,575 | 9,600 | 767 | 768 | 10,775 | 10,800 | 863 | 864 |
| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 10,800 | 10,825 | 865 | 866 | 12,000 | 12,025 | 961 | 962 | 13,200 | 13,225 | 1057 | 1058 |
| 10,825 | 10,850 | 867 | 868 | 12,025 | 12,050 | 963 | 964 | 13,225 | 13,250 | 1059 | 1060 |
| 10,850 | 10,875 | 869 | 870 | 12,050 | 12,075 | 965 | 966 | 13,250 | 13,275 | 1061 | 1062 |
| 10,875 | 10,900 | 871 | 872 | 12,075 | 12,100 | 967 | 968 | 13,275 | 13,300 | 1063 | 1064 |
| 10,900 | 10,925 | 873 | 874 | 12,100 | 12,125 | 969 | 970 | 13,300 | 13,325 | 1065 | 1066 |
| 10,925 | 10,950 | 875 | 876 | 12,125 | 12,150 | 971 | 972 | 13,325 | 13,350 | 1067 | 1068 |
| 10,950 | 10,975 | 877 | 878 | 12,150 | 12,175 | 973 | 974 | 13,350 | 13,375 | 1069 | 1070 |
| 10,975 | 11,000 | 879 | 880 | 12,175 | 12,200 | 975 | 976 | 13,375 | 13,400 | 1071 | 1072 |
| 11,000 | 11,025 | 881 | 882 | 12,200 | 12,225 | 977 | 978 | 13,400 | 13,425 | 1073 | 1074 |
| 11,025 | 11,050 | 883 | 884 | 12,225 | 12,250 | 979 | 980 | 13,425 | 13,450 | 1075 | 1076 |
| 11,050 | 11,075 | 885 | 886 | 12,250 | 12,275 | 981 | 982 | 13,450 | 13,475 | 1077 | 1078 |
| 11,075 | 11,100 | 887 | 888 | 12,275 | 12,300 | 983 | 984 | 13,475 | 13,500 | 1079 | 1080 |
| 11,100 | 11,125 | 889 | 890 | 12,300 | 12,325 | 985 | 986 | 13,500 | 13,525 | 1081 | 1082 |
| 11,125 | 11,150 | 891 | 892 | 12,325 | 12,350 | 987 | 988 | 13,525 | 13,550 | 1083 | 1084 |
| 11,150 | 11,175 | 893 | 894 | 12,350 | 12,375 | 989 | 990 | 13,550 | 13,575 | 1085 | 1086 |
| 11,175 | 11,200 | 895 | 896 | 12,375 | 12,400 | 991 | 992 | 13,575 | 13,600 | 1087 | 1088 |
| 11,200 | 11,225 | 897 | 898 | 12,400 | 12,425 | 993 | 994 | 13,600 | 13,625 | 1089 | 1090 |
| 11,225 | 11,250 | 899 | 900 | 12,425 | 12,450 | 995 | 996 | 13,625 | 13,650 | 1091 | 1092 |
| 11,250 | 11,275 | 901 | 902 | 12,450 | 12,475 | 997 | 998 | 13,650 | 13,675 | 1093 | 1094 |
| 11,275 | 11,300 | 903 | 904 | 12,475 | 12,500 | 999 | 1000 | 13,675 | 13,700 | 1095 | 1096 |
| 11,300 | 11,325 | 905 | 906 | 12,500 | 12,525 | 1001 | 1002 | 13,700 | 13,725 | 1097 | 1098 |
| 11,325 | 11,350 | 907 | 908 | 12,525 | 12,550 | 1003 | 1004 | 13,725 | 13,750 | 1099 | 1100 |
| 11,350 | 11,375 | 909 | 910 | 12,550 | 12,575 | 1005 | 1006 | 13,750 | 13,775 | 1101 | 1102 |
| 11,375 | 11,400 | 911 | 912 | 12,575 | 12,600 | 1007 | 1008 | 13,775 | 13,800 | 1103 | 1104 |
| 11,400 | 11,425 | 913 | 914 | 12,600 | 12,625 | 1009 | 1010 | 13,800 | 13,825 | 1105 | 1106 |
| 11,425 | 11,450 | 915 | 916 | 12,625 | 12,650 | 1011 | 1012 | 13,825 | 13,850 | 1107 | 1108 |
| 11,450 | 11,475 | 917 | 918 | 12,650 | 12,675 | 1013 | 1014 | 13,850 | 13,875 | 1109 | 1110 |
| 11,475 | 11,500 | 919 | 920 | 12,675 | 12,700 | 1015 | 1016 | 13,875 | 13,900 | 1111 | 1112 |
| 11,500 | 11,525 | 921 | 922 | 12,700 | 12,725 | 1017 | 1018 | 13,900 | 13,925 | 1113 | 1114 |
| 11,525 | 11,550 | 923 | 924 | 12,725 | 12,750 | 1019 | 1020 | 13,925 | 13,950 | 1115 | 1116 |
| 11,550 | 11,575 | 925 | 926 | 12,750 | 12,775 | 1021 | 1022 | 13,950 | 13,975 | 1117 | 1118 |
| 11,575 | 11,600 | 927 | 928 | 12,775 | 12,800 | 1023 | 1024 | 13,975 | 14,000 | 1119 | 1120 |
| 11,600 | 11,625 | 929 | 930 | 12,800 | 12,825 | 1025 | 1026 | 14,000 | 14,025 | 1121 | 1122 |
| 11,625 | 11,650 | 931 | 932 | 12,825 | 12,850 | 1027 | 1028 | 14,025 | 14,050 | 1123 | 1124 |
| 11,650 | 11,675 | 933 | 934 | 12,850 | 12,875 | 1029 | 1030 | 14,050 | 14,075 | 1125 | 1126 |
| 11,675 | 11,700 | 935 | 936 | 12,875 | 12,900 | 1031 | 1032 | 14,075 | 14,100 | 1127 | 1128 |
| 11,700 | 11,725 | 937 | 938 | 12,900 | 12,925 | 1033 | 1034 | 14,100 | 14,125 | 1129 | 1130 |
| 11,725 | 11,750 | 939 | 940 | 12,925 | 12,950 | 1035 | 1036 | 14,125 | 14,150 | 1131 | 1132 |
| 11,750 | 11,775 | 941 | 942 | 12,950 | 12,975 | 1037 | 1038 | 14,150 | 14,175 | 1133 | 1134 |
| 11,775 | 11,800 | 943 | 944 | 12,975 | 13,000 | 1039 | 1040 | 14,175 | 14,200 | 1135 | 1136 |
| 11,800 | 11,825 | 945 | 946 | 13,000 | 13,025 | 1041 | 1042 | 14,200 | 14,225 | 1137 | 1138 |
| 11,825 | 11,850 | 947 | 948 | 13,025 | 13,050 | 1043 | 1044 | 14,225 | 14,250 | 1139 | 1140 |
| 11,850 | 11,875 | 949 | 950 | 13,050 | 13,075 | 1045 | 1046 | 14,250 | 14,275 | 1141 | 1142 |
| 11,875 | 11,900 | 951 | 952 | 13,075 | 13,100 | 1047 | 1048 | 14,275 | 14,300 | 1143 | 1144 |
| 11,900 | 11,925 | 953 | 954 | 13,100 | 13,125 | 1049 | 1050 | 14,300 | 14,325 | 1145 | 1146 |
| 11,925 | 11,950 | 955 | 956 | 13,125 | 13,150 | 1051 | 1052 | 14,325 | 14,350 | 1147 | 1148 |
| 11,950 | 11,975 | 957 | 958 | 13,150 | 13,175 | 1053 | 1054 | 14,350 | 14,375 | 1149 | 1150 |
| 11,975 | 12,000 | 959 | 960 | 13,175 | 13,200 | 1055 | 1056 | 14,375 | 14,400 | 1151 | 1152 |

| Footage Down Road |  | Number | Number | Footage Down Road |  | Number |  | Number | Footage Down Road |  | Number | Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| From | To | Odd | Even | From | To | Odd |  | Even | From | To | Odd | Even |
| 0 | 50 | 1 | 2 | 2,400 | 2,450 | 97 |  | 98 | 4,800 | 4,850 | 193 | 194 |
| 50 | 100 | 3 | 4 | 2,450 | 2,500 | 99 |  | 100 | 4,850 | 4,900 | 195 | 196 |
| 100 | 150 | 5 | 6 | 2,500 | 2,550 | 101 |  | 102 | 4,900 | 4,950 | 197 | 198 |
| 150 | 200 | 7 | 8 | 2,550 | 2,600 | 103 |  | 104 | 4,950 | 5,000 | 199 | 200 |
| 200 | 250 | 9 | 10 | 2,600 | 2,650 | 105 |  | 106 | 5,000 | 5,050 | 201 | 202 |
| 250 | 300 | 11 | 12 | 2,650 | 2,700 | 107 |  | 108 | 5,050 | 5,100 | 203 | 204 |
| 300 | 350 | 13 | 14 | 2,700 | 2,750 | 109 |  | 110 | 5,100 | 5,150 | 205 | 206 |
| 350 | 400 | 15 | 16 | 2,750 | 2,800 | 111 |  | 112 | 5,150 | 5,200 | 207 | 208 |
| 400 | 450 | 17 | 18 | 2,800 | 2,850 | 113 |  | 114 | 5,200 | 5,250 | 209 | 210 |
| 450 | 500 | 19 | 20 | 2,850 | 2,900 | 115 |  | 116 | 5,250 | 5,300 | 211 | 212 |
| 500 | 550 | 21 | 22 | 2,900 | 2,950 | 117 |  | 118 | 5,300 | 5,350 | 213 | 214 |
| 550 | 600 | 23 | 24 | 2,950 | 3,000 | 119 |  | 120 | 5,350 | 5,400 | 215 | 216 |
| 600 | 650 | 25 | 26 | 3,000 | 3,050 | 121 |  | 122 | 5,400 | 5,450 | 217 | 218 |
| 650 | 700 | 27 | 28 | 3,050 | 3,100 | 123 |  | 124 | 5,450 | 5,500 | 219 | 220 |
| 700 | 750 | 29 | 30 | 3,100 | 3,150 | 125 |  | 126 | 5,500 | 5,550 | 221 | 222 |
| 750 | 800 | 31 | 32 | 3,150 | 3,200 | 127 |  | 128 | 5,550 | 5,600 | 223 | 224 |
| 800 | 850 | 33 | 34 | 3,200 | 3,250 | 129 |  | 130 | 5,600 | 5,650 | 225 | 226 |
| 850 | 900 | 35 | 36 | 3,250 | 3,300 | 131 |  | 132 | 5,650 | 5,700 | 227 | 228 |
| 900 | 950 | 37 | 38 | 3,300 | 3,350 | 133 |  | 134 | 5,700 | 5,750 | 229 | 230 |
| 950 | 1,000 | 39 | 40 | 3,350 | 3,400 | 135 |  | 136 | 5,750 | 5,800 | 231 | 232 |
| 1,000 | 1,050 | 41 | 42 | 3,400 | 3,450 | 137 |  | 138 | 5,800 | 5,850 | 233 | 234 |
| 1,050 | 1,100 | 43 | 44 | 3,450 | 3,500 | 139 |  | 140 | 5,850 | 5,900 | 235 | 236 |
| 1,100 | 1,150 | 45 | 46 | 3,500 | 3,550 | 141 |  | 142 | 5,900 | 5,950 | 237 | 238 |
| 1,150 | 1,200 | 47 | 48 | 3,550 | 3,600 | 143 |  | 144 | 5,950 | 6,000 | 239 | 240 |
| 1,200 | 1,250 | 49 | 50 | 3,600 | 3,650 | 145 |  | 146 | 6,000 | 6,050 | 241 | 242 |
| 1,250 | 1,300 | 51 | 52 | 3,650 | 3,700 | 147 |  | 148 | 6,050 | 6,100 | 243 | 244 |
| 1,300 | 1,350 | 53 | 54 | 3,700 | 3,750 | 149 |  | 150 | 6,100 | 6,150 | 245 | 246 |
| 1,350 | 1,400 | 55 | 56 | 3,750 | 3,800 | 151 |  | 152 | 6,150 | 6,200 | 247 | 248 |
| 1,400 | 1,450 | 57 | 58 | 3,800 | 3,850 | 153 |  | 154 | 6,200 | 6,250 | 249 | 250 |
| 1,450 | 1,500 | 59 | 60 | 3,850 | 3,900 | 155 |  | 156 | 6,250 | 6,300 | 251 | 252 |
| 1,500 | 1,550 | 61 | 62 | 3,900 | 3,950 | 157 |  | 158 | 6,300 | 6,350 | 253 | 254 |
| 1,550 | 1,600 | 63 | 64 | 3,950 | 4,000 | 159 |  | 160 | 6,350 | 6,400 | 255 | 256 |
| 1,600 | 1,650 | 65 | 66 | 4,000 | 4,050 | 161 |  | 162 | 6,400 | 6,450 | 257 | 258 |
| 1,650 | 1,700 | 67 | 68 | 4,050 | 4,100 | 163 |  | 164 | 6,450 | 6,500 | 259 | 260 |
| 1,700 | 1,750 | 69 | 70 | 4,100 | 4,150 | 165 |  | 166 | 6,500 | 6,550 | 261 | 262 |
| 1,750 | 1,800 | 71 | 72 | 4,150 | 4,200 | 167 |  | 168 | 6,550 | 6,600 | 263 | 264 |
| 1,800 | 1,850 | 73 | 74 | 4,200 | 4,250 | 169 |  | 170 | 6,600 | 6,650 | 265 | 266 |
| 1,850 | 1,900 | 75 | 76 | 4,250 | 4,300 | 171 |  | 172 | 6,650 | 6,700 | 267 | 268 |
| 1,900 | 1,950 | 77 | 78 | 4,300 | 4,350 | 173 |  | 174 | 6,700 | 6,750 | 269 | 270 |
| 1,950 | 2,000 | 79 | 80 | 4,350 | 4,400 | 175 |  | 176 | 6,750 | 6,800 | 271 | 272 |
| 2,000 | 2,050 | 81 | 82 | 4,400 | 4,450 | 177 |  | 178 | 6,800 | 6,850 | 273 | 274 |
| 2,050 | 2,100 | 83 | 84 | 4,450 | 4,500 | 179 |  | 180 | 6,850 | 6,900 | 275 | 276 |
| 2,100 | 2,150 | 85 | 86 | 4,500 | 4,550 | 181 |  | 182 | 6,900 | 6,950 | 277 | 278 |
| 2,150 | 2,200 | 87 | 88 | 4,550 | 4,600 | 183 |  | 184 | 6,950 | 7,000 | 279 | 280 |
| 2,200 | 2,250 | 89 | 90 | 4,600 | 4,650 | 185 |  | 186 | 7,000 | 7,050 | 281 | 282 |
| 2,250 | 2,300 | 91 | 92 | 4,650 | 4,700 | 187 |  | 188 | 7,050 | 7,100 | 283 | 284 |
| 2,300 | 2,350 | 93 | 94 | 4,700 | 4,750 | 189 |  | 190 | 7,100 | 7,150 | 285 | 286 |
| 2,350 | 2,400 | 95 | 96 | 4,750 | 4,800 |  | 191 | 192 | 7,150 | 7,200 | 287 | 288 |
| Footage Down Road |  | Number | Number | Footage Down Road |  |  | Number | Number | Footage Down Road |  | Number | Number |
| From | To | Odd | Even | From | To |  | Odd | Even | From | To | Odd | Even |
| 7,200 | 7,250 | 289 | 290 | 9,600 | 9,650 |  | 385 | 386 | 12,000 | 12,050 | 481 | 482 |
| 7,250 | 7,300 | 291 | 292 | 9,650 | 9,700 |  | 387 | 388 | 12,050 | 12,100 | 483 | 484 |
| 7,300 | 7,350 | 293 | 294 | 9,700 | 9,750 |  | 389 | 390 | 12,100 | 12,150 | 485 | 486 |
| 7,350 | 7,400 | 295 | 296 | 9,750 | 9,800 |  | 391 | 392 | 12,150 | 12,200 | 487 | 488 |
| 7,400 | 7,450 | 297 | 298 | 9,800 | 9,850 |  | 393 | 394 | 12,200 | 12,250 | 489 | 490 |
| 7,450 | 7,500 | 299 | 300 | 9,850 | 9,900 |  | 395 | 396 | 12,250 | 12,300 | 491 | 492 |
| 7,500 | 7,550 | 301 | 302 | 9,900 | 9,950 |  | 397 | 398 | 12,300 | 12,350 | 493 | 494 |
| 7,550 | 7,600 | 303 | 304 | 9,950 | 10,000 |  | 399 | 400 | 12,350 | 12,400 | 495 | 496 |
| 7,600 | 7,650 | 305 | 306 | 10,000 | 10,050 |  | 401 | 402 | 12,400 | 12,450 | 497 | 498 |
| 7,650 | 7,700 | 307 | 308 | 10,050 | 10,100 |  | 403 | 404 | 12,450 | 12,500 | 499 | 500 |
| 7,700 | 7,750 | 309 | 310 | 10,100 | 10,150 |  | 405 | 406 | 12,500 | 12,550 | 501 | 502 |
| 7,750 | 7,800 | 311 | 312 | 10,150 | 10,200 |  | 407 | 408 | 12,550 | 12,600 | 503 | 504 |
| 7,800 | 7,850 | 313 | 314 | 10,200 | 10,250 |  | 409 | 410 | 12,600 | 12,650 | 505 | 506 |
| 7,850 | 7,900 | 315 | 316 | 10,250 | 10,300 |  | 411 | 412 | 12,650 | 12,700 | 507 | 508 |
| 7,900 | 7,950 | 317 | 318 | 10,300 | 10,350 |  | 413 | 414 | 12,700 | 12,750 | 509 | 510 |
| 7,950 | 8,000 | 319 | 320 | 10,350 | 10,400 |  | 415 | 416 | 12,750 | 12,800 | 511 | 512 |
| 8,000 | 8,050 | 321 | 322 | 10,400 | 10,450 |  | 417 | 418 | 12,800 | 12,850 | 513 | 514 |
| 8,050 | 8,100 | 323 | 324 | 10,450 | 10,500 |  | 419 | 420 | 12,850 | 12,900 | 515 | 516 |
| 8,100 | 8,150 | 325 | 326 | 10,500 | 10,550 |  | 421 | 422 | 12,900 | 12,950 | 517 | 518 |
| 8,150 | 8,200 | 327 | 328 | 10,550 | 10,600 |  | 423 | 424 | 12,950 | 13,000 | 519 | 520 |
| 8,200 | 8,250 | 329 | 330 | 10,600 | 10,650 |  | 425 | 426 | 13,000 | 13,050 | 521 | 522 |
| 8,250 | 8,300 | 331 | 332 | 10,650 | 10,700 |  | 427 | 428 | 13,050 | 13,100 | 523 | 524 |
| 8,300 | 8,350 | 333 | 334 | 10,700 | 10,750 |  | 429 | 430 | 13,100 | 13,150 | 525 | 526 |
| 8,350 | 8,400 | 335 | 336 | 10,750 | 10,800 |  | 431 | 432 | 13,150 | 13,200 | 527 | 528 |
| 8,400 | 8,450 | 337 | 338 | 10,800 | 10,850 |  | 433 | 434 | 13,200 | 13,250 | 529 | 530 |
| 8,450 | 8,500 | 339 | 340 | 10,850 | 10,900 |  | 435 | 436 | 13,250 | 13,300 | 531 | 532 |
| 8,500 | 8,550 | 341 | 342 | 10,900 | 10,950 |  | 437 | 438 | 13,300 | 13,350 | 533 | 534 |
| 8,550 | 8,600 | 343 | 344 | 10,950 | 11,000 |  | 439 | 440 | 13,350 | 13,400 | 535 | 536 |
| 8,600 | 8,650 | 345 | 346 | 11,000 | 11,050 |  | 441 | 442 | 13,400 | 13,450 | 537 | 538 |
| 8,650 | 8,700 | 347 | 348 | 11,050 | 11,100 |  | 443 | 444 | 13,450 | 13,500 | 539 | 540 |
| 8,700 | 8,750 | 349 | 350 | 11,100 | 11,150 |  | 445 | 446 | 13,500 | 13,550 | 541 | 542 |
| 8,750 | 8,800 | 351 | 352 | 11,150 | 11,200 |  | 447 | 448 | 13,550 | 13,600 | 543 | 544 |
| 8,800 | 8,850 | 353 | 354 | 11,200 | 11,250 |  | 449 | 450 | 13,600 | 13,650 | 545 | 546 |
| 8,850 | 8,900 | 355 | 356 | 11,250 | 11,300 |  | 451 | 452 | 13,650 | 13,700 | 547 | 548 |
| 8,900 | 8,950 | 357 | 358 | 11,300 | 11,350 |  | 453 | 454 | 13,700 | 13,750 | 549 | 550 |
| 8,950 | 9,000 | 359 | 360 | 11,350 | 11,400 |  | 455 | 456 | 13,750 | 13,800 | 551 | 552 |
| 9,000 | 9,050 | 361 | 362 | 11,400 | 11,450 |  | 457 | 458 | 13,800 | 13,850 | 553 | 554 |
| 9,050 | 9,100 | 363 | 364 | 11,450 | 11,500 |  | 459 | 460 | 13,850 | 13,900 | 555 | 556 |
| 9,100 | 9,150 | 365 | 366 | 11,500 | 11,550 |  | 461 | 462 | 13,900 | 13,950 | 557 | 558 |
| 9,150 | 9,200 | 367 | 368 | 11,550 | 11,600 |  | 463 | 464 | 13,950 | 14,000 | 559 | 560 |
| 9,200 | 9,250 | 369 | 370 | 11,600 | 11,650 |  | 465 | 466 | 14,000 | 14,050 | 561 | 562 |
| 9,250 | 9,300 | 371 | 372 | 11,650 | 11,700 |  | 467 | 468 | 14,050 | 14,100 | 563 | 564 |
| 9,300 | 9,350 | 373 | 374 | 11,700 | 11,750 |  | 469 | 470 | 14,100 | 14,150 | 565 | 566 |
| 9,350 | 9,400 | 375 | 376 | 11,750 | 11,800 |  | 471 | 472 | 14,150 | 14,200 | 567 | 568 |
| 9,400 | 9,450 | 377 | 378 | 11,800 | 11,850 |  | 473 | 474 | 14,200 | 14,250 | 569 | 570 |
| 9,450 | 9,500 | 379 | 380 | 11,850 | 11,900 |  | 475 | 476 | 14,250 | 14,300 | 571 | 572 |
| 9,500 | 9,550 | 381 | 382 | 11,900 | 11,950 |  | 477 | 478 | 14,300 | 14,350 | 573 | 574 |
| 9,550 | 9,600 | 383 | 384 | 11,950 | 12,000 |  | 479 | 480 | 14,350 | 14,400 | 575 | 576 |
| Footage Down Road |  | Number | Number | Footage Down Road |  |  | Number | Number | Footage Down Road |  | Number | Number |
| From | To | Odd | Even | From | To |  | Odd | Even | From | To | Odd | Even |
| 14,400 | 14,450 | 577 | 578 | 16,800 | 16,850 |  | 673 | 674 | 19,200 | 19,250 | 769 | 770 |
| 14,450 | 14,500 | 579 | 580 | 16,850 | 16,900 |  | 675 | 676 | 19,250 | 19,300 | 771 | 772 |
| 14,500 | 14,550 | 581 | 582 | 16,900 | 16,950 |  | 677 | 678 | 19,300 | 19,350 | 773 | 774 |
| 14,550 | 14,600 | 583 | 584 | 16,950 | 17,000 |  | 679 | 680 | 19,350 | 19,400 | 775 | 776 |
| 14,600 | 14,650 | 585 | 586 | 17,000 | 17,050 |  | 681 | 682 | 19,400 | 19,450 | 777 | 778 |
| 14,650 | 14,700 | 587 | 588 | 17,050 | 17,100 |  | 683 | 684 | 19,450 | 19,500 | 779 | 780 |
| 14,700 | 14,750 | 589 | 590 | 17,100 | 17,150 |  | 685 | 686 | 19,500 | 19,550 | 781 | 782 |
| 14,750 | 14,800 | 591 | 592 | 17,150 | 17,200 |  | 687 | 688 | 19,550 | 19,600 | 783 | 784 |
| 14,800 | 14,850 | 593 | 594 | 17,200 | 17,250 |  | 689 | 690 | 19,600 | 19,650 | 785 | 786 |
| 14,850 | 14,900 | 595 | 596 | 17,250 | 17,300 |  | 691 | 692 | 19,650 | 19,700 | 787 | 788 |
| 14,900 | 14,950 | 597 | 598 | 17,300 | 17,350 |  | 693 | 694 | 19,700 | 19,750 | 789 | 790 |
| 14,950 | 15,000 | 599 | 600 | 17,350 | 17,400 |  | 695 | 696 | 19,750 | 19,800 | 791 | 792 |
| 15,000 | 15,050 | 601 | 602 | 17,400 | 17,450 |  | 697 | 698 | 19,800 | 19,850 | 793 | 794 |
| 15,050 | 15,100 | 603 | 604 | 17,450 | 17,500 |  | 699 | 700 | 19,850 | 19,900 | 795 | 796 |
| 15,100 | 15,150 | 605 | 606 | 17,500 | 17,550 |  | 701 | 702 | 19,900 | 19,950 | 797 | 798 |
| 15,150 | 15,200 | 607 | 608 | 17,550 | 17,600 |  | 703 | 704 | 19,950 | 20,000 | 799 | 800 |
| 15,200 | 15,250 | 609 | 610 | 17,600 | 17,650 |  | 705 | 706 | 20,000 | 20,050 | 801 | 802 |
| 15,250 | 15,300 | 611 | 612 | 17,650 | 17,700 |  | 707 | 708 | 20,050 | 20,100 | 803 | 804 |
| 15,300 | 15,350 | 613 | 614 | 17,700 | 17,750 |  | 709 | 710 | 20,100 | 20,150 | 805 | 806 |
| 15,350 | 15,400 | 615 | 616 | 17,750 | 17,800 |  | 711 | 712 | 20,150 | 20,200 | 807 | 808 |
| 15,400 | 15,450 | 617 | 618 | 17,800 | 17,850 |  | 713 | 714 | 20,200 | 20,250 | 809 | 810 |
| 15,450 | 15,500 | 619 | 620 | 17,850 | 17,900 |  | 715 | 716 | 20,250 | 20,300 | 811 | 812 |
| 15,500 | 15,550 | 621 | 622 | 17,900 | 17,950 |  | 717 | 718 | 20,300 | 20,350 | 813 | 814 |
| 15,550 | 15,600 | 623 | 624 | 17,950 | 18,000 |  | 719 | 720 | 20,350 | 20,400 | 815 | 816 |
| 15,600 | 15,650 | 625 | 626 | 18,000 | 18,050 |  | 721 | 722 | 20,400 | 20,450 | 817 | 818 |
| 15,650 | 15,700 | 627 | 628 | 18,050 | 18,100 |  | 723 | 724 | 20,450 | 20,500 | 819 | 820 |
| 15,700 | 15,750 | 629 | 630 | 18,100 | 18,150 |  | 725 | 726 | 20,500 | 20,550 | 821 | 822 |
| 15,750 | 15,800 | 631 | 632 | 18,150 | 18,200 |  | 727 | 728 | 20,550 | 20,600 | 823 | 824 |
| 15,800 | 15,850 | 633 | 634 | 18,200 | 18,250 |  | 729 | 730 | 20,600 | 20,650 | 825 | 826 |
| 15,850 | 15,900 | 635 | 636 | 18,250 | 18,300 |  | 731 | 732 | 20,650 | 20,700 | 827 | 828 |
| 15,900 | 15,950 | 637 | 638 | 18,300 | 18,350 |  | 733 | 734 | 20,700 | 20,750 | 829 | 830 |
| 15,950 | 16,000 | 639 | 640 | 18,350 | 18,400 |  | 735 | 736 | 20,750 | 20,800 | 831 | 832 |
| 16,000 | 16,050 | 641 | 642 | 18,400 | 18,450 |  | 737 | 738 | 20,800 | 20,850 | 833 | 834 |
| 16,050 | 16,100 | 643 | 644 | 18,450 | 18,500 |  | 739 | 740 | 20,850 | 20,900 | 835 | 836 |
| 16,100 | 16,150 | 645 | 646 | 18,500 | 18,550 |  | 741 | 742 | 20,900 | 20,950 | 837 | 838 |
| 16,150 | 16,200 | 647 | 648 | 18,550 | 18,600 |  | 743 | 744 | 20,950 | 21,000 | 839 | 840 |
| 16,200 | 16,250 | 649 | 650 | 18,600 | 18,650 |  | 745 | 746 | 21,000 | 21,050 | 841 | 842 |
| 16,250 | 16,300 | 651 | 652 | 18,650 | 18,700 |  | 747 | 748 | 21,050 | 21,100 | 843 | 844 |
| 16,300 | 16,350 | 653 | 654 | 18,700 | 18,750 |  | 749 | 750 | 21,100 | 21,150 | 845 | 846 |
| 16,350 | 16,400 | 655 | 656 | 18,750 | 18,800 |  | 751 | 752 | 21,150 | 21,200 | 847 | 848 |
| 16,400 | 16,450 | 657 | 658 | 18,800 | 18,850 |  | 753 | 754 | 21,200 | 21,250 | 849 | 850 |
| 16,450 | 16,500 | 659 | 660 | 18,850 | 18,900 |  | 755 | 756 | 21,250 | 21,300 | 851 | 852 |
| 16,500 | 16,550 | 661 | 662 | 18,900 | 18,950 |  | 757 | 758 | 21,300 | 21,350 | 853 | 854 |
| 16,550 | 16,600 | 663 | 664 | 18,950 | 19,000 |  | 759 | 760 | 21,350 | 21,400 | 855 | 856 |
| 16,600 | 16,650 | 665 | 666 | 19,000 | 19,050 |  | 761 | 762 | 21,400 | 21,450 | 857 | 858 |
| 16,650 | 16,700 | 667 | 668 | 19,050 | 19,100 |  | 763 | 764 | 21,450 | 21,500 | 859 | 860 |
| 16,700 | 16,750 | 669 | 670 | 19,100 | 19,150 |  | 765 | 766 | 21,500 | 21,550 | 861 | 862 |
| 16,750 | 16,800 | 671 | 672 | 19,150 | 19,200 |  | 767 | 768 | 21,550 | 21,600 | 863 | 864 |

| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 21,600 | 21,650 | 865 | 866 | 24,000 | 24,050 | 961 | 962 | 26,400 | 26,450 | 1057 | 1058 |
| 21,650 | 21,700 | 867 | 868 | 24,050 | 24,100 | 963 | 964 | 26,450 | 26,500 | 1059 | 1060 |
| 21,700 | 21,750 | 869 | 870 | 24,100 | 24,150 | 965 | 966 | 26,500 | 26,550 | 1061 | 1062 |
| 21,750 | 21,800 | 871 | 872 | 24,150 | 24,200 | 967 | 968 | 26,550 | 26,600 | 1063 | 1064 |
| 21,800 | 21,850 | 873 | 874 | 24,200 | 24,250 | 969 | 970 | 26,600 | 26,650 | 1065 | 1066 |
| 21,850 | 21,900 | 875 | 876 | 24,250 | 24,300 | 971 | 972 | 26,650 | 26,700 | 1067 | 1068 |
| 21,900 | 21,950 | 877 | 878 | 24,300 | 24,350 | 973 | 974 | 26,700 | 26,750 | 1069 | 1070 |
| 21,950 | 22,000 | 879 | 880 | 24,350 | 24,400 | 975 | 976 | 26,750 | 26,800 | 1071 | 1072 |
| 22,000 | 22,050 | 881 | 882 | 24,400 | 24,450 | 977 | 978 | 26,800 | 26,850 | 1073 | 1074 |
| 22,050 | 22,100 | 883 | 884 | 24,450 | 24,500 | 979 | 980 | 26,850 | 26,900 | 1075 | 1076 |
| 22,100 | 22,150 | 885 | 886 | 24,500 | 24,550 | 981 | 982 | 26,900 | 26,950 | 1077 | 1078 |
| 22,150 | 22,200 | 887 | 888 | 24,550 | 24,600 | 983 | 984 | 26,950 | 27,000 | 1079 | 1080 |
| 22,200 | 22,250 | 889 | 890 | 24,600 | 24,650 | 985 | 986 | 27,000 | 27,050 | 1081 | 1082 |
| 22,250 | 22,300 | 891 | 892 | 24,650 | 24,700 | 987 | 988 | 27,050 | 27,100 | 1083 | 1084 |
| 22,300 | 22,350 | 893 | 894 | 24,700 | 24,750 | 989 | 990 | 27,100 | 27,150 | 1085 | 1086 |
| 22,350 | 22,400 | 895 | 896 | 24,750 | 24,800 | 991 | 992 | 27,150 | 27,200 | 1087 | 1088 |
| 22,400 | 22,450 | 897 | 898 | 24,800 | 24,850 | 993 | 994 | 27,200 | 27,250 | 1089 | 1090 |
| 22,450 | 22,500 | 899 | 900 | 24,850 | 24,900 | 995 | 996 | 27,250 | 27,300 | 1091 | 1092 |
| 22,500 | 22,550 | 901 | 902 | 24,900 | 24,950 | 997 | 998 | 27,300 | 27,350 | 1093 | 1094 |
| 22,550 | 22,600 | 903 | 904 | 24,950 | 25,000 | 999 | 1000 | 27,350 | 27,400 | 1095 | 1096 |
| 22,600 | 22,650 | 905 | 906 | 25,000 | 25,050 | 1001 | 1002 | 27,400 | 27,450 | 1097 | 1098 |
| 22,650 | 22,700 | 907 | 908 | 25,050 | 25,100 | 1003 | 1004 | 27,450 | 27,500 | 1099 | 1100 |
| 22,700 | 22,750 | 909 | 910 | 25,100 | 25,150 | 1005 | 1006 | 27,500 | 27,550 | 1101 | 1102 |
| 22,750 | 22,800 | 911 | 912 | 25,150 | 25,200 | 1007 | 1008 | 27,550 | 27,600 | 1103 | 1104 |
| 22,800 | 22,850 | 913 | 914 | 25,200 | 25,250 | 1009 | 1010 | 27,600 | 27,650 | 1105 | 1106 |
| 22,850 | 22,900 | 915 | 916 | 25,250 | 25,300 | 1011 | 1012 | 27,650 | 27,700 | 1107 | 1108 |
| 22,900 | 22,950 | 917 | 918 | 25,300 | 25,350 | 1013 | 1014 | 27,700 | 27,750 | 1109 | 1110 |
| 22,950 | 23,000 | 919 | 920 | 25,350 | 25,400 | 1015 | 1016 | 27,750 | 27,800 | 1111 | 1112 |
| 23,000 | 23,050 | 921 | 922 | 25,400 | 25,450 | 1017 | 1018 | 27,800 | 27,850 | 1113 | 1114 |
| 23,050 | 23,100 | 923 | 924 | 25,450 | 25,500 | 1019 | 1020 | 27,850 | 27,900 | 1115 | 1116 |
| 23,100 | 23,150 | 925 | 926 | 25,500 | 25,550 | 1021 | 1022 | 27,900 | 27,950 | 1117 | 1118 |
| 23,150 | 23,200 | 927 | 928 | 25,550 | 25,600 | 1023 | 1024 | 27,950 | 28,000 | 1119 | 1120 |
| 23,200 | 23,250 | 929 | 930 | 25,600 | 25,650 | 1025 | 1026 | 28,000 | 28,050 | 1121 | 1122 |
| 23,250 | 23,300 | 931 | 932 | 25,650 | 25,700 | 1027 | 1028 | 28,050 | 28,100 | 1123 | 1124 |
| 23,300 | 23,350 | 933 | 934 | 25,700 | 25,750 | 1029 | 1030 | 28,100 | 28,150 | 1125 | 1126 |
| 23,350 | 23,400 | 935 | 936 | 25,750 | 25,800 | 1031 | 1032 | 28,150 | 28,200 | 1127 | 1128 |
| 23,400 | 23,450 | 937 | 938 | 25,800 | 25,850 | 1033 | 1034 | 28,200 | 28,250 | 1129 | 1130 |
| 23,450 | 23,500 | 939 | 940 | 25,850 | 25,900 | 1035 | 1036 | 28,250 | 28,300 | 1131 | 1132 |
| 23,500 | 23,550 | 941 | 942 | 25,900 | 25,950 | 1037 | 1038 | 28,300 | 28,350 | 1133 | 1134 |
| 23,550 | 23,600 | 943 | 944 | 25,950 | 26,000 | 1039 | 1040 | 28,350 | 28,400 | 1135 | 1136 |
| 23,600 | 23,650 | 945 | 946 | 26,000 | 26,050 | 1041 | 1042 | 28,400 | 28,450 | 1137 | 1138 |
| 23,650 | 23,700 | 947 | 948 | 26,050 | 26,100 | 1043 | 1044 | 28,450 | 28,500 | 1139 | 1140 |
| 23,700 | 23,750 | 949 | 950 | 26,100 | 26,150 | 1045 | 1046 | 28,500 | 28,550 | 1141 | 1142 |
| 23,750 | 23,800 | 951 | 952 | 26,150 | 26,200 | 1047 | 1048 | 28,550 | 28,600 | 1143 | 1144 |
| 23,800 | 23,850 | 953 | 954 | 26,200 | 26,250 | 1049 | 1050 | 28,600 | 28,650 | 1145 | 1146 |
| 23,850 | 23,900 | 955 | 956 | 26,250 | 26,300 | 1051 | 1052 | 28,650 | 28,700 | 1147 | 1148 |
| 23,900 | 23,950 | 957 | 958 | 26,300 | 26,350 | 1053 | 1054 | 28,700 | 28,750 | 1149 | 1150 |
| 23,950 | 24,000 | 959 | 960 | 26,350 | 26,400 | 1055 | 1056 | 28,750 | 28,800 | 1151 | 1152 |

| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 0 | 100 | 1 | 2 | 4,800 | 4,900 | 97 | 98 | 9,600 | 9,700 | 193 | 194 |
| 100 | 200 | 3 | 4 | 4,900 | 5,000 | 99 | 100 | 9,700 | 9,800 | 195 | 196 |
| 200 | 300 | 5 | 6 | 5,000 | 5,100 | 101 | 102 | 9,800 | 9,900 | 197 | 198 |
| 300 | 400 | 7 | 8 | 5,100 | 5,200 | 103 | 104 | 9,900 | 10,000 | 199 | 200 |
| 400 | 500 | 9 | 10 | 5,200 | 5,300 | 105 | 106 | 10,000 | 10,100 | 201 | 202 |
| 500 | 600 | 11 | 12 | 5,300 | 5,400 | 107 | 108 | 10,100 | 10,200 | 203 | 204 |
| 600 | 700 | 13 | 14 | 5,400 | 5,500 | 109 | 110 | 10,200 | 10,300 | 205 | 206 |
| 700 | 800 | 15 | 16 | 5,500 | 5,600 | 111 | 112 | 10,300 | 10,400 | 207 | 208 |
| 800 | 900 | 17 | 18 | 5,600 | 5,700 | 113 | 114 | 10,400 | 10,500 | 209 | 210 |
| 900 | 1,000 | 19 | 20 | 5,700 | 5,800 | 115 | 116 | 10,500 | 10,600 | 211 | 212 |
| 1,000 | 1,100 | 21 | 22 | 5,800 | 5,900 | 117 | 118 | 10,600 | 10,700 | 213 | 214 |
| 1,100 | 1,200 | 23 | 24 | 5,900 | 6,000 | 119 | 120 | 10,700 | 10,800 | 215 | 216 |
| 1,200 | 1,300 | 25 | 26 | 6,000 | 6,100 | 121 | 122 | 10,800 | 10,900 | 217 | 218 |
| 1,300 | 1,400 | 27 | 28 | 6,100 | 6,200 | 123 | 124 | 10,900 | 11,000 | 219 | 220 |
| 1,400 | 1,500 | 29 | 30 | 6,200 | 6,300 | 125 | 126 | 11,000 | 11,100 | 221 | 222 |
| 1,500 | 1,600 | 31 | 32 | 6,300 | 6,400 | 127 | 128 | 11,100 | 11,200 | 223 | 224 |
| 1,600 | 1,700 | 33 | 34 | 6,400 | 6,500 | 129 | 130 | 11,200 | 11,300 | 225 | 226 |
| 1,700 | 1,800 | 35 | 36 | 6,500 | 6,600 | 131 | 132 | 11,300 | 11,400 | 227 | 228 |
| 1,800 | 1,900 | 37 | 38 | 6,600 | 6,700 | 133 | 134 | 11,400 | 11,500 | 229 | 230 |
| 1,900 | 2,000 | 39 | 40 | 6,700 | 6,800 | 135 | 136 | 11,500 | 11,600 | 231 | 232 |
| 2,000 | 2,100 | 41 | 42 | 6,800 | 6,900 | 137 | 138 | 11,600 | 11,700 | 233 | 234 |
| 2,100 | 2,200 | 43 | 44 | 6,900 | 7,000 | 139 | 140 | 11,700 | 11,800 | 235 | 236 |
| 2,200 | 2,300 | 45 | 46 | 7,000 | 7,100 | 141 | 142 | 11,800 | 11,900 | 237 | 238 |
| 2,300 | 2,400 | 47 | 48 | 7,100 | 7,200 | 143 | 144 | 11,900 | 12,000 | 239 | 240 |
| 2,400 | 2,500 | 49 | 50 | 7,200 | 7,300 | 145 | 146 | 12,000 | 12,100 | 241 | 242 |
| 2,500 | 2,600 | 51 | 52 | 7,300 | 7,400 | 147 | 148 | 12,100 | 12,200 | 243 | 244 |
| 2,600 | 2,700 | 53 | 54 | 7,400 | 7,500 | 149 | 150 | 12,200 | 12,300 | 245 | 246 |
| 2,700 | 2,800 | 55 | 56 | 7,500 | 7,600 | 151 | 152 | 12,300 | 12,400 | 247 | 248 |
| 2,800 | 2,900 | 57 | 58 | 7,600 | 7,700 | 153 | 154 | 12,400 | 12,500 | 249 | 250 |
| 2,900 | 3,000 | 59 | 60 | 7,700 | 7,800 | 155 | 156 | 12,500 | 12,600 | 251 | 252 |
| 3,000 | 3,100 | 61 | 62 | 7,800 | 7,900 | 157 | 158 | 12,600 | 12,700 | 253 | 254 |
| 3,100 | 3,200 | 63 | 64 | 7,900 | 8,000 | 159 | 160 | 12,700 | 12,800 | 255 | 256 |
| 3,200 | 3,300 | 65 | 66 | 8,000 | 8,100 | 161 | 162 | 12,800 | 12,900 | 257 | 258 |
| 3,300 | 3,400 | 67 | 68 | 8,100 | 8,200 | 163 | 164 | 12,900 | 13,000 | 259 | 260 |
| 3,400 | 3,500 | 69 | 70 | 8,200 | 8,300 | 165 | 166 | 13,000 | 13,100 | 261 | 262 |
| 3,500 | 3,600 | 71 | 72 | 8,300 | 8,400 | 167 | 168 | 13,100 | 13,200 | 263 | 264 |
| 3,600 | 3,700 | 73 | 74 | 8,400 | 8,500 | 169 | 170 | 13,200 | 13,300 | 265 | 266 |
| 3,700 | 3,800 | 75 | 76 | 8,500 | 8,600 | 171 | 172 | 13,300 | 13,400 | 267 | 268 |
| 3,800 | 3,900 | 77 | 78 | 8,600 | 8,700 | 173 | 174 | 13,400 | 13,500 | 269 | 270 |
| 3,900 | 4,000 | 79 | 80 | 8,700 | 8,800 | 175 | 176 | 13,500 | 13,600 | 271 | 272 |
| 4,000 | 4,100 | 81 | 82 | 8,800 | 8,900 | 177 | 178 | 13,600 | 13,700 | 273 | 274 |
| 4,100 | 4,200 | 83 | 84 | 8,900 | 9,000 | 179 | 180 | 13,700 | 13,800 | 275 | 276 |
| 4,200 | 4,300 | 85 | 86 | 9,000 | 9,100 | 181 | 182 | 13,800 | 13,900 | 277 | 278 |
| 4,300 | 4,400 | 87 | 88 | 9,100 | 9,200 | 183 | 184 | 13,900 | 14,000 | 279 | 280 |
| 4,400 | 4,500 | 89 | 90 | 9,200 | 9,300 | 185 | 186 | 14,000 | 14,100 | 281 | 282 |
| 4,500 | 4,600 | 91 | 92 | 9,300 | 9,400 | 187 | 188 | 14,100 | 14,200 | 283 | 284 |
| 4,600 | 4,700 | 93 | 94 | 9,400 | 9,500 | 189 | 190 | 14,200 | 14,300 | 285 | 286 |
| 4,700 | 4,800 | 95 | 96 | 9,500 | 9,600 | 191 | 192 | 14,300 | 14,400 | 287 | 288 |
| Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number | Footage Down Road |  | Number | Number |
| From | To | Odd | Even | From | To | Odd | Even | From | To | Odd | Even |
| 14,400 | 14,500 | 289 | 290 | 19,200 | 19,300 | 385 | 386 | 24,000 | 24,100 | 481 | 482 |
| 14,500 | 14,600 | 291 | 292 | 19,300 | 19,400 | 387 | 388 | 24,100 | 24,200 | 483 | 484 |
| 14,600 | 14,700 | 293 | 294 | 19,400 | 19,500 | 389 | 390 | 24,200 | 24,300 | 485 | 486 |
| 14,700 | 14,800 | 295 | 296 | 19,500 | 19,600 | 391 | 392 | 24,300 | 24,400 | 487 | 488 |
| 14,800 | 14,900 | 297 | 298 | 19,600 | 19,700 | 393 | 394 | 24,400 | 24,500 | 489 | 490 |
| 14,900 | 15,000 | 299 | 300 | 19,700 | 19,800 | 395 | 396 | 24,500 | 24,600 | 491 | 492 |
| 15,000 | 15,100 | 301 | 302 | 19,800 | 19,900 | 397 | 398 | 24,600 | 24,700 | 493 | 494 |
| 15,100 | 15,200 | 303 | 304 | 19,900 | 20,000 | 399 | 400 | 24,700 | 24,800 | 495 | 496 |
| 15,200 | 15,300 | 305 | 306 | 20,000 | 20,100 | 401 | 402 | 24,800 | 24,900 | 497 | 498 |
| 15,300 | 15,400 | 307 | 308 | 20,100 | 20,200 | 403 | 404 | 24,900 | 25,000 | 499 | 500 |
| 15,400 | 15,500 | 309 | 310 | 20,200 | 20,300 | 405 | 406 | 25,000 | 25,100 | 501 | 502 |
| 15,500 | 15,600 | 311 | 312 | 20,300 | 20,400 | 407 | 408 | 25,100 | 25,200 | 503 | 504 |
| 15,600 | 15,700 | 313 | 314 | 20,400 | 20,500 | 409 | 410 | 25,200 | 25,300 | 505 | 506 |
| 15,700 | 15,800 | 315 | 316 | 20,500 | 20,600 | 411 | 412 | 25,300 | 25,400 | 507 | 508 |
| 15,800 | 15,900 | 317 | 318 | 20,600 | 20,700 | 413 | 414 | 25,400 | 25,500 | 509 | 510 |
| 15,900 | 16,000 | 319 | 320 | 20,700 | 20,800 | 415 | 416 | 25,500 | 25,600 | 511 | 512 |
| 16,000 | 16,100 | 321 | 322 | 20,800 | 20,900 | 417 | 418 | 25,600 | 25,700 | 513 | 514 |
| 16,100 | 16,200 | 323 | 324 | 20,900 | 21,000 | 419 | 420 | 25,700 | 25,800 | 515 | 516 |
| 16,200 | 16,300 | 325 | 326 | 21,000 | 21,100 | 421 | 422 | 25,800 | 25,900 | 517 | 518 |
| 16,300 | 16,400 | 327 | 328 | 21,100 | 21,200 | 423 | 424 | 25,900 | 26,000 | 519 | 520 |
| 16,400 | 16,500 | 329 | 330 | 21,200 | 21,300 | 425 | 426 | 26,000 | 26,100 | 521 | 522 |
| 16,500 | 16,600 | 331 | 332 | 21,300 | 21,400 | 427 | 428 | 26,100 | 26,200 | 523 | 524 |
| 16,600 | 16,700 | 333 | 334 | 21,400 | 21,500 | 429 | 430 | 26,200 | 26,300 | 525 | 526 |
| 16,700 | 16,800 | 335 | 336 | 21,500 | 21,600 | 431 | 432 | 26,300 | 26,400 | 527 | 528 |
| 16,800 | 16,900 | 337 | 338 | 21,600 | 21,700 | 433 | 434 | 26,400 | 26,500 | 529 | 530 |
| 16,900 | 17,000 | 339 | 340 | 21,700 | 21,800 | 435 | 436 | 26,500 | 26,600 | 531 | 532 |
| 17,000 | 17,100 | 341 | 342 | 21,800 | 21,900 | 437 | 438 | 26,600 | 26,700 | 533 | 534 |
| 17,100 | 17,200 | 343 | 344 | 21,900 | 22,000 | 439 | 440 | 26,700 | 26,800 | 535 | 536 |
| 17,200 | 17,300 | 345 | 346 | 22,000 | 22,100 | 441 | 442 | 26,800 | 26,900 | 537 | 538 |
| 17,300 | 17,400 | 347 | 348 | 22,100 | 22,200 | 443 | 444 | 26,900 | 27,000 | 539 | 540 |
| 17,400 | 17,500 | 349 | 350 | 22,200 | 22,300 | 445 | 446 | 27,000 | 27,100 | 541 | 542 |
| 17,500 | 17,600 | 351 | 352 | 22,300 | 22,400 | 447 | 448 | 27,100 | 27,200 | 543 | 544 |
| 17,600 | 17,700 | 353 | 354 | 22,400 | 22,500 | 449 | 450 | 27,200 | 27,300 | 545 | 546 |
| 17,700 | 17,800 | 355 | 356 | 22,500 | 22,600 | 451 | 452 | 27,300 | 27,400 | 547 | 548 |
| 17,800 | 17,900 | 357 | 358 | 22,600 | 22,700 | 453 | 454 | 27,400 | 27,500 | 549 | 550 |
| 17,900 | 18,000 | 359 | 360 | 22,700 | 22,800 | 455 | 456 | 27,500 | 27,600 | 551 | 552 |
| 18,000 | 18,100 | 361 | 362 | 22,800 | 22,900 | 457 | 458 | 27,600 | 27,700 | 553 | 554 |
| 18,100 | 18,200 | 363 | 364 | 22,900 | 23,000 | 459 | 460 | 27,700 | 27,800 | 555 | 556 |
| 18,200 | 18,300 | 365 | 366 | 23,000 | 23,100 | 461 | 462 | 27,800 | 27,900 | 557 | 558 |
| 18,300 | 18,400 | 367 | 368 | 23,100 | 23,200 | 463 | 464 | 27,900 | 28,000 | 559 | 560 |
| 18,400 | 18,500 | 369 | 370 | 23,200 | 23,300 | 465 | 466 | 28,000 | 28,100 | 561 | 562 |
| 18,500 | 18,600 | 371 | 372 | 23,300 | 23,400 | 467 | 468 | 28,100 | 28,200 | 563 | 564 |
| 18,600 | 18,700 | 373 | 374 | 23,400 | 23,500 | 469 | 470 | 28,200 | 28,300 | 565 | 566 |
| 18,700 | 18,800 | 375 | 376 | 23,500 | 23,600 | 471 | 472 | 28,300 | 28,400 | 567 | 568 |
| 18,800 | 18,900 | 377 | 378 | 23,600 | 23,700 | 473 | 474 | 28,400 | 28,500 | 569 | 570 |
| 18,900 | 19,000 | 379 | 380 | 23,700 | 23,800 | 475 | 476 | 28,500 | 28,600 | 571 | 572 |
| 19,000 | 19,100 | 381 | 382 | 23,800 | 23,900 | 477 | 478 | 28,600 | 28,700 | 573 | 574 |
| 19,100 | 19,200 | 383 | 384 | 23,900 | 24,000 | 479 | 480 | 28,700 | 28,800 | 575 | 576 |