Status Report

21673	Hampden

U.S. Survey Feet

--------------------------------------------------------------------------------------------------------------------------------------

CONTROL SUMMARY

COORD SYSTEM GROUP:  Maine State Plane      REALIZATION: NAD83(2011)

ZONE: Maine 2000 Central       DATUM TRANSFORMATION:  Moledensky

GEOID MODEL:  GEOID12A          VERTICAL DATUM:  NAVD88

COMBINED FACTOR (CF):  0.99998657

Date Delivered/By:   23 Jun 2017   BMD

Control was set using FastStatic on June 23, 2017.  21728-12 was used as a check in the network and checked very well.  New points 13, 14 and 15 were set.  13 and 14 are nails in pavement.  15 is a cabin spike in shoulder of the road.

The following CONTROL POINTS were held for TRAVERSE ADJUSTMENT calculations:

The following CORS STATIONS were used:

MECC, MEWA, MEDX

--------------------------------------------------------------------------------------------------------------------------------------

TRAVERSE SUMMARY

Date of Traverse: 04Dec17

Bench numbers used:

Traverse files used:

F21673A1.dc

Traverse numbers used:

FLY 600 – 603

TRAVERSE COMMENTS:

Traverse adjusted using InRoads

--------------------------------------------------------------------------------------------------------------------------------------

RTO SUMMARY

RTO FILES USED   &   POINT NUMBERS USED:

R21673A1.dc – R21673A3.dc      A1000 - A2808

RTO COMMENTS:

Last Point number used: A2808

SURVEY EDIT SUMMARY

- Mobile Lidar data delivered by E.A. was processed, creating or updating the following files:  3DContours.dgn, 3DLidar_09-11-17.dgn, 3DLidar_Text.dgn, 3DTriangles.dgn, Ground.dtm, Lidar Only.dtm.  Topo folder data has been created.

09-11-17	N.B. Dutil

- Traverse Orig.fwd adjusted using InRoads. (F21673A1.dc).

All InRoads editing has been done on R21673A1.dc - R21673A3.dc

This has been translated to MicroStation as: 3DTopo_xxxxx.dgn, 3DText.dgn, 3DPoints.dgn,

3DContours.dgn, 3DTriangles.dgn, Contours.dgn, Points.dgn, Text.dgn and Topo.dgn.

Microstation cleanup on Topo.dgn has been done.

Microstation cleanup on Text.dgn has not been done.

All InRoads and MicroStation files have been added to the appropriate folders on the Y:\ drive.

D Green   04Dec17