06-096	DEPARTMENT OF ENVIRONMENTAL PROTECTION

Chapter 137:	EMISSION STATEMENTS

SUMMARY: This regulation establishes requirements for the reporting of pollutant emissions from stationary sources of air pollution.

1.	Applicability

A.	This regulation applies statewide.

- This regulation applies to all stationary sources which are licensed to emit into the ambient air, pursuant to 06-096 CMR 115, “Major and Minor Source Air Emission License Regulations” or 06-096 CMR 140, “Part 70 Air Emission License Regulations,” any of the following air pollutants at or above the minimum required reporting level:

Criteria Air Pollutants				Minimum Reporting Threshold

(1)	Carbon monoxide (CO)	75 tpy

(2)	Sulfur dioxide (SO2)	40 tpy

(3)	Volatile organic compounds (VOC)	25 tpy

(4)	Nitrogen oxides (NOx) (in NO2 equivalents)	25 tpy

(5)	Fine Particulate Matter (PM10) 	15 tpy

(6)	Fine Particulate Matter (PM2.5)	15 tpy

(7)	Lead (Pb)	0.1 tpy

(8)	Ammonia (NH3)	50 tpy

C.	This regulation applies to those stationary sources required to report their emissions pursuant to Section 1(B) of this Chapter and that emit any of the following greenhouse gases:

- Carbon dioxide (CO2)

- Methane (CH4)

- Nitrous oxide (N2O)

- Hydrofluorocarbons (HFCs)

- Perfluorocarbons (PFCs)

- Sulfur hexafluoride (SF6)

D.	If a stationary source is licensed to emit any one pollutant as specified in Section 1(B) at or above the minimum required reporting level, the data for all pollutants listed in Section 1(B) and 1(C) must be collected and reported.

E.	This regulation applies to these additional stationary sources of greenhouse gases:

(1)	all electrical power transmission and distribution plants that emit any amount of sulfur hexafluoride (SF6); and

(2)	all greenhouse gas manufacturing facilities that emit any amount of greenhouse gases.

F.	This regulation applies to those stationary sources required to report their emissions pursuant to Section 1(B) of this Chapter and which:

(1) emit to the air a hazardous air pollutant at or above the applicable reporting threshold listed in Appendix A; or

(2) where actual emissions are not known, either use, process, or manufacture any hazardous air pollutant at or above the reporting threshold listed in Appendix A. For the purposes of this subsection, “manufacture” includes any hazardous air pollutant coincidentally manufactured, such as byproducts of a process or compounds resulting from combustion.

2.	Definitions. The following terms are defined for use in this Chapter:

- Criteria air pollutant (CAP). “Criteria air pollutant (CAP)” means one of the following pollutants: carbon monoxide; sulfur dioxide; volatile organic compounds; nitrogen oxides; fine particulate matter; lead and ammonia.

- Greenhouse gas (GHG). “Greenhouse gas (GHG)” means one of the following gases: carbon dioxide (CO2), methane (CH4), nitrous oxide (N2O), sulfur hexafluoride (SF6), or any gases classified as hydrofluorocarbons (HFCs) or perfluorocarbons (PFCs).

- Greenhouse gas manufacturing facility. “Greenhouse gas manufacturing facility” means a facility that intentionally produces a greenhouse gas for use as a raw material in the manufacturing process or for sale. This term does not include compounds that are coincidentally manufactured, such as byproducts of a process or combustion.

- Hazardous air pollutant (HAP). “Hazardous air pollutant (HAP)” means those pollutants listed in Appendix A of this Chapter.

- Hydrofluorocarbons (HFCs). “Hydrofluorocarbons (HFCs)” means those pollutants listed in Appendix B of this Chapter.

F.	Perfluorocarbons (PFCs). “Perfluorocarbons (PFCs)” means those pollutants listed in Appendix B of this Chapter.

G.	Process Unit. “Process Unit” means any combination of equipment or operation and material or fuel which emits pollutants or greenhouse gases.

H.	Tons per year (tpy). “Tons per year (tpy)” means tons per year of actual emissions.

I.	Transmission and Distribution Plant. “Transmission and Distribution Plant” means all real estate, fixtures and personal property owned, controlled, operated or managed in connection with or to facilitate the transmission, distribution or delivery of electricity for light, heat or power for public use and includes all conduits, ducts and other devices, material, apparatus and property for containing, holding or carrying conductors used, or to be used, for the transmission or distribution of light, heat or power for public use.

3.	Requirements

A.	Criteria Air Pollutant Emission Statements. The owner or operator of any facility meeting the applicability requirements in Section 1(B) must file an emission statement with the Department on an annual basis for those criteria air pollutants listed in Section 1(B) of this Chapter.

- For those pollutants listed in Section 1(B), the emission statement shall be limited to emissions from only equipment and processes required to be included in and described in their air emission license.

- De minimus emissions need not be reported. For criteria pollutants, de minimus emissions means those emissions, when aggregated on a facility basis, are less than one percent (1%) of the minimum reporting threshold in Section 1(B).

- Greenhouse Gas Emission Statements. The owner or operator of any facility meeting the applicability requirements in Sections 1(D) or (E) must file an emission statement with the Department on an annual basis for those greenhouse gases described in Section 1(C) of this Chapter.

(1)	For those greenhouse gases described in Section 1(C), the emission statement shall be limited to emissions from only equipment and processes required to be included in and described in their air emission license, with the exception of fugitive emissions of HFCs, PFCs and SF6 greater than the de minimis reporting level.

(2)	De minimus emissions need not be reported. For greenhouse gases, de minimus emissions means those emissions, when aggregated on a facility basis, that are less than one ton of carbon dioxide equivalents (CO2e), when calculated using the global warming potentials listed in Appendix B.

- Hazardous Air Pollutant Emission Statements. The owner or operator of any facility meeting the applicability requirements in Sections 1(B) and 1(F) must file an emission statement with the Department every three years, starting with data of emissions from calendar year 2008 for those hazardous air pollutants subject to reporting in Section 1(F).

- For hazardous air pollutants subject to reporting in Section 1(F), the emission statement shall be limited to emissions from only equipment and processes required to be included in and described in their air emission license.

- The emissions statement must include all hazardous air pollutant, which when taken in aggregate at the facility, exceed the reporting threshold in Appendix A.

- Notwithstanding subsection 3(C)(2) and the reporting thresholds in Appendix A, emissions of the following hazardous air pollutants must be reported for all fuel burning or combustion equipment required to be included and described in the air emission license:

    - acetaldehyde;

    - acrolein;

    - arsenic and arsenic compounds;

    - benzene;

    - cadmium and cadmium compounds;

    - chromium and chromium compounds - For the purposes of reporting chromium and chromium compounds, the owner or operators of the facility may report the total emissions for all chromium compounds or the emissions of hexavalent and trivalent chromium individually, however, not both the compounds and the metal in its oxidized forms;

    - cobalt and cobalt compounds;

    - dioxins – For the purposes of reporting dioxin and dioxin-like compounds, if the total toxicity equivalents of dioxin and dioxin-like compounds is over the 0.0002 pound threshold, the owner or operator of the facility must report the quantities of the individual dioxin isomers and dioxin-like compounds;

    - formaldehyde;

    - manganese and manganese compounds;

    - mercury and mercury compounds;

    - nickel and nickel compounds; and

    - polycyclic organic matter – For the purposes of reporting polycyclic organic matter, a owner or operator of a facility may report the sum total of polycyclic organic matter emissions or the quantities of the individual components of the polycyclic organic matter group, however, not both.

NOTE: This list of thirteen hazardous air pollutants represents a subset of the Air Toxics Priority list, developed for the Department by the Maine Air Toxics Initiative.

(4) De minimus emissions need not be reported. For hazardous air pollutants, de minimus refers to those hazardous air pollutants that do not have to be counted towards the reporting threshold determination if they are present in a mixture below certain concentrations. Those concentrations are one percent (1%) for non-carcinogens and one-tenth of one percent (0.1%) for hazardous air pollutants which meet the OSHA carcinogen standard. There is no de minimus concentration for persistent bioaccumulative toxic (PBT) chemicals.

D.	Emission statements required by subsections 3(A) through 3(C) must be filed with the Department no later than July 1 of the year following the inventory year. Beginning with inventory year 2009, emission statements required by subsections 3(A) through 3(C) must be filed with the Department no later than May 15 of the year following the inventory year.

NOTE: Beginning with the 2009 annual emissions inventory, the inventory submission deadline will move from July 1 to May 15. The 2009 annual emissions inventory must be reported no later than May 15, 2010.

4.	Emission Statement. The owner or operator of a facility that is subject to reporting shall file, at a minimum, the following information in a format prescribed by the Department.

A.	Certification – A certification that the information contained in the statement is accurate and complete to the best knowledge of the facility’s responsible official or his/her designee. The certification shall include the full name, title, signature, date of signature, and telephone number of the responsible official or designee.

B.	Inventory year – Calendar year for which emissions estimates are calculated.

C.	Facility Identification Information

- State FIPS code – The Federal Information Placement System (FIPS) is the system of unique numeric codes the government developed to identify States, counties and parishes for the entire United States, Puerto Rico and Guam.

- County FIPS code – The Federal Information Placement System (FIPS) is the system of unique numeric codes the government developed to identify States, counties and parishes for the entire United States, Puerto Rico and Guam.

- Facility ID code – The unique code for a facility that is generated by the Department.

- Site Name – The name of the facility as it appears on its air emission license or if unlicensed, the name of the facility as identified by the Bureau of Taxation.

- Physical Address – The street address for the facility where emissions occur. This must be the E911 address, when available.

- Mailing Address of the facility.

- Contact name, telephone number and e-mail address for both a knowledgeable person who can answer questions regarding the emission statement and the responsible official for the organization transmitting the data.

(8)	SIC/NAICS – The Standard Industrial Classification Code or North American

- Industry Classification System code which classifies business by products or services.

(9)	Latitude and Longitude or Universal Transverse Mercator (UTM) coordinates of the facility and Method Accuracy Description Codes used to define the accuracy of the geographic data.

D.	Emissions Information

- Pollutant Code – The unique code for each reported pollutant assigned by the U.S. Environmental Protection Agency or the Department.

- Control Status – An indication whether reported emissions are controlled or uncontrolled.

- Control Device Description(s) – The name and type of control device(s) , their individual capture and control efficiencies (percent), their operational status during the inventory period, and the total capture and control efficiency of all devices.

- Total Annual Activity/Throughput data – The total annual amount of a measurable factor or parameter that relates directly or indirectly to the emissions of an air pollution source. Depending on the type of source category, activity information may refer to the amount of fuel combusted, raw material processed, product manufactured, or material handled or processed.

- Annual Emissions – The actual emissions for a facility or process unit – measured or calculated that represent a calendar year. For fugitive (non-stack) emissions of greenhouse gases, emissions may be summed and reported, in aggregate, for the entire facility.

- Emission Factor – The ratio relating emissions of a specific pollutant to an activity or material throughput level. The source or basis for the emission factor must also be provided.

- Emission Calculation Method – A code for the method by which the emissions are calculated.

- Emission Operating Type Code – The code associated with the operating type of emissions being reported (routine, upset, or startup/shutdown).

- Estimation calculations with documentation supporting all input variables.

E.	Operating Information

(1)	Start time (hour) – The start time (if available) that was used to calculate the emissions estimates.

(2)	Actual Hours –The actual number of hours the equipment or process unit is active or operating during the reporting period.

3)	Average Hours Per Day – The hours per day that the emitting equipment or process unit operates, averaged over the inventory period.

(4)	Average Days Per Week – The days per week that the emitting equipment or process unit operates, averaged over the inventory period.

(5)	Average Weeks Per Year – The weeks per year that the emitting equipment or process unit operates, averaged over the inventory period.

- Design Capacity – The measure of the size of a point source, based on the reported maximum continuous throughput or output capacity of the unit. For a boiler, design capacity is based on the reported maximum continuous steam flow, usually in units of million BTU per hour.

- Maximum Nameplate Capacity – The measure of the size of a generator which is put on the unit’s nameplate by the manufacturer. The data element is reported in megawatts or kilowatts.

- Unit Type Code – A code that identifies the type of emissions unit (e.g., 100=”Boiler”, etc.)

- Unit Operating Status Code – A code that identifies the operating status of the emissions unit (e.g., PS=”permanently shut down”).

- Unit Operating Status Date – The year in which the unit status is applicable. For units being reported in inventory for the first time, it will be the calendar year minus one.

F.	Additional Activity/Throughput Data

- Activity/throughput (monthly) – The throughput on a monthly basis.

- Activity/Throughput (daily) – If applicable, an estimate of the daily average throughput, including the beginning and ending dates and times that define the emissions period used to estimate the daily activity rate/throughput.

- Spring Throughput (percent) – The part of throughput or activity for the three Spring months (March, April, and May) of the inventory period. It can be a percentage of the annual activity (e.g., out of 600 units produced per year, 180 are produced in spring = 30%) or a percentage of throughput (e.g., out of 1,000 gallons of fuel burned per year, 300 gallons are burned in the Spring quarter = 30%).

- Summer Throughput (percent) - The part of throughput or activity for the three Summer months (June, July and August) of the inventory period.

- Fall Throughput (percent) - The part of throughput or activity for the three Fall months (September, October, and November) of the inventory period.

- Winter Throughput (percent) – The part of throughput or activity for the three Winter months (December, January, and February) of the inventory period. all from the same calendar year.

G.	Release Point Data

- Emission Release Point Type – A code for the physical configuration of the release point (e.g., stack, fugitive, etc.)

- Latitude and Longitude or Universal Transverse Mercator (UTM) coordinates of stack or release point and Method Accuracy Description Codes used to define the accuracy of the geographic data.

- Stack or Release Point Height – The height above the surrounding terrain.

- Stack or Release Point Diameter – The inner physical diameter.

- Exit Gas Temperature – The numeric value of an exit gas stream’s temperature.

- Exit Gas Flow Rate – The numeric value of an exit gas’s flow rate.

- Exit Gas Velocity – The numeric value of an exit gas stream’s velocity.

H.	Fuel and Process Parameters

- SCC (Source classification code) – The process level code that describes the equipment or operation which is emitting pollutants.

- Fuel parameters

- Fuel type

- Fuel consumption (thousands of gallons of fuel oil, tons of coal or wood, etc.) - monthly and annually

- Heat Content (annual average of fuel) – The amount of thermal heat energy in the fuel.

- Sulfur Content (annual average of fuel) – The sulfur content of the fuel, expressed in percent.

- Ash Content (annual average of fuel) – The inert residual portion of the fuel, expressed in percent.

5.	Emissions Estimation Approaches

Air emissions reported to the Department pursuant to this Chapter shall be quantified/estimated in the manner which most accurately reflects actual emissions in the order, as follows below. The Department retains the right to review reports, question the emission procedure used, and require use of an estimation procedure that the Department determines is more accurate.

- For sources with specification CEMs/PEMs monitoring systems that are required by statute, regulation, or license condition, emission data generated by these systems shall serve as the basis for emissions reported pursuant to this Chapter;

- For sources not subject to subsection 5(A) and for which reference method emission testing that has been deemed by the Department to be representative of current and normal operating conditions, emission data from such testing shall serve as the basis for estimating emissions reported to the Department pursuant to this Chapter;

- For sources not subject to subsection 5(A) or (B), emissions reported pursuant to this Chapter shall be estimated and reported on the basis of a facility-specific emission factor approved by the Department;

- For sources not subject to subsection 5(A),(B) or (C), emissions reported pursuant to this Chapter shall be estimated and reported on the basis of EPA-published emission factors, where available;

- For sources not subject to subsection 5(A),(B),(C) or (D), emissions reported pursuant to this Chapter shall be estimated and reported based on emissions factors from other industry and trade groups based on sound science, where available;

- For sources not subject to subsection 5(A),(B),(C),(D) or (E), emissions reported pursuant to this Chapter shall be estimated and reported based on default emission factors published by the Department, where available; or

- For sources not subject to subsection 5(A),(B),(C),(D),(E) or (F), emissions reported pursuant to this Chapter shall be estimated and reported based on best engineering judgement.

________________________________________________

AUTHORITY: 38 M.R.S.A., Section 585-A, 585-C, and 575

EFFECTIVE DATE:	December 12, 1993

Amended:	November, 2, 1997

MINOR CORRECTIONS:	May 13, 1998 - formatting; last two Appendix pages.

June 23, 1998 - correcting last two Appendix pages.

AMENDED:	July 6, 2004 - filing 2004-251

AMENDED:	November 8, 2008 – filing 2008-528

________________________________________________

- APPENDIX A: HAZARDOUS AIR POLLUTANTS – REPORTING THRESHOLDS

CHAPTER 137 EMISSION STATEMENTS

| CAS Number | Pollutant Code | Hazardous Air Pollutant Name | Reporting Threshold (lbs) | Reporting Threshold (tons) |
| --- | --- | --- | --- | --- |
| 75-07-0 | 75070 | Acetaldehyde | 1,000 | 0.5 |
| 60-35-5 | 60355 | Acetamide | 1,000 | 0.5 |
| 75-05-8 | 75058 | Acetonitrile | 2,000 | 1 |
| 98-86-2 | 98862 | Acetophenone | 2,000 | 1 |
| 53-96-3 | 53963 | 2-Acetylaminofluorene | 2,000 | 1 |
| 107-02-8 | 107028 | Acrolein | 10 | 0.005 |
| 79-06-1 | 79061 | Acrylamide | 200 | 0.1 |
| 79-10-7 | 79107 | Acrylic Acid | 200 | 0.1 |
| 107-13-1 | 107131 | Acrylonitrile | 1,000 | 0.5 |
| 107-05-1 | 107051 | Allyl Chloride | 200 | 0.1 |
| 92-67-1 | 92671 | 4-Aminobiphenyl | 2,000 | 1 |
| 62-53-3 | 62533 | Aniline | 200 | 0.1 |
| 90-04-0 | 90040 | o-Anisidine | 200 | 0.1 |
|  | 92 | Antimony & Antimony Compounds†† | 200 | 0.1 |
|  | 93 | Arsenic & Arsenic Compounds (Also Inorganic Arsine) †† | 20 | 0.01 |
| 1332-21-4 | 1332214 | Asbestos | 10 | 0.005 |
| 71-43-2 | 71432 | Benzene | 1,000 | 0.5 |
| 92-87-5 | 92875 | Benzidine | 10 | 0.005 |
| 98-07-7 | 98077 | Benzotrichloride | 20 | 0.01 |
| 100-44-7 | 100447 | Benzyl Chloride | 1,000 | 0.5 |
|  | 109 | Beryllium & Beryllium Compounds†† | 20 | 0.01 |
| 92-52-4 | 92524 | Biphenyl | 2,000 | 1 |
| 117-81-7 | 117817 | Bis(2-Ethylhexyl) Phthalate | 2,000 | 1 |
| 542-88-1 | 542881 | Bis(Chloromethyl) Ether | 10 | 0.005 |
| 75-25-2 | 75252 | Bromoform | 2,000 | 1 |
| 106-99-0 | 106990 | 1,3-Butadiene | 200 | 0.1 |
|  | 125 | Cadmium & Cadmium Compounds†† | 10 | 0.005 |
| 156-62-7 | 156627 | Calcium Cyanamide | 1,000 | 0.5 |
| 133-06-2 | 133062 | Captan | 2,000 | 1 |
| 63-25-2 | 63252 | Carbaryl | 2,000 | 1 |
| 75-15-0 | 75150 | Carbon Disulfide | 2,000 | 1 |
| 56-23-5 | 52635 | Carbon Tetrachloride | 1,000 | 0.5 |
| 463-58-1 | 463581 | Carbonyl Sulfide | 1,000 | 0.5 |
| 120-80-9 | 120809 | Catechol | 2,000 | 1 |
| 133-90-4 | 133904 | Chloramben | 2,000 | 1 |
| 57-74-9 | 57749 | Chlordane | 10 | 0.005 |
| 7782-50-5 | 7782505 | Chlorine | 200 | 0.1 |
| 10049-04-4 | 10049044 | Chlorine Dioxide | 200 | 0.1 |
| 79-11-8 | 79118 | Chloroacetic Acid | 1,000 | 0.5 |
| 532-27-4 | 523374 | 2-Chloroacetophenone | 10 | 0.005 |
| 108-90-7 | 108907 | Chlorobenzene | 2,000 | 1 |
| 510-15-6 | 510156 | Chlorobenzilate | 1,000 | 0.5 |
| 67-66-3 | 67663 | Chloroform | 1,000 | 0.5 |
| 107-30-2 | 107302 | Chloromethyl Methyl Ether | 200 | 0.1 |
| 126-99-8 | 126998 | Chloroprene | 200 | 0.1 |
| 16065-83-1 | 16065831 | Chromium (III) (Trivalent Chromium) | ††† | ††† |
| 18540-29-9 | 18540299 | Chromium (VI) (Hexavalent Chromium) | ††† | ††† |
|  | 136 | Chromium & Chromium Compounds (includes chromium metal and all oxidation states)†† | 10 | 0.005 |
|  | 139 | Cobalt & Cobalt Compounds†† | 10 | 0.005 |
|  | 140 | Coke Oven Emissions | 200 | 0.1 |
| 95-48-7 | 95487 | o-Cresol | 2,000 | 1 |
| 108-39-4 | 108394 | m-Cresol | 2,000 | 1 |
| 106-44-5 | 106445 | p-Cresol | 1,000 | 0.5 |
| 1319-77-3 | 1319773 | Cresols/Cresylic Acid | 2,000 | 1 |
| 98-82-8 | 98828 | Cumene | 2,000 | 1 |
|  | 144 | Cyanide & Cyanide Compounds†† | 1,000 | 0.5 |
| 94-75-7 | 94757 | 2,4-Dichlorophenoxy Acetic Acid (salts and esters) | 2,000 | 1 |
| 334-88-3 | 334883 | Diazomethane | 2,000 | 1 |
| 132-64-9 | 132649 | Dibenzofuran | 2,000 | 1 |
| 96-12-8 | 96128 | 1,2-Dibromo-3-Chloropropane | 20 | 0.01 |
| 84-74-2 | 84742 | Dibutyl Phthalate | 2,000 | 1 |
| 106-46-7 | 106467 | 1,4-Dichlorobenzene | 2,000 | 1 |
| 91-94-1 | 91941 | 3,3'-Dichlorobenzidine | 200 | 0.1 |
| 111-44-4 | 111444 | Dichloroethyl Ether | 2,000 | 1 |
| 542-75-6 | 542756 | 1,3-Dichloropropene | 1,000 | 0.5 |
| 62-73-7 | 62737 | Dichlorovos | 200 | 0.1 |
| 111-42-2 | 111422 | Diethanolamine | 20 | 0.01 |
| 121-69-7 | 121697 | N,N-Dimethylaniline | 1,000 | 0.5 |
| 64-67-5 | 64675 | Diethyl Sulfate | 200 | 0.01 |
| DIISCOMP |  | Diisocynates | 2,000 |  |
| 119-90-4 | 119904 | 3,3'-Dimethoxybenzidine | 2,000 | 1 |
| 60-11-7 | 60117 | 4-Dimethylaminoazobenzene | 200 | 0.1 |
| 119-93-7 | 119937 | 3,3'-Dimethylbenzidine | 20 | 0.01 |
| 79-44-7 | 79447 | Dimethylcarbamoyl Chloride | 2,000 | 1 |
| 68-12-2 | 68122 | N,N-Dimethylformamide | 2,000 | 1 |
| 57-14-7 | 57147 | 1,1-Dimethyl Hydrazine | 200 | 0.1 |
| 131-11-3 | 131113 | Dimethyl Phthalate | 1,000 | 0.5 |
| 77-78-1 | 77781 | Dimethyl Sulfate | 1,000 | 0.5 |
| 534-52-1 | 534521 | 4,6-Dinitro-o-Cresol | 200 | 0.1 |
| 51-28-5 | 51285 | 2,4-Dinitrophenol | 1,000 | 0.5 |
| 121-14-2 | 121142 | 2,4-Dinitrotoluene | 1,000 | 0.5 |
| 123-91-1 | 123911 | 1,4-Dioxane (p-Dioxane) | 2,000 | 1 |
|  | 600 | Dioxin & Dioxin-like Compounds* | 0.0002 | 1x10-7 |
| 122-66-7 | 122667 | 1,2-Diphenylhydrazine | 200 | 0.1 |
| 106-89-8 | 106898 | Epichlorohydrin | 200 | 0.1 |
| 106-88-7 | 106887 | 1,2-Epoxybutane (1,2-Butylene oxide) | 2,000 | 1 |
| 140-88-5 | 140885 | Ethyl Acrylate | 2,000 | 1 |
| 100-41-4 | 100414 | Ethyl Benzene | 2,000 | 1 |
| 51-79-6 | 51796 | Ethyl Carbamate Chloride (Urethane) | 200 | 0.1 |
| 75-00-3 | 75003 | Ethyl Chloride (Chloroethane) | 20,000 | 10 |
| 106-93-4 | 106934 | Ethylene Dibromide (Dibromomethane) | 10 | 0.005 |
| 107-06-2 | 107062 | Ethylene Dichloride ( 1,2-Dichloroethane) | 1,000 | 0.5 |
| 107-21-1 | 107211 | Ethylene Glycol | 2,000 | 1 |
| 151-56-4 | 151564 | Ethyleneimine (Aziridine) | 2,000 | 1 |
| 75-21-8 | 75218 | Ethylene Oxide | 200 | 0.1 |
| 96-45-7 | 96457 | Ethylene Thiourea | 200 | 0.1 |
| 75-34-3 | 75343 | Ethylidene Dichloride | 2,000 | 1 |
|  | 383 | Fine Mineral Fibers | 2,000 | 1 |
| 50-00-0 | 50000 | Formaldehyde | 1,000 | 0.5 |
|  | 171 | Glycol Ethers†† | 2,000 | 1 |
| 76-44-8 | 76448 | Heptachlor | 10 | 0.005 |
| 118-74-1 | 118741 | Hexachlorobenzene | 10 | 0.005 |
| 87-68-3 | 87683 | Hexachlorobutadiene | 200 | 0.1 |
| 77-47-4 | 77474 | Hexachlorocyclopentadiene | 200 | 0.1 |
| 67-72-1 | 67721 | Hexachloroethane | 1,000 | 0.5 |
| 822-06-0 | 822060 | Hexamethylene Diisocynate | 20 | 0.001 |
| 680-31-9 | 680319 | Hexamethylphosphoramide | 2,000 | 1 |
| 110-54-3 | 110543 | Hexane | 2,000 | 1 |
| 302-01-2 | 302012 | Hydrazine | 20 | 0.01 |
| 7647-01-0 | 7647010 | Hydrochloric Acid (Acid Aerosol Only) | 2,000 | 1 |
| 7664-39-3 | 7664393 | Hydrogen Fluoride (Hydrofluoric Acid) | 1,000 | 0.5 |
| 7783-06-4 | 7783064 | Hydrogen Sulfide | 200 | 0.1 |
| 123-31-9 | 123319 | Hydroquinone | 1,000 | 0.5 |
| 78-59-1 | 78591 | Isophorone | 20,000 | 10 |
|  | 195 | Lead & Lead Compounds†† | 100 | 0.05 |
| 58-89-9 | 58899 | Lindane (1,2,3,4,5,6-Hexachlorocyclyhexane) | 200 | 0.1 |
| 108-31-6 | 108316 | Maleic Anhydride | 1,000 | 0.5 |
|  | 198 | Manganese & Manganese Compounds†† | 20 | 0.01 |
|  | 199 | Mercury & Mercury Compounds†† | 10 | 0.005 |
| 67-56-1 | 67561 | Methanol | 2,000 | 1 |
| 72-43-5 | 72435 | Methoxychlor | 100 | 0.05 |
| 74-83-9 | 74839 | Methyl Bromide (Bromomethane) | 1,000 | 0.5 |
| 74-87-3 | 74873 | Methyl Chloride | 2,000 | 1 |
| 71-55-6 | 71556 | Methyl Chloroform (1,1,1-Trichloroethane) | 2,000 | 1 |
| 60-34-4 | 60344 | Methyl Hydrazine | 200 | 0.1 |
| 74-88-4 | 74884 | Methyl Iodide (Iodomethane) | 1,000 | 0.5 |
| 108-10-1 | 108101 | Methyl Isobutyl Ketone | 2,000 | 1 |
| 624-83-9 | 624839 | Methyl Isocyanate | 200 | 0.1 |
| 74931 |  | Methyl Mercaptan | 1,000 |  |
| 80-62-6 | 80626 | Methyl Methacrylate | 2,000 | 1 |
| 1634-04-4 | 1634044 | Methyl Tert Butyl Ether | 20,000 | 10 |
| 101-14-4 | 101144 | 4,4'-Methylenebis(2-Chloroaniline) (MBOCA) | 200 | 0.1 |
| 75-09-2 | 75092 | Methylene Chloride (Dichloromethane) | 2,000 | 1 |
| 101-68-8 | 101688 | 4,4'-Methylenediphenyl Diisocyanate (MDI) | 200 | 0.1 |
| 1313-27-5 | 1313275 | Molybdenum Trioxide | 200 | 0.1 |
| 101-77-9 | 101779 | 4,4'-Methylenedianiline | 200 | 0.1 |
| 91-20-3 | 91203 | Napthalene | 1,000 | 0.5 |
|  | 226 | Nickel & Nickel Compounds†† | 20 | 0.01 |
| 98-95-3 | 98953 | Nitrobenzene | 1,000 | 0.5 |
| 92-93-3 | 92933 | 4-Nitrobiphenyl | 200 | 0.1 |
| 100-02-7 | 100027 | 4-Nitrophenol | 1,000 | 0.5 |
| 79-46-9 | 79469 | 2-Nitropropane | 20 | 0.01 |
| 684-93-5 | 684935 | N-Nitroso-N-Methylurea | 2,000 | 1 |
| 62-75-9 | 62759 | N-Nitrosodimethylamine | 10 | 0.005 |
| 59-89-2 | 59892 | N-Nitrosomorpholine | 20 | 0.01 |
| 56-38-2 | 56382 | Parathion | 1,000 | 0.5 |
| 82-68-8 | 82688 | Pentachloronitrobenzene (Quintobenzene) | 1,000 | 0.5 |
| 87-86-5 | 87865 | Pentachlorophenol | 1,000 | 0.5 |
| 108-95-2 | 108952 | Phenol | 2,000 | 1 |
| 106-50-3 | 106503 | p-Phenylendiamine | 2,000 | 1 |
| 75-44-5 | 75445 | Phosgene | 200 | 0.1 |
| 7803-51-2 | 7803512 | Phosphine | 200 | 0.1 |
| 7723-14-0 | 7723140 | Phosphorus | 20 | 0.01 |
| 85-44-9 | 85449 | Phthalic Anhydride | 2,000 | 1 |
| 1336-36-3 | 1336363 | Polychlorinated Biphenyls | 10 | 0.005 |
|  | 246 | Polycyclic Organic Matter*** | 100 | 0.05 |
| 1120-71-4 | 1120714 | 1,3-Propanesultone | 200 | 0.1 |
| 57-57-8 | 57578 | Beta-Propiolacetone | 2,000 | 1 |
| 123-38-6 | 123386 | Propionaldehyde | 2,000 | 1 |
| 114-26-1 | 114261 | Propoxur (Baygon) | 1,000 | 0.5 |
| 78-87-5 | 78875 | Propylene Dichloride (1,2-Dichloropropane) | 1,000 | 0.5 |
| 75-56-9 | 75569 | Propylene Oxide | 1,000 | 0.5 |
| 75-55-8 | 75558 | 1,2-Propylenimine (2-Methyl Aziridine) | 10 | 0.005 |
| 91-22-5 | 91225 | Quinoline | 200 | 0.1 |
| 106-51-4 | 106514 | Quinone | 200 | 0.1 |
|  | 605 | Radionuclides (Including Radon) | 2,000 | 1 |
|  | 253 | Selenium & Selenium Compounds†† | 200 | 0.1 |
| 100-42-5 | 100425 | Styrene | 2,000 | 1 |
| 96-09-3 | 96093 | Styrene Oxide | 2,000 | 1 |
| 7664-93-9 | 7664393 | Sulfuric Acid (Acid Aerosol Only) | 1,000 | 0.5 |
| 79-34-5 | 79345 | 1,1,2,2-Tetrachloroethane | 2,000 | 1 |
| 127-18-4 | 127184 | Tetrachloroethylene (Perchloroethylene) | 2,000 | 1 |
| 109-99-9 | 109999 | Tetrahydrofuran | 2,000 | 1 |
| 7550-45-0 | 7550450 | Titanium Tetrachloride | 20 | 0.01 |
| 108-88-3 | 108883 | Toluene | 2,000 | 1 |
| 95-80-7 | 95807 | Toluene-2,4-Diamine (2,4-Diaminotoluene) | 200 | 0.1 |
| 584-84-9 | 584849 | 2,4-Toluene Diisocyanate | 20 | 0.01 |
| 91-08-7 | 91087 | 2,6-Toluene Diisocyanate | 20 | 0.01 |
| 95-53-4 | 95534 | o-Toluidine | 1,000 | 0.5 |
| 8001-35-2 | 8001352 | Toxaphene (Chlorinated Camphene) | 10 | 0.005 |
| 120-82-1 | 120821 | 1,2,4-Trichlorobenzene | 2,000 | 1 |
| 79-00-5 | 79005 | 1,1,2-Trichloroethane | 1,000 | 0.5 |
| 79-01-6 | 79016 | Trichloroethylene | 2,000 | 1 |
| 88-06-2 | 88062 | 2,4,6-Trichlorophenol | 2,000 | 1 |
| 95-95-4 | 95954 | 2,4,5-Trichlorophenol | 2,000 | 1 |
| 121-44-8 | 121448 | Triethylamine | 1,000 | 0.5 |
| 1582-09-8 | 1582098 | Trifluralin | 100 | 0.05 |
| 540-84-1 | 540840 | 2,2,4-Trimethylpentane | 2,000 | 1 |
| 8006642 |  | Turpentine | 2,000 |  |
| 108-05-4 | 108054 | Vinyl Acetate | 2,000 | 1 |
| 593-60-2 | 593602 | Vinyl Bromide | 1,000 | 0.5 |
| 75-01-4 | 75014 | Vinyl Chloride | 200 | 0.01 |
| 75-35-4 | 75354 | Vinylidene Chloride (1,1-Dichloroethylene) | 1,000 | 0.5 |
| 106-42-3 | 106423 | p-Xylenes | 2,000 | 1 |
| 95-47-6 | 95476 | o-Xylenes | 2,000 | 1 |
| 108-38-3 | 108383 | m-Xylenes | 2,000 | 1 |
| 1330-20-7 | 1330207 | Xylenes (Isomers & Mixture) | 2,000 | 1 |

* For purposes of this rule, “Dioxin and Dioxin Like Compounds” means specific compounds in the following chemical classes that are “Dioxin like”: polychlorinated dibenzo-p-dioxins (PCDDs or CDDs), polychlorinated dibenzofurans (PCDFs or CDFs), polybrominated dibenzo-p-dioxins (PBDDs or BDDs), polybrominated dibenzofurans (PBDFs or BDFs), and polychlorinated biphenyls (PCBs). “Dioxin like” means those compounds that have a similar chemical structure and similar physical-chemical properties as 2,3,7,8 tetrachlorodibenzo-p-dioxin, and therefore induce a similar toxic response. Because of their hydrophobic nature and resistance towards metabolism, these chemicals persist and bioaccumulate in fatty tissues of animals and humans. The 7 dioxin like CDDs, 7 dioxin like BDDs, 10 dioxin like CDFs and 10 dioxin like BDFs are the congeners that have chlorine/bromine substitutions in, at a minimum, the 2, 3, 7, and 8 positions. Additionally, 13 PCB congeners have dioxin like toxicity; these are PCBs with 4 or more lateral chlorines with 1 or no substitution in the ortho position. These compounds are sometimes referred to as coplanar, meaning that they can assume a flat configuration with rings in the same plane. Mixed chlorinated and brominated congeners of dioxins, furans, and biphenyls may also meet the definition of dioxin like. “Dioxin and Dioxin Like Compounds” includes the following specific dioxin congeners, furan congeners, and dioxin like PCBs:

| Chemical notation | Isomer Groups | Reference Number |
| --- | --- | --- |
|  | Dioxin Congeners |  |
| 2,3,7,8-TCDDs | The isomers of tetrachlorodibenzo-p-dioxin with chlorine substitutions in the 2,3,7,8 positions |  |
| 1,2,3,7,8-PeCDDs | The isomers of pentachlorodibenzo-p-dioxin with chlorine substitutions in the 1,2,3,7,8 positions |  |
| 1,2,3,4,7,8-HxCDDs | The isomers of hexachlorodibenzo-p-dioxin with chlorine substitutions in the 2,3,7,8 positions |  |
| 1,2,3,6,7,8-HxCDDs | The isomers of hexachlorodibenzo-p-dioxin with chlorine substitutions in the 1,2,3,6,7,8 positions |  |
| 1,2,3,7,8,9-HxCDDs | The isomers of hexachlorodibenzo-p-dioxin with chlorine substitutions in the 1,2,3,7,8,9 positions |  |
| 1,2,3,4,6,7,8-HpCDDs | The isomers of heptachlorodibenzo-p-dioxin with chlorine substitutions in the 1,2,3,4,6,7,8 positions |  |
| OCDDs | The isomers of octachlorodibenzo-p-dioxin with chlorine substitutions in any position |  |
|  | Furan Congeners |  |
| 2,3,7,8-TCDFs | The isomers of tetrachlorodibenzofuran with chlorine substitutions in the 2,3,7,8 positions |  |
| 1,2,3,7,8-PeCDFs | The isomers of pentachlorodibenzofuran with chlorine substitutions in the 1,2,3,7,8 positions |  |
| 2,3,4,7,8-PeCDFs | The isomers of pentachlorodibenzofuran with chlorine substitutions in the 2,3,4,7,8 positions |  |
| 1,2,3,4,7,8-HxCDFs | The isomers of hexachlorodibenzofuran with chlorine substitutions in the 1,2,3,4,7,8 positions |  |
| 1,2,3,6,7,8-HxCDFs | The isomers of hexachlorodibenzofuran with chlorine substitutions in the 1,2,3,6,7,8 positions |  |
| 1,2,3,7,8,9-HxCDFs | The isomers of hexachlorodibenzofuran with chlorine substitutions in the 1,2,3,7,8,9 positions |  |
| 2,3,4,6,7,8-HxCDFs | The isomers of hexachlorodibenzofuran with chlorine substitutions in the 2,3,4,6,7,8 positions |  |
| 1,2,3,4,6,7,8-HpCDFs | The isomers of heptachlorodibenzofuran with chlorine substitutions in the 1,2,3,4,6,7,8 positions |  |
| 1,2,3,4,7,8,9-HpCDFs | The isomers of heptachlorodibenzofuran with chlorine substitutions in the 1,2,3,4,7,8,9 positions |  |
| OCDFs | The isomers of octachlorodibenzofuran with chlorine substitutions in any position |  |
|  | Dioxin Like PCBs | IUPAC** Number |
| 3,3',4,4'-TeCB | tetrachlorinated biphenyls with chlorine substituted in the 3,3',4,4' positions | PCB-77 |
| 3,4,4',5-TCB | tetrachlorinated biphenyls with chlorine substituted in the 3,4,4',5 positions | PCB-81 |
| 2,3,3',4,4'-PeCB | pentachlorinated biphenyls with chlorine substituted in the 2,3,3',4,4' positions | PCB-105 |
| 2,3,4,4',5-PeCB | pentachlorinated biphenyls with chlorine substituted in the 2,3,4,4',5 positions | PCB-114 |
| 2,3',4,4',5-PeCB | pentachlorinated biphenyls with chlorine substituted in the 2,3',4,4',5 positions | PCB-118 |
| 2',3,4,4',5-PeCB | pentachlorinated biphenyls with chlorine substituted in the 2',3,4,4',5 positions | PCB-123 |
| 3,3',4,4',5-PeCB | pentachlorinated biphenyls with chlorine substituted in the 3,3',4,4',5 positions | PCB-126 |
| 2,3,3',4,4',5-HxCB | hexachlorinated biphenyls with chlorine substituted in the 2,3,3',4,4',5 positions | PCB-156 |
| 2,3,3',4,4',5'-HxCB | hexachlorinated biphenyls with chlorine substituted in the 2,3,3',4,4',5' positions | PCB-157 |
| 2,3',4,4',5,5'-HxCB | hexachlorinated biphenyls with chlorine substituted in the 2,3',4,4',5,5' positions | PCB-167 |
| 3,3',4,4',5,5'-HxCB | hexachlorinated biphenyls with chlorine substituted in the 3,3',4,4',5,5' positions | PCB-169 |
| 2,3,3',4,4',5,5'-HpCB | heptachlorinated biphenyls with chlorine substituted in the 2,3,3',4,4',5,5' positions | PCB-189 |

** International Union of Pure and Applied Chemistry (http://iupac.chemsoc.org/general/about.html)

Reference: Draft Final Report: Exposure and Human Health Reassessment of 2,3,7,8-Tetrachlorodibenzo-p-Dioxin (TCDD) and Related Compounds, Part I: Estimating Exposure to Dioxin like Compounds, Volume 2: Sources of Dioxin like Compounds in the United States (Exposure Assessment and Risk Characterization Group, National Center for Environmental Assessment - Washington Office, Office of Research and Development, U.S. Environmental Protection Agency, Washington, DC; EPA/600/P-00/001Bb; http://www.epa.gov/ncea, (http://www.epa.gov/ncea/pdfs/dioxin/part1/volume2/volume2.pdf) September 2000.

***For the purposes of this rule, “Polycyclic Organic Matter” means specific compounds with more than one benzene ring and with a boiling point greater than or equal to 212º F. When determining whether emissions exceed the reporting threshold, the compounds must be summed and the total of all individual compounds compared against the reporting threshold of 100 pounds (0.05 tons). “Polycyclic Organic Matter” includes the following list of compounds.

| CAS Number | Pollutant Code | Polycyclic Organic Matter Compounds |
| --- | --- | --- |
| 208-96-8 | 208968 | Acenaphthylene |
| 83-32-9 | 83329 | Acenaphthene |
| 120-12-7 | 120127 | Anthracene |
| 56-55-3 | 56553 | Benz[a]Anthracene |
| 50-32-8 | 50328 | Benzo[a]Pyrene |
| 203-33-8 | 203338 | Benzo(a)fluoranthene |
| 205-99-2 | 205992 | Benzo[b]Fluoranthene |
| 195-19-7 | 195197 | Benzo(c)phenanthrene |
| 192-97-2 | 192972 | Benzo[e]Pyrene |
| 56832-73-6 | 56832736 | Benzofluoranthenes |
| 203-12-3 | 203123 | Benzo(g,h,i)Fluoranthene |
| 191-24-2 | 191242 | Benzo[g,h,i]Perylene |
| 205-82-3 | 205823 | Benzo[j]Fluoranthene |
| 207-08-9 | 207089 | Benzo[k]Fluoranthene |
| 91-58-7 | 91587 | 2-Chloronaphthalene |
| 218-01-9 | 218019 | Chrysene |
| 192-65-4 | 192654 | Dibenzo[a,e]Pyrene |
| 53-70-3 | 53703 | Dibenzo[a,h]Anthracene |
| 189-64-0 | 189640 | Dibenzo[a,h]Pyrene |
| 189-55-9 | 189559 | Dibenzo[a,i]Pyrene |
| 224-42-0 | 224420 | Dibenzo[a,j]Acridine |
| 191-30-0 | 191300 | Dibenzo[a,l]Pyrene |
| 57-97-6 | 57976 | 7,12-Dimethylbenz[a]Anthracene |
| 42397-64-8 | 42397648 | 1,6-Dinitropyrene |
| 42397-65-9 | 42397659 | 1,8-Dinitropyrene |
| 206-44-0 | 206440 | Fluoranthene |
| 86-73-7 | 86737 | Fluorene |
| 193-39-5 | 193395 | Indeno[1,2,3-cd]Pyrene |
| 65357-69-9 | 65357699 | Methylbenzopyrene |
| 56-49-5 | 56495 | 3-Methylcholanthrene |
| 26914-18-1 | 26914181 | Methylanthracene |
| 779-02-2 | 779022 | 9-Methyl Anthracene |
| 2422-79-9 | 2422799 | 12-Methylbenz[a]Anthracene |
| 41637-90-5 | 41637905 | Methylchrysene |
| 3697-24-3 | 3697243 | 5-Methylchrysene |
| 90-12-0 | 90120 | 1-Methylnaphthalene |
| 91-57-6 | 91576 | 2-Methylnaphthalene |
| 832-69-9 | 832699 | 1-Methylphenanthrene |
| 2381-21-7 | 2381217 | 1-Methylpyrene |
| 7496-02-8 | 7496028 | 6-Nitrochrysene |
| 607-57-8 | 607578 | 2-Nitrofluorene |
| 5522-43-0 | 5522430 | 1-Nitropyrene |
| 85-01-8 | 85018 | Phenanthrene |
| 198-55-0 | 198550 | Perylene |
| 129-00-0 | 129000 | Pyrene |

†† For the purposes of this rule, all listings above which contain the word "compounds" and for glycol ethers, the following applies: Unless otherwise specified, these listings are defined as including any unique chemical substance that contains the named chemical (i.e., antimony, arsenic, etc.) as part of that chemical's infrastructure.

††† For the purposes of determining whether chromium emissions exceed the reporting threshold, the trivalent and hexavalent forms must be summed with any other reported chromium compound emissions for the facility and compared against the reporting threshold of 10 pounds (0.005 tons).

APPENDIX B: HYDROFLUOROCARBONS AND PERFLUOROCARBONS

| CAS Number | Pollutant Code | Greenhouse Gas Name | Designation/ Refrigerant Number | Global Warming Potential (100 year/Third Assessment Report) |
| --- | --- | --- | --- | --- |
| Hydrofluorocarbons (HFCs) |  |  |  |  |
| 75-46-7 | 75467 | Trifluoromethane | HFC-23 / R-23 | 12,000 |
| 75-10-5 | 75105 | Difluoromethane | HFC-32 / R-32 | 550 |
| 593-53-3 | 593533 | Fluoromethane/Methyl Fluoride | HFC-41 / R-41 | 97 |
| 138495-42-8 | 138495428 | 1,1,1,2,3,4,4,5,5-Decafluoropentane | HFC-43-10mee | 1,500 |
| 354-33-6 | 354336 | Pentafluoroethane | HFC-125 / R-125 | 3,400 |
| 359-35-3 | 359-35-3 | 1,1,1,2-Tetrafluoroethane (Structure: CHF2CHF2) | HFC-134 | 1,100 |
| 811-97-2 | 811972 | 1,1,1,2-Tetrafluoroethane (Structure: CH2FCF3) | HFC-134a / R-134a | 1,300 |
| 430-66-0 | 460660 | 1,1,2-Trifluoroethane | HFC-143 | 330 |
| 420-46-2 | 420462 | 1,1,1-Trifluoroethane | HFC-143a / R-143a | 4,300 |
| 624-72-6 | 624726 | 1,2-Difluoroethane | HFC-152 / Freon 152 | 43 |
| 75-37-6 | 75376 | 1,1-Difluoroethane | HFC-152a / R-152a | 120 |
| 353-36-6 | 353366 | Fluoroethane | HFC-161 / R-161 | 12 |
| 431-89-0 | 431890 | 1,1,1,2,3,3,3-Heptafluoropropane | HFC-227ea | 3,500 |
| 677-56-5 | 677565 | 1,1,1,2,2,3-Hexafluoropropane | HFC-236cb | 1,300 |
| 431-63-0 | 431630 | 1,1,1,2,3,3-Hexafluoropropane | HFC-236ea | 1,200 |
| 690-39-1 | 690391 | 1,1,1,3,3,3-Hexafluoropropane | HFC-236fa | 9,400 |
| 679-86-7 | 679867 | 1,1,2,2,3-Pentafluoropropane | HFC-245ca | 640 |
| 460-73-1 | 460731 | 1,1,1,3,3-Pentafluoropropane | HFC-245fa | 950 |
| 406-58-5 | 406586 | 1,1,1,3,3-Pentafluorobutane | HFC-365mfc | 890 |
| Perfluorocarbons (PFCs) |  |  |  |  |
| 75-73-0 | 75730 | Carbon Tetrafluoride (CF4) | R-14 | 5,700 |
| 76-16-4 | 76164 | Hexafluoroethane (C2F6) | R-116 | 11,900 |
| 76-19-7 | 79197 | Perfluoropropane (C3F8) | R-218 | 8,600 |
| 355-25-9 | 355259 | Perfluoro-n-butane (C4F10) |  | 8,600 |
| 115-25-3 | 115253 | Octofluorcyclobutane (c-C4F8) |  | 10,000 |
| 678-26-2 | 678262 | Perfluoropentane (C5F12) |  | 8,900 |
| 355-42-0 | 355420 | Perfluorohexane (C6F14) |  | 9,000 |
| 7783-54-2 | 7783542 | Nitrogen Trifluoride (NF3) |  | 10,800 |