# Chapter 149:			GENERAL PERMIT REGULATION FOR NONMETALLIC MINERAL PROCESSING PLANTS

SUMMARY: Through this Chapter, the Department of Environmental Protection (Department) regulates stationary and portable nonmetallic mineral processing plants (NMMPPs) that are subject to Federal New Source Performance Standards (NSPS) and those subject to state regulations. Owners and operators of eligible NMMPPs may obtain specific streamlined regulatory coverage under this permit by rule (also referred to as a general permit) pursuant to 38 Maine Revised Statutes (M.R.S.) § 344(7) in lieu of an air emission license issued pursuant to Major and Minor Source Air Emission License Regulation, 06-096 Code of Maine Rules (C.M.R.) ch. 115. Such parties do so by obtaining a Crusher Identification Number (CIN) for each individual rock crusher and submitting a Notification of Intent to Comply (NOITC) attesting to their formal agreement to abide by all applicable requirements of this Chapter. If the construction, modification, or operation of an NMMPP would not comply with all requirements of this Chapter, the owner or operator must apply for and obtain an air emission license under 06-096 C.M.R. ch. 115, before beginning the actual construction, modification, or operation of the source.

Applicability

- This Chapter applies statewide.

- The registrations and notifications required to comply with this general permit are exempt from Processing of Applications and Other Administrative Matters, 06-096 C.M.R. ch. 2 pursuant to section 2(B) of that rule.

- This Chapter applies to all NMMPPs not included in an air emission license issued pursuant to 06-096 C.M.R. ch. 115. This Chapter also includes requirements in section 5 of this Chapter for nonroad engines that are used to power NMMPPs.

- The Owner must obtain a CIN pursuant to section 3 of this Chapter for each individual crusher that is part of a NMMPP.

- Requirements for engines identified in section 5 of this Chapter do not apply to nonroad engines used in or on a piece of equipment that is self-propelled or serves a dual purpose by both propelling itself and performing another function.

- This Chapter does not exempt a source from any applicable state or federal requirements including, but not limited to, those included in 40 Code of Federal Regulations (C.F.R.) Part 60 and 40 C.F.R. Part 63.

- Notwithstanding an activity being listed above, if upon review of a registration or NOITC filed pursuant to this Chapter the Department determines the proposed activity may result in significant impact on the environment, the Department may require an owner or operator to obtain an air emission license for the activity pursuant to 06-096 C.M.R. ch. 115.

- Exclusions. A rock crusher is not eligible for a CIN and the owner or operator must instead obtain an air emission license pursuant to 06-096 C.M.R. ch. 115 if any of the following circumstances apply:

  - A control method other than water sprays or carryover from up-stream water sprays is used to control emissions from the crusher and other NMMPP equipment;

  - The Department determines that the owner or operator cannot comply with the requirements of this Chapter;

  - Any part of the NMMPP is powered by an engine that cannot meet the definition of nonroad engine;

  - The owner or operator owes any past due fees or civil penalties to the Department from previous air emission licenses, permits, or consent agreements related to air emissions;

  - The Department has reasonable cause to believe that emissions from the proposed, modified, or relocated source will violate the control strategy or interfere with attainment or maintenance of a national ambient air quality standard in Maine or in a neighboring state; or

  - The NMMPP is to be operated at a major source, and the NMMPP and major source will be under common control.

Definitions

The following terms, as used in this Chapter, have the following meanings:

- Nonmetallic mineral processing plant. “Nonmetallic mineral processing plant” or “NMMPP” means any combination of equipment that is used to crush or grind any nonmetallic mineral including crushers, grinding mills, screening operations, bucket elevators, belt conveyors, bagging operations, storage bins, and enclosed truck or railcar loading stations.

- Nonroad engine. “Nonroad engine” means an internal combustion engine that meets any of the following criteria:

- It is (or will be) used in or on a piece of equipment that is self-propelled or serves a dual purpose by both propelling itself and performing another function.

- It is portable or transportable, meaning designed to be and capable of being carried or moved from one location to another. Indicia of transportability include, but are not limited to, wheels, skids, carrying handles, dolly, trailer, or platform. Engines that are portable or transportable do not include engines that remain or will remain at a location for more than 12 consecutive months or a shorter period of time for an engine located at a seasonal source. A location is any single site at a building, structure, facility, or installation. For any engine that replaces an engine at a location and that is intended to perform the same or similar function as the engine replaced, include the time period of both engines in calculating the consecutive time period. 

An engine located at a seasonal source is an engine that remains at a seasonal source during the full annual operating period of the seasonal source. A seasonal source is a stationary source that remains in a single location on a permanent basis (i.e., at least two years) and that operates at that single location approximately three months (or more) each year.

- Operator. “Operator” means any person who has direct control or supervision over a regulated source (rock crusher and/or associated engine) and who has registered agreement to operate such equipment in accordance with the conditions of this Chapter. The operator may also be the owner.

- Owner. “Owner” means any person who legally owns a regulated source (rock crusher and/or engine) and who receives a CIN for that regulated source pursuant to this Chapter. The owner may also be the operator.

- Performance test. “Performance test” means a certified visible emissions observation performed pursuant to 40 C.F.R. Part 60, Appendix A-4, Method 9 (Method 9). The duration of the Method 9 observation must be no less than 30 minutes (five 6-minute averages).

- Portable plant. “Portable plant” means any NMMPP that is mounted on any chassis or skids and that may be moved by the application of a lifting or pulling force. A portable plant may not have any cable, chain, turnbuckle, bolt, or other means (except electrical connections) by which it is attached or clamped to any anchor, slab, or structure, including bedrock, that must be removed prior to the application of a lifting or pulling force for the purpose of transporting the unit.

- Rock crusher or crusher. “Rock crusher” or “crusher” means a machine used to crush any nonmetallic minerals, and includes, but is not limited to, the following types: jaw, gyratory, cone, roll, rod mill, hammermill, and impactor.

- Safe access. “Safe access” means reasonable access to the regulated source in compliance with applicable safety requirements of all local, state, or federal regulating authorities and the written safety standard operating procedures for that source.

- Stationary plant. “Stationary plant” means any NMMPP that does not meet the definition of “portable plant.”

General Terms and Conditions for Registration

- Registration of owner and operator. Prior to the operation of a NMMPP, the owner must either obtain an air emission license pursuant to 06-096 C.M.R. ch. 115 or register the equipment with the Department pursuant to this Chapter and receive a CIN for each unit to be operated. If the owner elects to register the equipment pursuant to this Chapter, the operator must submit the NOITC to the Department pursuant to section 3(D) of this Chapter prior to operation of the equipment. Provided the owner or operator meet the applicability requirements of this Chapter, the registration and NOITC will be considered to be in effect upon submittal to the Department.

- Required CIN registration form and additional information. The owner must submit to the Department the information listed below, unless otherwise specified by this Chapter, and any other additional information required by the Department. Failure to provide all information required by the Department to determine the applicability of this Chapter may result in rejection of the registration. This information must be provided on a form prescribed by the Department.

  - Identifying information, including contact information for the owner;

  - The type (e.g., cone or jaw), maximum processing rate, and date of manufacture of the crusher for which a CIN is sought;

  - A unique identifier, such as a serial number, associated with the crusher;

  - Whether the crusher is powered by an engine and, if so, confirmation that the engine meets the definition of nonroad engine;

  - Confirmation that the crusher has operational water sprays;

  - Any other information that may be necessary to implement and enforce any federal or state applicable requirements for air emissions;

  - If required by the Department, proposed monitoring, testing, recordkeeping and reporting protocols, and results of previous performance tests; and

  - A certification statement pursuant to section 3(F) of this Chapter.

- Equipment identification. Once a registration has been received, the Department will assign a unique CIN to the rock crusher. The rock crusher associated with the CIN must be clearly marked (engraved, stenciled, etched, or otherwise permanently affixed) with one of the following:

  - The CIN number, or

  - A serial number or other unique equipment number that is also listed in the CIN  registration described in section 3(B) of this Chapter.

- Notice of intent to comply. The operator must submit a NOITC on a form designated by the Department. The NOITC must be submitted to the Department at least two state working days prior to commencing operation of the equipment. A copy of the NOITC must also be provided to the municipality where the equipment will be located, except in the case of an unorganized territory where notification must be provided to the respective county commissioner. Once the NOITC is submitted, the operator is bound by the conditions of this Chapter and must comply with all applicable conditions until such time as the operator informs the Department, in writing, that they no longer intend to operate the listed equipment, the owner voluntarily surrenders the CIN, or the CIN is transferred to a new owner pursuant to section 8 of this Chapter .

- Required NOITC information. The NOITC must be in a form prescribed by the Department. The NOITC form must include, but is not limited to, the following elements:

  - The CIN issued by the Department if the NOITC is submitted separately from the CIN registration form;

  - Identifying information, including the NMMPP location along with contact information for the operator;

  - A statement that the operator intends to comply with and operate the listed equipment in accordance with the terms and conditions set forth in this Chapter;

  - Any other information that may be necessary to implement and enforce any applicable federal or state requirements for air emissions;

  - A certification statement pursuant to section 3(F) of this Chapter.

- Certification. All CIN registrations and NOITC forms submitted to the Department in accordance with this chapter must contain a certification of truth, accuracy, and completeness with the signature and printed name of the responsible official as defined in Definitions Regulation, 06-096 C.M.R. ch. 100. The signature must either be an original wet-ink signature or an electronic signature submitted through a licensing system provided by the Department, if available. The certification must state:

“I certify under penalty of law that I have personally examined the information submitted in the document and all attachments thereto, and that, based on my inquiry of those individuals immediately responsible for obtaining the information, I believe the information is true, accurate, and complete. I authorize the Department to enter the property that is the subject of this registration at reasonable hours, including buildings, structures, or conveyances on the property, to determine the accuracy of any information provided herein. I am aware there are significant civil and criminal penalties for submitting false information, including the possibility of fine and imprisonment.”

The NOITC form must also include the following certification:

“I certify that the equipment listed in this notification will be operated in compliance with the terms and conditions of General Permit Regulation for Nonmetallic Mineral Processing Plants, 06-096 C.M.R. ch. 149, and any other applicable state or federal air emission regulations.”

Upon becoming aware that incomplete or incorrect information has been submitted, the owner or operator, as applicable, must provide the Department with the supplementary facts or corrected information.

- Public notice of intent to file. Public notification is not required for the submission or processing of a CIN registration or NOITC.

- Fees. The owner must pay an annual fee to the Department pursuant to 38 M.R.S. § 353-A(10). The first year’s fee is due with the CIN application and covers the time period from the issuance of the CIN until January 1st of the following year.

- Submittal address. The CIN registration and NOITC must be filed with the Department of Environmental Protection, Bureau of Air Quality, 17 State House Station, Augusta, ME 04333-0017. Submittal may be made electronically through a licensing system provided by the Department, if available.

- Source obligation. A CIN or submittal of a NOITC does not relieve any owner or operator of a source from the responsibility to comply with any other requirements applicable to the source.

- Public access to information and confidentiality. All information and data submitted to the Department is subject to the provisions of the Freedom of Access Law, 1 M.R.S. § 401 et seq., as amended. Documents which the owner or operator believes may not be subject to disclosure under the Freedom of Access Law should be clearly marked as “claimed confidential” at the time of submission. Such a claim of confidentiality does not in and of itself protect the documents from disclosure but alerts the Department to the owner or operator’s position that the documents may not be subject to disclosure.

General Permit Requirements for Owners

- General Conditions

- Employees and authorized agents of the Department may enter and inspect the operator’s business premises pursuant to 38 M.R.S. § 347-C. The owner or a designee must provide safe access in compliance with safety requirements of all local, state, and federal regulating authorities as well as all written safety standard operating procedures for that source.

- The owner must pay the annual air emission general permit fee to the Department, pursuant to 38 M.R.S. § 353-A(10).

- The CIN does not convey any property rights of any sort or any exclusive privilege.

- The owner must comply with all applicable terms and conditions of this general permit. The notification of planned changes or anticipated noncompliance, or the filing of an application for an air emission license issued pursuant to 06-096 C.M.R. ch. 115 does not stay any condition of this general permit.

- The owner may not use as a defense in an enforcement action that the disruption, cessation, or reduction of operations would have been necessary in order to maintain compliance with the conditions of the general permit.

- Notwithstanding any other provisions in the State Implementation Plan approved by the U.S. Environmental Protection Agency (EPA) or Section 114(a) of the Clean Air Act, any credible evidence may be used for the purpose of establishing whether a person has violated or is in violation of any statute, regulation, or requirement pursuant to this Chapter.

- Upon written request from the Department, the owner must establish and maintain such records; make such reports; install, use, and maintain such monitoring equipment; sample such emissions in accordance with such methods, at such locations, at such intervals, and in such a manner as the Department prescribes; and provide other information as the Department may reasonably require to determine compliance status.

- Rock crushers subject to NSPS. An owner of a stationary plant with a maximum capacity greater than 25 tons/hour or a portable plant with a maximum capacity greater than 150 tons/hour is subject to and must comply with 40 C.F.R. Part 60, Subpart OOO, Standards of Performance for Nonmetallic Mineral Processing Plants, if any construction, reconstruction, or modification of the crusher began after August 31, 1983. Notwithstanding 40 C.F.R. § 60.11(c), the visible emission standards apply at all times. Requirements include, but are not limited to:

- The owner must either have an initial performance test performed on the rock crusher pursuant to the applicable sections of 40 C.F.R. § 60.675 or provide documentation to the Department that the initial performance test was previously performed, along with the results of the test. Documentation that a successful initial performance test on the equipment was performed outside of Maine is acceptable.

- An initial performance test must be completed within 60 days after achieving the maximum production rate at which the unit will be operated, but no later than 180 days after initial startup of the unit. If the deadline for an initial performance test for a source falls within a seasonal shutdown, then, with written approval from the Department, the initial performance test may be postponed until no later than 60 calendar days after the next initiation of operation of the affected equipment.

- The owner must submit a test notice to the Department’s regional air compliance inspector at least seven days prior to a performance test.

- For crushers for which construction, reconstruction, or modification began after April 22, 2008, the owner must, at the time of the initial performance test, designate which water sprays will be regularly inspected as part of the continuous compliance demonstration.

General Permit Requirements for Operators

- General Conditions

- Employees and authorized agents of the Department may enter and inspect the operator’s business premises pursuant to 38 M.R.S. § 347-C. The operator, or a designee, must provide safe access in compliance with safety requirements of all local, state, and federal regulating authorities, as well as the written safety standard operating procedures for that source.

- The operator must establish and maintain a continuing program of best management practices for suppression of fugitive particulate matter emissions during any period of construction, reconstruction, or operation which may result in fugitive dust and must submit a description of the program to the Department upon request.

- The CIN does not convey any property rights of any sort, or any exclusive privilege.

- The operator must maintain and operate as necessary all emission units and air pollution systems required by the general permit in a manner consistent with good air pollution control practice for minimizing emissions.

- The operator must maintain sufficient records to accurately document compliance with emission standards and general permit conditions and must maintain such records for a minimum of six years. The records must be submitted to the Department upon written request.

- The operator must comply with all applicable terms and conditions of this general permit. The notification of planned changes or anticipated noncompliance, or the filing of an application for a license issued pursuant to 06-096 C.M.R. ch. 115 does not stay any condition of this general permit.

- The operator may not use as a defense in an enforcement action that the disruption, cessation, or reduction of operations would have been necessary in order to maintain compliance with the conditions of the general permit.

- In accordance with the Department’s air emission compliance test protocol and 40 C.F.R. Part 60, Appendix A, or other method approved or required by the Department, the operator must:

- Perform testing to demonstrate compliance with the applicable emission standards under circumstances representative of the source’s normal process and operating conditions:

- Within 60 calendar days of receipt of a notification to test from the Department or the EPA, if visible emissions, equipment operating parameters, staff inspection, air monitoring, or other cause indicate to the Department that equipment may be operating out of compliance with emission standards or requirements of this Chapter; or

- Pursuant to any other requirement of this general permit to perform testing.

- Install or make provisions to install test ports that meet the criteria of 40 C.F.R. Part 60, Appendix A, and test platforms, if necessary, and other accommodations necessary to allow emission testing; and

- Submit a written report to the Department within 30 days from date of test completion.

- If the results of a test performed under circumstances representative of the source’s normal process and operating conditions indicate emissions in excess of the applicable standards, then:

- Within 30 days following receipt of such test results, the operator must re-test the non-complying emission source under circumstances representative of the source’s normal process and operating conditions and in accordance with the Department’s air emission compliance test protocol and 40 C.F.R. Part 60 or other method approved or required by the Department;

- The days of violation include the day of the test showing noncompliance and each and every day of operation thereafter until compliance is demonstrated under normal and representative process and operating conditions, except to the extent that the source can prove to the satisfaction of the Department that there were intervening days during which no violation occurred or that the violation was not continuing in nature; and

- The operator may, upon the approval of the Department and following the successful demonstration of compliance at alternative load conditions, operate under such alternative load conditions on an interim basis in accordance with the air emission license prior to a demonstration of compliance under normal and representative process and operating conditions.

- Notwithstanding any other provisions in the State Implementation Plan approved by the EPA or Section 114(a) of the CAA, any credible evidence may be used for the purpose of establishing whether a person has violated or is in violation of any statute, regulation, or requirement under this Chapter.

- The operator must maintain records of malfunctions, failures, downtime, and any other similar change in operation of air pollution control systems or the emissions unit itself that would affect emissions and that is not consistent with the terms and conditions of this general permit. The operator must notify the Department within two days or by the next state working day, whichever is later, of any occasions where such changes result in an increase of emissions. The operator must report all excess emissions in the units of the applicable emission limitation.

- Upon written request from the Department, the operator must establish and maintain such records; make such reports; install, use, and maintain such monitoring equipment; sample such emissions (in accordance with such methods, at such locations, at such intervals, and in such a manner as the Department prescribes); and provide other information as the Department may reasonably require to determine compliance status.

- The operator must maintain a copy of the NOITC and provide it to Department representatives upon request.

- The operator must ensure that the equipment operator(s) are familiar with the terms and conditions of this general permit.

- Notwithstanding any part of this Chapter, the operator is subject to the applicable requirements of Visible Emissions Regulation, 06-096 C.M.R. ch. 101, including limits on fugitive emissions and must comply with such requirements.

- NMMPPs

- Visible emissions from the rock crusher must not exceed 10% opacity on a six-minute block average basis.

- Visible emissions from any affected source other than a rock crusher, including grinding mills, screening operations, bucket elevators, transfer points on belt conveyors, bagging operations, storage bins, or enclosed truck or railcar loading stations must not exceed 7% opacity on a six-minute block average basis.

- The operator must install and maintain spray nozzles for control of particulate matter and operate them as necessary (continuous or intermittent basis depending on whether processed material contains sufficient moisture) to limit visible emissions to the opacity standards listed in sections 5(B)(1) and (2) above.

- The operator must maintain a log detailing the maintenance performed on particulate matter control equipment including, but not limited to, spray nozzles. At a minimum, the operator must perform monthly inspections of all water sprays to confirm water is flowing to the correct locations and initiate corrective action within 24 hours if water is found to not be flowing properly. Records of the date of each inspection and any corrective action required must be included in the maintenance log. The maintenance log must be kept on-site at the rock crushing location.

- The operator must maintain records of the dates and times of all operating hours for the rock crusher. The operation log must be kept on-site at the rock crushing location and made available to Department staff upon request.

- Engines used to power NMMPPs. The requirements of this section apply to nonroad engines that power NMMPPs. These requirements do not apply to nonroad engines used in or on a piece of equipment that is self-propelled or serves a dual purpose by both propelling itself and performing another function.

- Engines must meet the definition of nonroad engine in section 2(B) of this Chapter.

- Engines must fire only distillate fuel with a sulfur content not to exceed 15 parts per million, propane, or natural gas.

- The operator must not operate any engine as a dispatchable load generator (e.g., provide power to ISO New England or any other electrical system operator).

- The operator must operate and maintain each engine in accordance with the manufacturer’s written instructions. The operator may only change settings as approved by the manufacturer.

- If the engine is equipped with a diesel particulate filter, the operator must keep records of all corrective action taken when the high backpressure limit is approached.

- Equipment Relocation

- An operator must provide written notification to the Department prior to relocation of a crusher. The notification must include the physical address or GPS coordinates of the equipment’s new location and the CIN pertaining to the relocated equipment.

Note: It is preferred that notice of the relocation be submitted through the Department’s on-line e-notice at the website listed below or other online form provided by the Department:

www.maine.gov/dep/air/compliance/forms/relocation

Written notice may also be sent by mail to the address below.

Attn: Relocation Notice

Maine DEP

Bureau of Air Quality

17 State House Station

Augusta, ME 04333-0017

- Written notification must also be provided to the municipality where the equipment will be relocated, except in the case of an unorganized territory, in which case notification must be provided to the respective county commissioner.

- If the Department has reasonable cause to believe that emissions from the relocated source will violate the control strategy or interfere with attainment or maintenance of a national ambient air quality standard in Maine or in a neighboring state, the Department may require the source to obtain an air emission license pursuant to 06-096 C.M.R. ch. 115.

- Fugitive Emissions

The operator must take measures to ensure that visible emissions from fugitive emission sources, such as stockpiles and roadways, meet the visible emission standards for fugitive emissions contained in 06-096 C.M.R. ch. 101, § 4(C).

Severability. Each part of this Chapter is severable, and in the event that any part of this Chapter is held to be invalid, the remainder of the Chapter will continue in full force and effect.

Voluntary Surrender. If the annual fee required by section 4(A)(2) of this Chapter is not paid in full by July 31st of each year, the Department will consider the owner to have voluntarily surrendered the CIN pursuant to 38 M.R.S. § 344(10), and the owner must register for a new CIN, and the operator must submit a new NOITC before resuming operation of the NMMPP.

Transfer. The following outlines the procedure for transferring a CIN to a new owner.

- Applicability. A CIN may be transferred to a new owner if the CIN has not expired. The transfer of a CIN to a new owner immediately voids any existing NOITC associated with the CIN.

- Required information. The transferee must submit:

- The CIN registration form as specified in section 3(B) of this Chapter identifying the CIN associated with the NMMPP and the new owner;

- The NOITC as specified in sections 3(D) and (E) of this Chapter for any operator who intends to operate the NMMPP;

- Written statement from the previous owner acknowledging that they are relinquishing the CIN; and

- Payment in full of any past-due fees associated with the CIN.

- Criteria for transfer approval. Approval of the CIN transfer will be based on the acceptability of the information required in the transfer submittal.

________________________________________

AUTHORITY: 38 M.R.S. §§ 344(7) and 585-A

EFFECTIVE DATE:

August 4, 2008, filing 2008-337

AMENDED:

April 27, 2014 – filing 2014-083

AMENDED:

December 1, 2025 – filing 2025-235

APAO ACCESSIBILITY CHECK: December 4, 2025 (no issues detected)