# Chapter 165:			GENERAL PERMIT FOR CLASS IV-A INCINERATORS

SUMMARY: Through this Chapter, the Department of Environmental Protection (Department) regulates air emissions from Class IV-A incinerators, including veterinary incinerators and human crematory incinerators. Owners or operators of eligible Class IV-A incinerators may obtain specific streamlined regulatory coverage under this permit by rule (also referred to as a general permit) pursuant to 38 Maine Revised Statutes (M.R.S.) § 344(7) in lieu of an air emission license issued pursuant to Major and Minor Source Air Emission License Regulation, 06-096 Code of Maine Rules (C.M.R.) ch. 115. Such parties must do so by obtaining a Class IV-A incinerator General Permit Number (GPN) for each Class IV-A incinerator unit by submitting a registration attesting to formal agreement to abide by all conditions of this Chapter. If the construction, modification, or operation of a Class IV-A incinerator by the owner or operator would not comply with all requirements of this Chapter, the owner or operator must apply for and obtain an air emission license pursuant to 06-096 C.M.R. ch. 115 before beginning the actual construction, modification, or operation of the source.

Applicability

- This Chapter applies statewide.

- The registrations and notifications required to comply with this general permit are exempt from Processing of Applications and Other Administrative Matters, 06-096 C.M.R. ch. 2 pursuant to section 2(B) of that rule.

- This Chapter applies to all Class IV-A veterinary incinerators and crematory incinerators unless the unit would be required to be included in an air emission license issued pursuant to 06-096 C.M.R. ch. 115.

- The Owner must obtain a GPN pursuant to section 3 of this Chapter for each individual Class IV-A Incinerator.

- This Chapter does not exempt a source from any applicable state or federal requirements including, but not limited to, those included in 40 Code of Federal Regulations (C.F.R.) Part 60 and 40 C.F.R. Part 63.

- Notwithstanding an activity being listed above, if upon review of a registration filed pursuant to this Chapter the Department determines the proposed activity may result in significant impact on the environment, the Department may require an owner or operator to obtain an air emission license for the activity pursuant to 06-096 C.M.R. ch. 115.

- Exclusions. A Class IV-A incinerator is not eligible for a GPN pursuant to this Chapter and the owner or operator must instead obtain an air emission license pursuant to 06-096 C.M.R. ch. 115 if any of the following circumstances apply:

- The Department determines that the owner or operator cannot comply with the requirements of this Chapter;

- The Class IV-A incinerator has a maximum heat input (all burners combined) equal to or greater than 10.0 million British thermal units per hour (MMBtu/hr);

- The owner or operator owes any past due fees or civil penalties to the Department from previous emission licenses, permits, or consent agreements related to air emissions;

- The Department has reasonable cause to believe that emissions from the proposed, modified, or relocated source will violate the control strategy or interfere with attainment or maintenance of a national ambient air quality standard in Maine or in a neighboring state; or

- The Class IV-A incinerator is to be operated at a major source, and the Class IV-A incinerator and major source will be under common control.

Definitions

The following terms, as used in this Chapter, have the following meanings:

- Class IV-A human crematory incinerator. “Class IV-A human crematory incinerator” or “crematory incinerator” means any crematory and pathological incinerators suitable for human remains such as bodies, organs, and solid organic wastes from hospitals, laboratories and similar sources consisting of up to 85% moisture, up to 5% incombustible solids, and a heating value of approximately 1,000 British thermal units per pound (BTU/lb) as fired.

- Class IV-A veterinary incinerator. “Class IV-A veterinary incinerator” or “veterinary incinerator” means any crematory or pathological incinerators suitable for the cremation of animal remains such as carcasses, organs, and solid organic wastes consisting of up to 85% moisture, up to 5% incombustible solids, and a heating value of approximately 1,000 BTU/lb as fired.

- Incinerator operator. “Incinerator operator” means any person designated to operate a veterinary incinerator or a crematory incinerator unit in accordance with the manufacturer’s operating guidelines under the supervision of an owner or operator.

- Owner or operator. “Owner or operator” means any person who owns or has direct control or supervision over a veterinary incinerator or crematory incinerator and who receives a GPN under the terms of this Chapter.

- Safe access. “Safe access” means reasonable access to the regulated source in compliance with applicable safety requirements of any local, state, or federal regulating authority as well as the written safety standard operating procedures for that source.

General Terms and Conditions for Registration

- Registration of owner or operator. Prior to the construction, modification, or operation of a veterinary incinerator or crematory incinerator, the owner or operator must either obtain an air emission license pursuant to the requirements of 06-096 C.M.R. ch. 115 or register the equipment with the Department to obtain a GPN for each unit to be operated.  Provided the owner or operator meets the applicability requirements of this Chapter, the registration and NOITC will be considered to be in effect upon submittal to the Department.

- Required GPN registration form and additional information. The owner or operator must submit to the Department the information listed below, unless otherwise specified by this Chapter, and any other additional information required by the Department. Failure to provide all information required by the Department to determine the applicability of this Chapter may result in rejection of the registration. Information must be provided in a form prescribed by the Department.

  - Identifying information, including the Class IV-A incinerator location and contact information for the owner or operator;

  - The manufacturer, model, date(s) of manufacture and installation, maximum heat input, and maximum charging rate of the Class IV-A incinerator to be issued a GPN;

  - A unique identifier, such as a serial number, associated with the Class IV-A incinerator;

  - Any other information that may be necessary to implement and enforce any applicable state or federal requirements for air emissions;

  - If required by the Department, proposed monitoring, testing, recordkeeping and reporting protocols, and results of previously performed performance tests;

  - A certification statement as set forth in section 3(D) of this Chapter; and

  - Proof of publication of the public notice of intent to file required by section 3(E) of this Chapter.

- Equipment identification. Once registration has been received, the Department will assign a unique GPN for each veterinary incinerator or crematory incinerator. The Class IV-A incinerator associated with the GPN must be clearly marked (engraved, stenciled, etched, or otherwise permanently affixed) with one of the following:

  - The GPN number, or

  - A serial number or other unique equipment number that is also listed in the GPN registration described in section 3(B) of this Chapter.

- Certification. All GPN registrations submitted to the Department in accordance with this Chapter must contain a certification of truth, accuracy, and completeness with the signature and printed name of the responsible official as defined in Definitions Regulation, 06-096 C.M.R. ch. 100. The signature must either be an original wet-ink signature or an electronic signature submitted through a licensing system provided by the Department. The certification must state:

“I certify that the equipment listed in this registration will be operated in compliance with the terms and conditions of General Permit for Class IV-A Incinerators, 06-096 C.M.R. ch. 165, and any other applicable state or federal air emission regulations. I certify under penalty of law that I have personally examined the information submitted in the document and all attachments thereto, and that, based on my inquiry of those individuals immediately responsible for obtaining the information, I believe the information is true, accurate, and complete. I authorize the Department to enter the property that is the subject of this registration at reasonable hours, including buildings, structures, or conveyances on the property, to determine the accuracy of any information provided herein. I am aware there are significant civil and criminal penalties for submitting false information, including the possibility of fine and imprisonment.”

Upon becoming aware that incomplete or incorrect information has been submitted, the owner or operator must provide the Department with the supplementary facts or corrected information.

- Public notice of intent to file. The owner or operator must give public notice of intent to file as described in 06-096 C.M.R. ch. 115, § 2(D).

- Fees. The owner or operator must pay an annual fee to the Department pursuant to 38 M.R.S. § 353-A(10). The first year’s fee is due with the GPN registration and covers the time period from the issuance of the GPN until January 1st of the following year.

- Submittal address. The GPN registration must be filed with the Department of Environmental Protection, Bureau of Air Quality, 17 State House Station, Augusta, ME 04333-0017. Submittal may be made electronically through a licensing system provided by the Department, if available.

- Source obligation. A GPN does not relieve any owner or operator of a source from the responsibility to comply with any other requirements applicable to the source.

- Public access to information and confidentiality. All information and data submitted to the Department is subject to the provisions of the Freedom of Access Law, 1 M.R.S. § 401 et seq., as amended. Documents which the owner or operator believes may not be subject to disclosure under the Freedom of Access Law should be clearly marked as “claimed confidential” at the time of submission. Such a claim of confidentiality does not in and of itself protect the documents from disclosure but alerts the Department to the owner or operator’s position that the documents may not be subject to disclosure.

Class IV-A Incinerator General Permit and Emission Control Requirements

- General Conditions

- Employees and authorized agents of the Department may enter and inspect the operator’s business premises pursuant to 38 M.R.S. § 347-C. The owner or operator, or a designee, must provide safe access in compliance with safety requirements of all local, state, or federal regulating authorities as well as all written safety standard operating procedures for that source.

- The owner or operator must pay the annual air emissions general permit fee to the Department, pursuant to 38 M.R.S. § 353-A(10).

- The GPN does not convey any property rights of any sort or any exclusive privilege.

- The owner or operator must comply with all applicable terms and conditions of this general permit. The notification of planned changes or anticipated noncompliance, or the filing of an application for an air emissions license pursuant to 06-096 C.M.R. ch. 115 does not override any condition of this general permit.

- The owner or operator may not use as a defense in an enforcement action that the disruption, cessation, or reduction of operations would have been necessary in order to maintain compliance with the conditions of the general permit.

- Notwithstanding any other provisions in the State Implementation Plan approved by the U.S. Environmental Protection Agency (EPA) or Section 114(a) of the Clean Air Act (CAA), any credible evidence may be used for the purpose of establishing whether a person has violated or is in violation of any statute, regulation, or requirement pursuant to this Chapter.

- Upon written request from the Department, the owner or operator must establish and maintain such records; make such reports; install, use, and maintain such monitoring equipment; sample such emissions in accordance with such methods, at such locations, at such intervals, and in such a manner as the Department prescribes; and provide other information as the Department may reasonably require to determine compliance status.

- The owner or operator must ensure that the incinerator operator(s) are familiar with the terms and conditions of this general permit.

- Notwithstanding any part of this Chapter, the owner or operator is subject to the applicable requirements of Visible Emissions Regulation, 06-096 C.M.R. ch. 101, including limits on fugitive emissions, and must comply with such requirements.

- Class IV-A Incinerator Operation and Emission Control Requirements

- The owner or operator must establish and maintain a continuing program of best management practices for suppression of fugitive particulate matter emissions during any period of construction, reconstruction, or operation which may result in fugitive dust and must submit a description of the program to the Department upon request.

- The incinerator must only be used for the disposal of type 4 waste as defined in 06-096 C.M.R. ch. 100. The incinerator must not be used for the disposal of plastics, cytotoxic (antineoplastic) drugs, or any radioactive wastes and must not be used to dispose of any medical waste classified as type 7 waste, as defined in 06-096 C.M.R. ch. 100. However, the incidental incineration of plastics used in wrapping animal carcasses for handling and storage purposes is allowed.

- The incinerator must not exceed its maximum design charging rate.

- The incinerator must fire only propane, natural gas, or distillate fuel with a sulfur content not to exceed 15 parts per million (ppm) as auxiliary fuel in the unit’s primary and secondary chamber.

- The incinerator combustion gases must vent to a stack that has a height of at least 60% of Good Engineering Practice (GEP). For purposes of this chapter, 100% GEP stack height is determined through use of the following formula:

Hg = H + 1.5L

where Hg = full (100%) GEP stack height

H = height of the overall controlling structure as measured from the ground-level elevation at the base of the stack

L = the lesser dimension of height or projected width of the same structure.

All structures within 5L of the proposed stack shall be considered when determining the overall controlling structure.

- The secondary chamber of the Class IV-A incinerator must be preheated as specified by the manufacturer to a minimum of 1,600 F prior to combusting waste and must be maintained at or above 1,600 F throughout the duration of the burn.

- Once the burn cycle has commenced by introduction of primary chamber combustion, the Class IV-A incinerator must be operated in an efficient manner and as specified by the manufacturer for the period of time between initiation of preheating and reaching the set operational temperature of 1,600 F in the secondary chamber.

- A pyrometer and ¼-inch test port must be operated and maintained at the location on the incinerator or refractory lined stack that provides sufficient volume to ensure a flue gas retention time of not less than 1.0 seconds at the minimum temperature of 1,600 F.

- The Class IV-A incinerator must not exceed a particulate matter emission limit of 0.12 grains per dry standard cubic foot (gr/dscf) corrected to 12% carbon dioxide (CO2) from the auxiliary fuel.

- Visible emissions from the Class IV-A incinerator must not exceed 10% opacity on a six minute block average basis.

- The owner or operator must ensure that all persons designated as incinerator operator(s) receive adequate training to operate the incinerator in accordance with the manufacturer’s specifications and are familiar with the terms of the general permit as it pertains to the operation of the incinerator.

Class IV-A Incinerator General Permit Recordkeeping, Testing, and Reporting Requirements

A.	Recordkeeping Requirements

      - The owner or operator must maintain sufficient records to accurately document compliance with emission standards and general permit conditions and must maintain such records for a minimum of six years. The records must include a copy of the manufacturer’s operating instructions for the incinerator. The records must be submitted to the Department upon written request.

      - The owner or operator must maintain a log detailing and quantifying the hours of operation on a daily basis for the Class IV-A incinerator. The log must record the weight of each charge to the incinerator, preheating temperature, preheating time, charging time, and afterburner temperature directly after charging and every 60 minutes after startup until and including final shutdown time. The source may make use of a temperature chart recorder on the incinerator unit to record the start time, date, and weight of remains charged. The operation log must be kept on site at the incinerator location.

      - The owner or operator must maintain a log detailing the maintenance performed on emission control equipment. Records of the date of each inspection and any corrective action required must be included in the maintenance log. The maintenance log must be kept on-site at the incinerator location.

      - The owner or operator must maintain records of malfunctions, failures, downtime, and any other similar change in operation of air pollution control systems or the emissions unit itself that would affect emissions and that is not consistent with the terms and conditions of this general permit.

      - If the Department determines that continuous recording devices are necessary, the owner or operator must install and begin operation of such continuous recording devices within 120 days of written notification by the Department.

B.	Testing and Reporting Requirements

(1)	In accordance with the Department’s air emission compliance test protocol and 40 C.F.R. Part 60, Appendix A, or other approved method as required by the Department, the owner or operator must:

(a)	Perform testing to demonstrate compliance with applicable emission standards under circumstances representative of the source’s normal process and operating conditions:

- Within 60 calendar days of receipt of a notification to test from the Department or the EPA, if visible emissions, equipment operating parameters, staff inspection, air monitoring, or other cause indicate that equipment may be operating out of compliance with emission standards or requirements of this Chapter;

- Pursuant to any other requirement of this general permit to perform testing.

(b)	Install or make provisions to install test ports that meet the criteria of 40 C.F.R. Part 60, Appendix A, and test platforms, if necessary, and other accommodations necessary to allow emission testing; and

(c)	Submit a written report of test results to the Department within 30 days from the date of test completion.

(3)	If the results of a test performed under circumstances representative of the source’s normal process and operating conditions indicate emissions in excess of the applicable standards, then:

(a)	Within 30 days following receipt of such test results, the owner or operator must correct malfunctions in the system and must re-test the non-complying emission source under circumstances representative of the source’s normal process and operating conditions and in accordance with the Department’s air emission compliance test protocol and 40 C.F.R. Part 60 or other method approved or required by the Department;

(b)	The days of violation include the day of the test showing noncompliance and each and every day of operation thereafter until compliance is demonstrated under normal and representative process and operating conditions, except to the extent that the owner or operator can prove to the satisfaction of the Department that there were intervening days during which no violation occurred or that the violation was not continuing in nature; and

(c)	The owner or operator may, upon the approval of the Department following the successful demonstration of compliance at alternative load conditions, operate under such alternative load conditions on an interim basis prior to a demonstration of compliance under normal and representative process and operating conditions.

(4)	The owner or operator must notify the Department within two days or by the next state working day, whichever is later, of occasions where malfunctions, failures, downtime, and any other similar change in operation of air pollution control systems or the emissions unit that result in an increase of emissions. The owner or operator must report all excess emissions in the units of the applicable emission limitation.

- Severability. Each part of this Chapter is severable, and in the event that any part of this Chapter is held to be invalid, the remainder of the Chapter will continue in full force and effect.

Voluntary Surrender. If the annual fee required by section 4(A)(2) of this Chapter is not paid in full by July 31st of each year, the Department will consider the owner or operator to have voluntarily surrendered the GPN pursuant to 38 M.R.S. § 344(10), and the owner or operator register for a new GPN before resuming operation of the Class IV-A incinerator.

Transfer. The following outlines the procedure for transferring a GPN to a new owner or operator.

- Applicability. A GPN may be transferred to a new owner or operator if the GPN has not expired.

- Required information. The transferee must submit:

- The GPN registration form as specified in section 3(B) of this Chapter identifying the GPN associated with the Class IV-A incinerator and the new owner or operator;

- Proof of publication of the public notice of intent to file as specified in section 3(E) of this Chapter;

- Written statement from the previous owner or operator acknowledging that they are relinquishing the GPN; and

- Payment in full of any past-due fees associated with the GPN.

- Criteria for transfer approval. Approval of the GPN transfer will be based on the acceptability of the information required in the transfer submittal.

_________________________________________

AUTHORITY: 38 M.R.S. §§ 344(7) and 585-A

EFFECTIVE DATE: May 6, 2012 – filing 2012-136

AMENDED:

December 1, 2025 – filing 2025-237

APAO ACCESSIBILITY CHECK: December 4, 2025 (no issues detected)